3GPP TSG-SA WG6 Meeting #68	S6-253762

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253868)

Source:	Ericsson, Nokia

Title:	Pseudo-CR on solution evaluation for technical gap #1

Spec:	3GPP TR 23.700-35 v. 1.0.0

Agenda item:	9.3

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR introduces solution evalution for the existing solutions for technical gap#1.

2. Reason for Change

As the study has completed 75% in SA6#67, this pCR starts looking into the overall evaluation of the existing solutions for technical gap #1.

3. Conclusions

A solution evalution for gap#1 is introduced in this pCR.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v. 1.0.0

* * * First Change * * * *

# 9	Overall evaluation

## 9.1	Solution evaluations

### 9.1.1	Technical gap #1

Technical gap #1 focuses on NRM service to support the latest network capabilities, such as multi-modal traffic. Technical solutions 6, 7, and 10 in clause 8.1.6, 8.1.7, and 8.1.10 respectively address this gap.

Solution 6 focuses on application layer requirements by introducing a new set of NRM APIs to support different applications. The NRM services focuses on specific application requirements such as QoS control service for network transport with high data rate and low latency, QoS control service for network transport with low latency, QoS control service for network transport with low latency and high reliability, QoS control service for network transport with low latency & high bandwidth & high reliability. This enables to create, update and delete the QoS control service for network transport with application specific QoS requirements.

On the other hand, solution 7 focuses on the enhancement of input parameters of the network resource adaptation request from VAL server to the NRM server. The network resource adaptation request is enhanced by adding traffic type to indicate different types of traffic involved (for e.g. audio, video, haptic), priority of the traffic types, and QoS requirement per traffic. The enhanced NRM network resource adaptation service allows the application client/server to add the QoS requirements for each traffic type separately. This addition supports newer network capabilities such as multi-modal application traffic that requires to send traffic type specific QoS requirements to the 5GC. This is later mapped with the AFSessionWithQoS for multi-modal service. Hence, it introduces an enhanced NRM service that makes it suitable for scenarios with single or multiple traffic types, irrespective of the application type.

Furthermore, Solution 10 focuses on single NRM service for network service QoS control for applications. The single NRM API includes inputs to define the basic QoS requirements for multiple applications. The input includes QoS requirements such as Data rate (Maximum uplink data rate and Maximum downlink data rate), Delay characteristics (maximum uplink delay, Maximum downlink delay, round trip delay, delay variance), and Maximum packet loss rate.

* * * End of Changes * * * *