3GPP TSG-SA WG6 Meeting #68	S6- 253779

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6- 253733)

Source:	Samsung

Title:	New key issue on traffic identification and differentiated QoS for multiplexed media flows

Spec:	3GPP TR 23.700-51 v0.2.0

Agenda item:	9.5

Document for:	Approval

Contact:	jerry.bae@samsung.com

1. Introduction

This pCR is proposed to introduce a New key issue on traffic identification and differentiated QoS for multiplexed media flows

2. Reason for Change

This key issue is mainly related to the objective c) in FS_XRM_Ph3_APP.

c.	End to end support for multiplexed data flows, including multiplexed media flows identification and information provisioning in application enabler layer based on (S)RTP Multiplexed Media Identification Information or additional packet filter, providing application enabler layer differentiated QoS service for multiplexed data flows,etc.

Traffic identification and differentiated QoS for multiplexed media flows is as defined in clause 5.37.11 of TS 23.501 and enables differentiated QoS handling for (S)RTP media flows that are multiplexed on the same end-to-end transport layer connection.

In order to request differentiated QoS for multiplexed media flows, the AF has to provide per media flow the (S)RTP Multiplexed Media Identification Information in the Flow Description and the specific QoS requirements.

(S)RTP Multiplexed Media Identification Information may include a combination of at least one of Synchronization Source (SSRC), Payload Type (PT), RTCP SDES item Media Identification (MID) and RTP SDES header extension for MID and RTCP packet type as defined in 5.7.6.2 of TS 23.501.

If a UE indicates support of (S)RTP Multiplexed Media Identification Information in the IP packet filter, it shall indicate this to the SMF at PDU Session establishment and the SMF provides this UE capability information to the PCF. The PCF considers such UE capability information when providing the PCC rules to the SMF.

If (S)RTP Multiplexed Media Identification Information is received in the PCC rule, the SMF provides (S)RTP Multiplexed Media Identification Information to the UPF in the corresponding PDR(s) and also to the UE in the QoS Rule if the UE has indicated that it supports (S)RTP Multiplexed Media Identification Information in IP packet filter.

If the UE has not indicated so, the SMF shall not provide the (S)RTP Multiplexed Media Identification information in the QoS Rule to the UE.

There is a gap between SA6 and SA2 solutions on how SA6 could enhance the utilization of traffic identification and differentiated QoS for multiplexed media flows. For example, the SEALDD client may not know whether differentiated QoS for multiplexed media flows is supported in uplink traffic. Therefore, according to the differentiated QoS for multiplexed media flows request of the SEALDD client, appropriate mapping information should be generated in the SEALDD server and delivered it to the SEALDD client. In addition, when traffic identification and differentiated QoS for multiplexed media flows using MID, the corresponding MID information should be delivered to the SEALDD client, and the SEALDD client can use the MID information during SDP negotiation.SA6 could review and investigate the enhancement to support the traffic identification and differentiated QoS for multiplexed media flows.

NOTE : Unlike the existing KI#2 in TR 23.500-51, this KI focuses on only (S)RTP Multiplexed Media Identification information without calculating traffic characteristics.

This pCR proposes new key issue and the related open issues for study.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-51 v0.2.0.

* * * First Change * * * *

4.x	Key Issue #X: traffic identification and differentiated QoS for multiplexed media flows

4.x.1	Description

Traffic identification and differentiated QoS for multiplexed media flows is as defined in clause 5.37.11 of TS 23.501 and enables differentiated QoS handling for (S)RTP media flows that are multiplexed on the same end-to-end transport layer connection.

In order to request differentiated QoS for multiplexed media flows, the AF has to provide per media flow the (S)RTP Multiplexed Media Identification Information in the Flow Description and the specific QoS requirements.

(S)RTP Multiplexed Media Identification Information may include a combination of at least one of Synchronization Source (SSRC), Payload Type (PT), RTCP SDES item Media Identification (MID) and RTP SDES header extension for MID and RTCP packet type as defined in 5.7.6.2 of TS 23.501.

If a UE indicates support of (S)RTP Multiplexed Media Identification Information in the IP packet filter, it shall indicate this to the SMF at PDU Session establishment and the SMF provides this UE capability information to the PCF. The PCF considers such UE capability information when providing the PCC rules to the SMF.

If (S)RTP Multiplexed Media Identification Information is received in the PCC rule, the SMF provides (S)RTP Multiplexed Media Identification Information to the UPF in the corresponding PDR(s) and also to the UE in the QoS Rule if the UE has indicated that it supports (S)RTP Multiplexed Media Identification Information in IP packet filter.

If the UE has not indicated so, the SMF shall not provide the (S)RTP Multiplexed Media Identification information in the QoS Rule to the UE.

Therefore, if the VAL server does not provide (S)RTP Multiplexed Media Identification Information and specific QoS requirements, the SEALDD server should be able to provide those informations.

There is a gap between SA6 and SA2 solutions on how SA6 could enhance the utilization of traffic identification and differentiated QoS for multiplexed media flows. For example, the SEALDD server, based on whether the UE supports (S)RTP Multiplexed Media Information, could determine how to handle the differented QoS for the UL flow.. Therefore, according to the differentiated QoS for multiplexed media flows request of the VAL server, appropriate mapping information should be generated in the SEALDD server and delivered it to the SEALDD client.

NOTE : Unlike the existing KI#2 in TR 23.500-51, this KI focuses on only (S)RTP Multiplexed Media Identification information without calculating traffic characteristics.

4.x.2	Open Issue

Based on the above description, the following aspects need to be studied:

- Whether and how the SEALDD server can obtain UE support indication of (S)RTP Multiplexed Media Identification Information and coordinate with the SEALDD client applying specific QoS requirements received from the VAL server for each multiplexed flow.