3GPP TSG-SA WG6 Meeting #69	S6-254303

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	China Mobile

Title:	Pseudo-CR on Solution on enablement of standalone DC

Spec:	3GPP TR 23.700-57 v0.3.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR provides the solution of KI#3 on enablement of standalone DC.

This pCR proposes this new solution for TS23.700-57.

2. Reason for Change

This pCR provides the solution of KI#3 on enablement of standalone DC

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.3.0.

* * * First Change * * * *

6.x	Solution #x: Enablement of standalone DC

6.x.1	Description

This solution resolves Key Issue #3: Enablement of standalone DC.

This solution uses the architecture of MMTel Enabler as specified 3GPP TS 23.392 [2].

6.x.2	Procedures

Figure 6.x.2-1 below illustrates the procedure of Enablement of standalone DC. This procedure provides applications with direct access to Standalone Data Channel capabilities for application-layer interactions (e.g., menu content exchange), abstracting away the underlying IMS session awareness requirements.

Pre-conditions:

1.	The Application Server is authorized to use APIs provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [4] or authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [3].

.

Figure 6.x.2-1: Enablement of standalone DC

1.	The Application server sends a Send application message request to the MMTel Enabler client. The request carries an application payload destined for the DC application on the UE, which is its client counterpart. The request message includes information elements as specified in Table 6.x,2-1.

Table 6.x.2-1: Information elements in Send application message request

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID (see NOTE) | M | The identifier of the Application server. |
| Terminating ID (see NOTE) | M | The identifier of the callee. |
| Message ID | M | The unique identifier of this application message |
| application payload | M | The application specific payload destined for the DC application on the UE |
| NOTE: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |

2.	Upon receiving the request, the MMTel Enabler server checks whether the MMTel session with Data Channel capability has been established between the Application server by using the procedure specified in clause 8.4.2 of 3GPP TS 23.392 [2]. If the MMTel session with Data Channel capability has been established, the MMTel Enabler server can send the Send application message request to the UE directly through the Data Channel and step 3 to step 6 can be skipped.

3.	The MMTel Enabler server caches the Send application message request until the standalone DC has been established in step 6.

4.	The MMTel Enabler server utilizes the IMS session management capabilities provided by IMS AS as specified in Annex AA.2.4 of  3GPP TS 23.228 [4] or IMS session management capabilities provided by NEF as specified in clause 5.2.6.40 of 3GPP TS 23.502 [3] to establish the standalone DC between MMTel Enabler server and UE. In this procedure, the MMTel Enabler server utilizes the procedure specified in Annex AG.2.2.2 of 3GPP TS 23.228 [4].

5.	the IMS handles the establishment of standalone DC between MMTel Enabler server and UE as specified in Annex AG.2.2.2 of  3GPP TS 23.228 [4].

6.	The standalone Application Data Channel is established between MMTel Enabler server and UE.

7.	The MMTel Enabler server sends the cached Send application message request to the MMTel Enabler client in the UE via the Application Data Channel. The MMTel Enabler client sends the application payload to the corresponding DC application in the UE.

NOTE:	how the MMTel Enabler client sends the application payload to the corresponding DC application in the UE is internal in the UE and implementation specific.

8.	The MMTel Enabler client sends the Send application message response to the MMTel Enabler server. The response message includes information elements as specified in Table 6.x,2-2.

Table 6.x.2-1: Information elements in Send application message response

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID (see NOTE 1) | M | The identifier of the Application server. |
| Terminating ID (see NOTE 1) | M | The identifier of the callee. |
| Message ID | M | Identifier of the Request that is being responded to. |
| Result | M | Indicates if the sending of this application message is success or failure. |
| Reason (see NOTE 2) | O | The reason for failure |
| NOTE 1: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. NOTE 2: This IE shall only be present when the sending result is failure. |  |  |

9.	The MMTel Enabler server sends the Send application message response to the Application server.

10.	Subsequent application interactions can take place over the established standalone application Data Channel.

6.1.3	Impacts on Existing Nodes and Functionality

In this solution, the MMTel Enabler server needs to utilize the procedure specified in Annex AG.2.2.2 of  3GPP TS 23.228 [4].

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.392: "Application enablement aspects for MMTel".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[5]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[6]	GSMA NG.110: "Multi Device".

[7]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[8]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital assets".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *