3GPP TSG-SA WG6 Meeting #69	S6-254319

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	Sepura Ltd

Title:	Discussion on implementing non-3GPP security message in the MCPTT plane

Agenda Item:	10.1

Contact:	Kit Kilgour (kit dot Kilgour at Sepura dot com)

Abstract: The Non-3GPP security message option for transporting LMR KMMs is currently intended to be available across both the MCPTT & MCData planes and has been implemented in Stage 3 for the MCData plane. This contribution discusses whether implementation in the MCPTT plane is needed.

# Discussion

## 1	Background

The Non-3GPP security message (3GPP TS 23.283 [1] clause 10.12) is the SA6 message corresponding to the Interworking Security Data (InterSD) message described by SA3 in 3GPP TS 33.180 [2] clause 11.2. The purpose of the message is to provide an identifiable non-SDS transport option for LMR Key Management Messages in the case that a 3GPP MC system is interworked with an LMR system that both supports and has deployed the relevant LMR native End to End Encryption (E2EE) solution, such as TETRA or P25 deployed for Public Safety purposes. LMR KMMs can also be transported in MC via MCData SDS.

SA6 decided in Rel-15 that the Non-3GPP security message should be MC service independent. The rationale in SA6 at the time was that there might be an interworked LMR network with E2EE that was deployed with voice only and no short messaging (SDS) like functionality. Stage 3 has currently implemented it in the MCData plane but not yet the MCPTT plane.

## 2	Implementation considerations

The MCData plane is well suited to the transmission of such messages as the MCData architecture is inherently designed to transport standalone messages (such as SDS) and has parameters in place that can be re-used to determine whether a Non-3GPP security message is suitable for transmission over the MCData plane – e.g. maximum allowed message length.

Implementation in the MCPTT plane would be possible by modelling a deployment similar to that in the MCData plane which is based on the SDS architecture (but without the message being tagged as an SDS). However, from an architectural perspective this would involve an approximate re-integration of MCData SDS architecture (to transport a SIP MESSAGE between servers) and associated parameters back into the MCPTT architecture, when this was deliberately separated out from voice delivery by the creation of the MC Core architecture during Release-14.

In terms of deployment, the author is not aware of an LMR network deployed with LMR E2EE that does not at least have an SDS-like capability to transport KMMs.

The use case which would be excluded if the MCPTT plane implementation were not carried out would be if a MC system that supported only MCPTT (and possibly video) but no MCData was interworked with a Public Safety LMR network wishing to use E2EE for interworking.

## 3	Conclusion

Does SA6 feel that support of full LMR E2EE interworking of voice via an MCPTT-only MC system is necessary? If not, Sepura Ltd can bring a CR to remove it from the Stage 2 specification.

## 4	References

[1]	3GPP TS 23.283: "Mission Critical Communication Interworking with Land Mobile Radio Systems; Stage 2".

[2]	3GPP TS 33.180: "Security of the mission critical service".