3GPP TSG-SA WG6 Meeting #69	S6-254321

Wuhan, China 13th – 17th October 2025	(revision of S6-253xxx)

Source:	Ericsson

Title:	Basic functions required for Sensing service

Spec:	3GPP TR 23.700-15 v0.1

Agenda item:	9.13

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

The pCR provides a solution for Key Issue #1.

2. Reason for Change

Key Issue #1 describes:

Whether and how to enhance the application enablement architecture and related functions to support the mapping of sensing service requirements of vertical applications.

There are various sensing applications like sensing UAV, sensing human, etc., from simple tasks like detecting presence/absence to complex tasks like characterization of the objects (like size). In order to support the sensing application, it is essential for the enablement layer to provide fundamental functionality for various aspects of sensing.

The functionalities can include registration, discovery, and requesting sensing results from the sensing entities, which can sense the target. For example, it is important for the sensing enablement layer to use the right sensing entities, like outdoor sensing for drone detection, indoor sensing for elderly fall detection. For the enablement layer, to find the right sensing entity, the entities need to register and be discoverable within the enablement layer.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1

* * * First Change * * * *

## 6.x1	Solution #x1: Sensing enablement client registration

### 6.x1.1	Solution description

The sensing enablement client registration procedure provides a solution for KI#1. It is essential for the sensing enabler layer to understand the capabilities of the sensing clients in order to use the correct sensing clients for sensing service.

### 6.x1.2	Procedures

#### 6.x1.2.x1	Sensing enablement client registration

Figure 6.x1.2.x1-1 illustrates the procedure for the SAE client registration.

Figure 6.x1.2.x1-1: Sensing enablement client registration

1.	The SAE client sends a SAE client capability registration request to register its sensing capability information to the SAE server. The sensing capability information includes the type of sensing client (e.g., wifi sensing, Bluetooth sensing, LiDAR etc.) sensing capabilities like sensing tasks (e.g., presence/absence/movement/gesture detection, movement/gesture characterization, object characterization (like size, velocity, direction, angle), tracking, localization), sensing client deployment environment (e.g., indoor, outdoor).

2.	The SAE server registers the SAE client and assigns a unique identifier to identify the SAE client and sends a response to the SAE client.

* * * Next Change * * * *

## 6.x2	Solution #x2: Sensing clients discovery

### 6.x2.1	Solution description

The sensing client discovery procedure provides a solution for KI#1. The solution enables consumers to discover the sensing clients for their sensing service.

### 6.x2.2	Procedures

#### 6.x2.2.x1	Sensing clients discovery

Figure 6.x2.2.x1-1 illustrates the procedure for the Sensing client discovery

Figure 6.x2.2.x1-1: Sensing client discovery

1.	The consumer (e.g. SAE server#1) sends a sensing clients discovery request to discover the sensing client to SAE server#1. The request includes the type of sensing application (e.g., UAV detection), and sensing client discovery filters. The filter includes sensing tasks (e.g., presence/absence/movement/gesture detection, movement/gesture characterization, object characterization (e.g., size, velocity, direction, angle), tracking, localization), sensing environment (e.g. indoor, outdoor), and location information.

2.	The SAE server#2 identifies the sensing clients that matches the sensing discovery filters provided in the request with the stored sensing client capability information and sends discovery response to the consumer. The response includes the list of SAE client identifier and their SAE client capability information.

* * * End of Changes * * * *