| TSG-T WG1 Ad-hoc on RRM testing issues |  |  | T1S-010254 |
| --- | --- | --- | --- |
| Sophia Antipolis, France |  | 11th & 12th October 2001 |  |
| Title | Split of responsibilities for RACH testing |  |  |
| Source | Anritsu |  |  |
| Agenda Item |  |  |  |
| Document for | Agreement |  |  |
| Contact | Dan Fox (Anritsu) dan.fox@eu.anritsu.com Tel: +44 1582 433357 |  |  |

# Overview

Certain elements of RACH procedures are specified in TS 25.321 (MAC), which typically is the responsibility of T1 Signalling SWG to test. However, RACH procedures are also tested during RF tests. The purpose of this document is to see if any of the RACH tests that would arise from TS 25.321 overlap RF tests.

# Possible tests MAC Level RACH procedures

Tests of Access Service Class selection (11.2.1)

- Check that the UE chooses the correct ASC according to the signalled logical channel priority. The test can probably only check that a valid PRACH partition and a non-zero persistence value are applied.

    - Control of RACH transmissions for FDD mode

- Check that the correct value of persistence PI is applied for a given ASC. This procedure is probably not testable.

- Check that the backoff timer is correctly applied.

- Check that the backoff time is not applied when no AICH ACK or NACK is received.

- Check that the retry counter (M) is correctly applied.

- Check that if a persistence of zero is applied, no access takes place.

    - Control of RACH transmissions for TDD

FFS.

# Decision

For each of the above tests, it needs to be agreed whether enough coverage is already provided, or intended to be provided by TS 34.121, or whether an additional test case should be added in TS 34.123-1.

## References

3G TS 25.321 version 3.8.0 MAC protocol specification

3G TS 25.214 version 3.7.0 Physical layer procedures (FDD)