3GPP TSG-T1 Meeting #22 	Tdoc T1-040308

Hyderabad, India, 2-6 Feb 2004

| CR-Form-v7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.108 | CR | - |  rev | - |  Current version: | 4.9.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | CR to 34.108 R99: Correction to handling of Entered Parameter IE in default contents for Initial Direct Transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Qualcomm |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI |  |  |  |  |  |  | Date:  |  | 4/2/2003 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-4 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) |  |
| Reason for change:  |  | Core spec imposes no clear requirement on the value of Entered Parameter sent by the UE. RAN2 and SA2 have indicated that this value is not used by the network and should not be checked. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Entered Parameter IE changed from “False” to “Not checked” in default. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Good UE could fail test cases. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| Affected: |  | X |  | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

### 9.1.1	Default RRC Message Contents (FDD)

…

###### Contents of INITIAL DIRECT TRANSFER message: AM

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Integrity check info |  |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |
| CN domain identity | Checked to see if set to supported CN domain as specified in the IXIT statements. |
| Intra Domain NAS Node Selector |  |
| - CHOICE version | R99 |
| - CHOICE CN type | GSM-MAP |
| - CHOICE Routing basis | Local (P)TMSI |
| - Routing parameter | If the IE "CN domain identity" is equal to "CS domain", this bit string is set to to bits b14 through b23 of the TMSI. If the IE "CN domain identity" is equal to "PS domain", this bit string is set to to bits b14 through b23 of the P-TMSI. The TMSI/P-TMSI consists of 4 octets (32bits). This can be represented by a string of bits numbered from b0 to b31, with bit b0 being the least significant. The "Routing parameter" bit string consists of bits b14 through b23 of the TMSI/ PTMSI. The first/ leftmost/ most significant bit of the bit string contains bit b23 of the TMSI/ PTMSI. |
| - Entered parameter | Not checked |
| NAS message | Set according to that indicated in specific message content for each test case |
| START | Not checked |
| Measured results on RACH | Not checked |