3GPP TSG-T Meeting #22 	Tdoc T1-040259

Hyderabad, India, 2nd February – 6th February 2004

Title:	LS on Permissibility of Separate RRC Connections for Sequential CS and PS Registration

Source:	TSG T WG1

To:	TSG CN WG1

Cc:	TSG RAN WG2

Contact Person:

#### Name:	Nathan Tenny

E-mail Address:	ntenny@qualcomm.com

1. Overall Description:

T1 have recently become aware of a possible ambiguity in the expected behaviour of a Class A UMTS UE performing sequential CS and PS registrations on a network operating in NMO II.  In the case that the PS registration is caused by MMI or AT commands, our test procedure calls for the UE to use separate RRC connections for the two registration procedures.  We are uncertain as to whether it is still permissible for a Class A UE to register using two separate RRC connections in the case that the PS registration happens because of an automatic PS attach at power-on.

To resolve this question, T1 would like to request guidance from CN1 regarding the correct behaviour of a Class A UE..  If it is permitted to register on separate RRC connections, T1 would also like to ask whether the UE is required to take any further action, e.g.,  to indicate to the UTRAN that the first RRC connection is being released locally after the CS registration procedure.

2.  Actions:

To CN1 group.

ACTION: T1 ask the CN1group kindly to answer the following questions:

- In the non-combined environment (NMO II), do the core specs permit a Class A UMTS UE  that automatically registers in the PS domain at power-up to use two separate RRC connections for the CS and PS registration procedures?

- If the answer to 1. is positive, do the core specs impose any requirement on the UE’s behaviour with respect to the first RRC connection after the CS registration procedure is complete?

3. Date of Next T1  Meeting:

T1 Meeting #23 	10 – 14 May 2004	Beijing