3GPP TSG-T WG1 EMC #4 meeting                Tdoc TSG T1E-99033

# Scotland  30 July 1999

Agenda item :

Source : Panasonic (Matsushita Communication Industrial Co., Ltd.)

Title :  Comment for 3GPP EMC #4 Meeting

Definition of EMC for 3GPP Mobile Communication System

1. Radiated Emission : Emission under receiving condition (including idling state of transmitter).

Reason : We must divide the spurious emission of transmitting (concern with carrier wave) from

another radiation of digital circuit (ex. Clock OSC.) or receiving circuit (ex. Local OSC. ).

Because generally spurious emission level is estimated as the quality of carrier wave in regional

standard or law (ex. Radio law in Japan).

So I think we should not care for these spurious emission(radiation) as a conformance test of EMC

requirement, and we should separate the Radiated Emission items concerned with the active state

of transmitter from 3GPP EMC standards in order to be not confused

(ex. That’s why, in Japan, spurious emission is mandatory standard but EMC standard is not. So If

standards of spurious emission and standards of another radiated emission are on the same EMC

table, we may misunderstand that standards of another radiated emission as mandatory. )

and it should be specified into 3GPP RF standards.

We should measure the radiated emission under the condition as follows.

1) State of receiving for Receiver and Transceiver

2) State of idling for Transmitter (We should not care the active sate of transmitter as a radiated

emission of EMC.)

Therefore we should delete the Radiated Emission items concerned with the active state of

transmitter in Table 1 (EMC regulatory specifications areas and regional requirements).

2. Immunity : Basically we should refer to international standards and the present standards of mobile

communication system of ETSI 300 342-1/2 and ARIB T-57 2nd edition(draft).

(But we should check the test method, level and performance criteria for applying to W-CDMA

systems.)  Or we should investigate necessity of original tests for W-CDMA systems.

? I think the immunity requirements in the 3GPP EMC standard should be considered as one of

product quality issue.