ETSI SMG4 / 3GPP TSG-T MExE	Tdoc SMG4 4M99-054

Helsinki, Finland

17th-19th February, 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.4.0 |  |  |  |
| Submitted to SMG |  |  | For approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE |
| --- | --- |

| Source: | Motorola | Date: | 12/01/99 |
| --- | --- | --- | --- |

| Subject: | MExE functionality on/off switch |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 | X |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Following on from the Fort Worth meeting, it was proposed that an on/off switch for the MExE functionality of a MExE MS is required. It is proposed that the enabling/disabling of MExE functionality is performed as part of the user interface customisation, stored in the user profile, and communicated via the capability negotiation. |
| --- | --- |

| Clauses affected: | 4.4, 4.5, 4.6 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

Content negotiation is used to select the best representation of an entity when there are multiple representations of the entity available from the MSE.  The entity (e.g. a service, an image, etc) is located behind a URI, and the application in the MExE MS connects to the URI by using HTTP/1.1 or an HTTP/1.1 derived protocol. The best representation of an entity can be decided by the server (server-driven negotiation) or by the client application (agent-driven negotiation).

The content negotiation transferred between the MExE MS and the MSE is identified in subclause 4.4.2 Client Capability Report onwards.

### 4.4.1	Capability negotiation characteristics

The generic set of capabilities which may be negotiated between the client and the server consists of the subsequently identified characteristics.:-

| Token | Characteristics |  |  | Optionality |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MS to MSE | MSE to MS |  |
| MExE-class | MExE Classmark |  |  | M | O | Classmark of the MExE device, e.g. 1, 2, etc. Example coding: MExE-class: 1 |
| MExE-status | MExE Status |  |  | O | O | Indicates whether MExE functionality is been enabled or disabled on the MS, e.g. enabled/disabled. Example coding: MExE-status: enabled |
| MExE-Spec | MExE specification |  |  | O | O | Version of the MExE Stage 2 standard supported by the MS; represents the first two digits of the 03.57 specification version number. Example coding: MExE-Spec: 7.0 |
| MExE-srvcs | MExE services |  |  | O | O | Identification on where information on the PLMN-provided MExE services may be found, e.g. URL Example coding: MExE-srvcs: http:://telco.mexe.com/services |
| MExE-manuf | Device | Manufacturer |  | O | O | Identification of the manufacturer, e.g. Motorola, Nokia, Ericsson, etc Example coding: MExE-manuf: Motorola |
| MExE-model |  | Model |  | O | O | Model type, e.g. 130, 8810, 688, etc. Example coding: MExE-model: 130 |
| MExE-SN |  | Serial Number |  | O | O | Manufacturer’s model, e.g. 1.1.0 etc. Example coding: MExE-SN: 1.1.0 |
| MExE-SV |  | SoftwareVersion |  | O | O | Manufacturer’s software version, IMEISV Example coding: MExE-SV: nnnnnnnnnnnnnnn |
| MExE-res | Display | Size | resolution | O | O | Pixel size of the display, e.g. 200x150 etc. Example coding: MExE-resol: 1.1.0 |
| MExE-chars |  |  | Characters | O | O | Number of lines and characters per line, e.g. 3x20, 4x15 etc. Example coding: MExE-chars: 3x20 |
| MExE-clrs |  | Colour/monochrome | Number of colours | O | O | The number of colours supported, e.g. . 1, 256, 16,000 etc. Example coding: MExE-clrs: 256 |
| MExE-grey |  |  | Greyscale | O | O | The number of gray scale, e.g. 2 for black and white only Example coding: MExE-grey: nnn |
| MExE-voice | Input mode | Voice |  | O | O | Indication of whether voice capture is supported, e.g. yes/no Example coding: MExE-voice: yes |
| MExE-kbdid |  | Keyboard type | Keyboard identity | O | O | Type of keyboard supported, e.g. default, PC-AT-102 Example coding: MExE-kbdid: default |
| MExE-kbdlg |  |  | Human language | O | O | Language used on keyboard, i.e. EN-UK etc. Example coding: MExE-kbdlg: EN-UK |
| MExE-keys |  |  | Number of keys | O | O | The number of keys supported by the keyboard, i.e. default, 15, 102, etc. Example coding: MExE-keys:default |
| MExE-pntr |  | Pointing device | Identity | O | O | Type of pointing device, i.e. none, touchscreen, mouse, etc. Example coding: MExE-pntr: none |
| MExE-hlang | Human Language |  |  | O | O | Preferred language in which the user wishes to work, i.e EN-UK etc. Example coding: MExE-hlang: EN-UK |
| MExE-flang | Formal Language |  |  | O | O | Prioritised list of renditions for formal languages, e.g. HTML, WML, etc. Example coding: MExE-flang: WML/HTML |
| MExE-adprt | Additional Protocols |  |  | O | O | List of additionally supported protocols of the MExE device, e.g. SAT (SIM Application Toolkit) Example coding: MExE-adprt: SAT |
| MExE-bear | Supported Bearers |  |  | O | O | Prioritised list of preferred bearers, e.g. SMS, USSD, GPRS, etc. Example coding: MExE-bear: GPRS/SMS |
| MExE-cldif | Difference to base Classmark |  |  | O | O | Identification of the devices’ components differing from the base Classmark definition Example coding: MExE-cldif: Java.rmi |
| MExE-memty | User authorised available memory | Type of memory |  | O | O | Type of memory, e.g. RAM, memory, hard disk, etc. Example coding: MExE-memty: RAM |
| MExE-mem |  | Amount of memory |  | O | O | Amount of memory authorised by the user for MExE, e.g. 1, 6, 10, etc. Example coding: MExE-mem: 1 |
| MExE-audio | Multimedia | Audio codec type |  | O | O | Prioritised list of preferred audio codecs (cf. H.323) e.g. G.711, G.723 etc. Example coding: MExE-audio: G.711 |
| MExE-video |  | Video codec type |  | O | O | Prioritised list of preferred video codecs (cf. H.323), e.g. H.261, H.263 Example coding: MExE-video: H.261 |
| MExE-tsize | Transfer limits | Size |  | O | O | Maximum size of transfers in Kb, e.g. 1,2, 4, 8, etc. Example coding: MExE-tsize: 4 |
| MExE-ttime |  | Time |  | O | O | Maximum duration of transfers in minutes, eg. 1,2, 4, 8, etc. Example coding: MExE-tsize: 2 |
| MExE-loc | Location Information | Support of location information |  | O | O | Type of location informationsupported, if any, e.g. No, GPRS, cell, other E-OTD, TOA, etc. Example coding: MExE-loc: E-OTD |
| MExE-locin |  | Location information |  | O | O | Actual location information (as defined by type) Example coding: MExE-locin: xxxxxxxxxxxxxxx |
| MExE-uprofile | User profile |  |  | O | O | Transfer of the User Profile; individual characteristic(s) of the profile Example coding: MExE-uprof: nnnnnnnn |
| MExE-URL |  |  |  | O | O | An indirect reference to a location where the MExE MS’s characteristics may be obtained Example coding: MExE-URL: http:://name.co.com/profile |
| MExE-addch | Additional characteristics |  |  | O | O | Support of additional characteristics, e.g. none, smart card reader, docking station, etc. Example coding: MExE-addch: docking-station |
| MExE-addin | Additional Information |  |  | O | O | Manufacturer defined additional characteristics; free format Example coding: MExE-addin: XYZ |

Table 1: Capability negotiation characteristics

The support of the above characteristics between the MS and the MSE is either mandatory (marked with an M) or optional (marked with an O), as indicated in the Optionality column.  M indicates that it must be sent by that entity, and O indicates that it may be sent by that entity if it supports it.  A characteristic marked as O, does not necessarily require to be supported by the receiving entity.

Generally, the combination of user profile preferences and ME logic will determine the information sent in the capability negotiation from the MExE device to the MExE Service Environment.  As an example, for the support of location information the user’s profile controls  if and when location information may be sent to the MExE Service Environment (e..g. never sent, always sent, only after user confirmation).

The capability negotiation process shall be used by both the client and the server to permit transfer of capabilities from one to the other.  By transferring its capabilities, the client or server will support efficient use of resources both over the radio interface as well as in the client or server.  Capability negotiation shall performed by the server prior to transfer over the radio interface to verify as far as possible the ability of the client to support any services to be downloaded.

In order to transfer the capability information between the MExE MS and the MSE, “tokens” are identified for each transferable characteristic in the HTTP request and response headers.

### 4.4.2	Client Capability Report

The client may report its capabilities either spontaneously, or shall do so as a result of an enquiry from a server; the characteristics to be reported in the client capability report are identified in Table 1.

The client reports it's capabilities to the server by using appropriate HTTP request headers. The following capabilities can be reported:

- Client software (product): User-Agent header

- MIME media types: Accept header

- Character set: Accept-Charset header

- Content encoding: Accept-Encoding header

- Language: Accept-Language header

There is no need for MExE to specify any tokens for content negotiation, as they are already defined in HTTP.  The formats for these headers are specified in [9].

Example:

The following HTTP request reports that the name of client software is "GSM-Phone" version "1.0". The MExE-Spec token identifies the first two digits of the MExE specification.  The client accepts both compiled WML and compiled WMLScript, and supports both the English and Swedish as languages.

GET / HTTP/1.1

Host: www.company.com

User-Agent: GSM-Phone/1.0

MExE-Spec: 7.0

Accept: application/x-wap.wmlc, application/x-wap.wmlscriptc

Accept-Language: en, sv

...

The basic format of the User-Agent: header is:  User-Agent: software-name/version. A comment can be attached enclosed in parentheses to give more specific information. For example, operating system, display size, supported software extensions, script libraries, etc. The format of the comment is, however, not specified in [6].

### 4.4.3	Server-driven Negotiation

The server may request the capabilities of a client whenever required, and shall enquire of the client’s capabilities prior to making each transaction resulting in a set of transfers to the client; the characteristics which may be reported in the client capability report are identified in Table 1.

In server-driven negotiation the server signals to the client that the response entity was selected from a set of available representation. To do this, the server attaches the Vary: response header in the response to the client. The Vary: header includes a list of request headers. For example:

HTTP/1.1 200 OK

Vary: User-Agent, Accept-Language

Content-type: application/x-wap.wmlc

Content-language: en

...

Indicates that the entity is available for multiple languages and user-agents. The selected entity is the English version.

### 4.4.4	Client-driven Negotiation

In client-driven negotiation the client selects the best representation after having received an initial response from the server. The response from the server is a 300 Multiple Choices response and contains a list of available representations. The selection of the available representations may be done automatically by the client application or by the (human) user from a menu.

It is noted that there is an implicit overhead of (at least) one additional round-trip delay with client-driven negotiation.  The client-driven negotiation will always require an additional request/response iteration, due to the fact that the initial response from the server to the client’s initial request may be a 300 Multiple choices response, or an equivalent presentation of available choices.   After the user has selected one of the available options, the client sends the request for the actual content to the server.

### 4.4.5	Example client-server negotiation

The initial client request  is the same as for subclause 4.4.2 “Client capability reporting” (with the difference being that the client does not indicate its acceptable or preferred format(s), in this initial request).  Also, the client’s request does not refer to a specific document (note, that there is no file-type in the URL), but rather some abstract resource “/late_works/sunflowers“ that is to be interpreted by the server (e.g. as a subdirectory containing miscellaneous versions of the said painting):

GET / HTTP/1.1

Host: http://www.vincent_van_gogh_museum.nl/late_works/14sunflowers

User-agent: GSM-phone/1.0

MExE-Spec: 7.0

Accept-Language: en, ge

...

The server recognises that it has several versions and/or formats of the client’s requested document, and thus issues a 300 multiple choices response to the client.  The response includes a list (here: a .html-document) describing the available formats. This list of available formats is displayed to the user who selects one, and the user-agent then requests the selected document from the server in a second request/response iteration.  Note (as shown in this example), that the server can optionally include its preferred or suggested representation (e.g. the one it deems most appropriate for that particular user-agent) in the Location field.  A user-agent can use this to perform an automatic redirection if it chooses not to display the obtained document to the user, in which case this method falls back to server-driven negotiation):

HTTP/1.1 300 Multiple Choices

Content-type: text/html

Location: /late_works/sunflowers/14sunflowers_midres.gif

<html>

<head>

...

</head>

<body>

<b>We have several versions and formats of the requested painting available - please select:</b>

<br>

icon size (30x20):

<a href="14sunflowers_icon.gif">GIF-format</a>,

<a href="14sunflowers_icon.bmp">bitmap-format (Windows)</a>.

<br>

low resolution (200x300):

<a href="14sunflowers_lowres.gif">GIF-format</a>,

<a href="14sunflowers_lowres.jpg">JPEG-format</a>.

<br>

medium resolution (400x600):

<a href="14sunflowers_midres.gif">GIF-format</a>,

<a href="14sunflowers_midres.jpg">JPEG-format</a>.

<br>

high resolution (800x1200):

<a href="14sunflowers_highres.gif">GIF-format</a>,

<a href="14sunflowers_highres.jpg">JPEG-format</a>.

<br>

super-high resolution (1200x1600):

<a href="14sunflowers_shighres.gif">GIF-format</a>,

<a href="14sunflowers_shighres.jpg">JPEG-format</a>.

</body>

</html>

...

Editor’s note: RFC 2068 states that "the server SHOULD include an entity containing the list of resource characteristics and location(s) from which the user or the user-agent can choose the one most appropriate".  The RFC does not state, how a user(-agent) could possible select any format, if the server does NOT send any such further indication about available formats...).

However, the RFC does not name or describe any specific format, in which such a list should be transferred, but rather notes that "the (list) entity format is specified by the media type given in the Content-type header field".  As there appears to be no special "format-list" media-type, the typical format for such a list is probably a simple .html-document describing the miscellaneous format choices (as shown in the above example).  The document can be displayed to the user, and the user then picks the desired format by clicking on one of the embedded hyper-links.

While it is very easy to implement this "carbon-supported" selection mechanism in a Web-browser, the possibility to select the correct format automatically is drastically limited, because the browser does not have any semantic understanding of the kind and attributes of the document versions listed in the .html document.  The RFC authors were probably quite aware of this limitation as they also noted "However, this specification does not define any standard for such automatic selection.". They probably expect(ed) that such a list format will be specified in another document.

In the absence of any such standardisation activities, it will probably be necessary to standardise such an attribute-list format so that MExE-user-agents can "understand" and automatically select from the list, such that automatic content format selection (not to speak of other capability negotiation issues...) may be enabled.

Editor’s note: Final two paragraphs of the above editorial note requires to be reconsidered and preferably deleted, and the editorial note converted into main text.

## 4.5	User profile

The user profile (which may consist of one or more user profiles) contains the characterisation of the MExE MS as defined by the user and service provider. The user’s characterisation of the MExE MS in the user profile may be modified at any time by the user, and changes affected at the earliest possible opportunity.  The user profile acts as a repository (which is always available in the MExE MS) defining the MExE MS behaviour, identifying one or more of the following:-

|  | ME specific | Location(s) | Description |
| --- | --- | --- | --- |
| user interface |  |  | Refer to sub-clause 4.6 User interface customisation for details of the specific user interface customisation characterisation which may be stored in the user profile |
| background - colour - pattern/image |  |  |  |
| - font - type - colour - size |  |  |  |
| - interfaces - softkey definition - additional menu items - enabled/disabled extended menus - icon settings |  |  |  |
| - country - language - date - time - currency |  |  |  |
| - Modify supplementary services settings (i.e. diversion, times) | - ? |  | - ? |
| ###### Modify phone settings - ringer tone/vibrate - enabling SIM Lock - Keypad Lock |  |  |  |
| - Preferences |  |  | - 03.57, section 4.4.1 - ”Support of the user profile preferences is subject to the capabilities of the device. Generally the combination of user profile preferences and ME logic will determine the information sent in the capability negotiation from the MExE device to MSE.” |
| - terminal properties - language in which the user wishes to work - preferred input mode - amount of memory authorised by the user for MExE - MExE status |  |  |  |
| - data transfer - prioritised list of preferred bearers, e.g. SMS, USSD, GPRS - maximum size of transfer - maximum duration of transfer - transfer of location information - transfer of the user profile or parts of it |  |  | - may, or may not be, sent to the MSE - could be shifted in Security section - could be shifted in Security section |
| - codecs - prioritised list of preferred audio codecs - prioritised list of preferred image codecs - prioritised list of preferred video codecs - prioritised list of preferred multimedia codecs |  |  |  |
| Service Management |  |  | Refer to sub-clauses 4.5, 4.7.1, 4.7.2 controls the ability to determine which services are transferred to, resident, configured or executing on the MS, which may be refined by use of one or more of the following characteristics: identified security level specific permitted services types of permitted services date or times for actions priorities of services and identifies service preferences which may be set globally for all services per security level on a per service basis and may be identified at installation of the service, or be subsequently modified. |
| transfer of services |  |  |  |
| automatic configuration |  |  |  |
| execution of services |  |  |  |
| residence on the MExE MS |  |  |  |
| personalisation of services |  |  | 02.57, section 6.1 for further details see section VHE of this table |
| Security |  |  | control of the following permissions maybe refined by use of one or more of the following characteristics: - universal authorisation for all calls/connections/ messages (i.e. yes/no) - authorisation by calls/connections/messages type (i.e. SMS, USSD etc.) - authorisation by identified services - authorisation by service type (i.e. security level) - authorisation by date/time/day of week |
| call authorisation |  |  | 03.57, section 4.5 |
| Implicit permission for the initiation of events (includes responses made by the MS to external events) of transferred applications, applets |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Implicit permission for the transfer of applications, applets and content |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Network event journal - status of the optional network event journal which is used to maintain a record of application initiated network events (i.e. activated/deactivated) |  |  |  |
| Miscellaneous |  |  |  |
| phonebook entry |  |  | 03.57, section 4.6 |
| ###### Redirect - voicemail box number - fax-mail box number |  |  |  |
| - reset call meters |  |  | - 03.57, section 4.6 |
| Classmark 1 specific Entries |  |  |  |
| Classmark 2 specific Entries |  |  |  |
| ###### WAP browser - pre-installed browser overwrite on upgrade - location of user preferred WAP browser |  |  | - applicable to MExE classmark 2 |
| VHE |  |  | TS 22.xx :Provision of Services in UMTS – The Virtual Home Environment (Tdoc SMG1 825/98) |
| - for each VHE profile: - Incoming session routing address |  |  |  |
| - for each service within one VHE profile: - service state - service appearance - service appearance customisation parameters - service functionality customisation parameters - service data |  |  | - e.g. activate/deactivate/not available - e.g. level of functionality complexity - e.g. forwarded-to numbers |

The above list is not exhaustive.

Support of the user profile preferences is subject to the capabilities of the device.

Editor’s Note: details on the location of the user profile is for further study.

## 4.6	User interface customisation

The MS interface consists of the buttons, menus, screens and MMI as designed and provided by the MS manufacturer; the nature of this MS interface is naturally evolving, MS specific and proprietary to the individual manufacturers of the industry.  This interface is the one normally seen by the user in normal operation of his MS.  This specification does not place any requirements or limitations on the individual manufacturers’ MS interface.

The MExE MMI, in turn, is the interface available to the user to support MExE services and functionality on the MS.  The nature of the MExE MMI interface, like the normal MS interface described above, is not standardised in any way, to allow for manufacturer innovation, cater for evolving market needs, and permit manufacturer differentiation.  The MExE MMI, depending on different manufacturer implementations, may consist of the normal MS interface, the normal MS interface with modifications, a different interface to the normal MS interface, or some combinations thereof etc.  MExE services operate within, and using the capabilities of, the MExE MMI.

User interface customisation consists of two parts.  The first part refers to the user’s ability to request, and verify, the preferred changes to the user interface; thus the user’s preferences, as supported by the specific MS, require to be recorded.  The second part refers to the MExE MS’s support of the user’s preferences for the interface, wihtin the capabilities of an MS.  By defining the user interface customisation to consist of two stages, the preferences which have been recorded by the user may be transferred (as part of the user profile), and thereby provide protability of the user’s preferences.

### 4.6.1	MExE user interface customisation

Customisation of the user interface offers the MExE Service Environment and or the user, the ability to inform the MExE MS of the desired extent of customisation.  All support of the user interface customisation is optional, not mandatory on any class of MS, and subject to the capabilities of the MS.  Depending on the capability of the MS, the customisation may be fully supported, partially supported, interpreted or ignored.

Customisation of the user interface is not restricted to the modifying the appearance of the MMI, but also the modification of MMI parameters (e.g. programming of the voicemail number).  The user’s customisation of the interface is retained as part of the user profile.

The MExE Service Environment, and or the user, defines the level of customisation requirement via the form of customisation tokens, and each token uniquely identifies a customisation characteristic.  The MExE MS in turn determines the level of customisation from the tokens, and subject to the capabilities of the terminal, may or may not support the required customisation. The customisation tokens are defined in table x.

Access to the different characteristics of the user interface customisation is subject to the security levels which also apply to MExE services.  Direct user customisation is considered to be equivalent to a security level 3 MExE service.

### 4.6.2	User interface customisation tokens

To support the user customisation of the interface, and portability of the preferences, table x defines the customisation tokens.

|  |  | Security Level |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Token | User Interface Customisation | 1 | 2 | 3 | 4 |
| Logo | Modify service provider logo | O | - | - | - |
| Custom-services | Enable/disable network operator custom services | O | - | - | - |
| Background | Modify background colour | - | - | O | - |
| Font-colour | Modify font colour | - | - | O | - |
| Font-size | Modify font size | - | - | O | - |
| Icon | Add/modify icon | - | - | O | - |
| Menu-modify | Add/remove menu item | - | - | O | - |
| Menu-activation | Enable/disable menu item | - | - | O | - |
| Softkey | Softkey definition | - | - | O | - |
| Phonebook | Add/modify phonebook entry | - | - | O | - |
| Language | Preferred language | - | - | O | - |
| Input-mode | Preferred input mode | - | - | O | - |
| Supplementary-Services | Modify supplementary services settings (i.e. diversion, times) | - | - | O | - |
| Voicemail | Add/modify voicemail number | - | - | O | - |
| Phone-settings | Modify phone settings (i.e. ringer tone/vibrate, enabling SIM Lock, Keypad Lock, MExE status etc.) | - | - | O | - |
| Extended-menus | Enable/Disable extended menus | - | - | O | - |
| Meters | Reset call meters | - | - | O | - |
| Add-char | Additional characteristics | - | - | O | - |
| Add-info | Additional Information | - | - | O | - |

Table x: User interface customisation

Legend:-

M	the user interface customisation is mandatory and supported by the MS

O 	the user interface customisation is optionally supported by the MS, and subject to the capabilities of the terminal.  If supported by the MS, the extent of support of the user interface customisation is purely within the manufacturer’s discretion

-	the user interface customisation is not supported by the MS

## 4.7	Management of services

- In this clause the term service may be an application, applet, and its related content.

### 4.7.1	Service configuration

- A transferred service may be automatically configured in the MS user interface using icons, browsers or menus as applicable.  The service name may be contained in the package in which it was received (i.e. a JAR file or script.).  If enabled, the user is notified of its automatic configuration.