ETSI SMG4 MExE	Tdoc 4M99-074

Fort Worth, Dallas

12-14.1.1999

| CHANGE REQUEST No : |  |  |  |  |  |  | . |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 3.57 |  | Version |  | 1.3.0 |  |  |  |
| Submitted to SMG | 4 |  | for approval |  | x | without presentation ("non-strategic") |  |  |  | x |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE |
| --- | --- |

| Source: | Alcatel Business System | Date: | 14.01.1999 |
| --- | --- | --- | --- |

| Subject: | Language value coding for negotiation language token |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | ISO standard has already define a language coding and country coding. This coding is already used in GSM (SMS CB language, SIM file language preference) and in HTTP |
| --- | --- |

| Clauses affected: | 2.1 and 4.4.1 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

## 2.1	Normative references

[1]	GSM 01.04 (ETR 350): “Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms”.

[2]	GSM 02.57: "Digital cellular telecommunications system (Phase 2+);MExE Stage 1 Description"

[3]	Personal Java, this specification is available from Sun Microsystems’s website at http://java.sun.com/products/personaljava/index.html.

[4]	MNCRS WG 4 specifications are still under development. When completed, MNCRS WG 4 specifications will be available at http://www.mncrs.org/.

[5]	JTAPI 1.2, this specification, currently in the ECTF approval process, is available from Sun Microsystems’s website at http://www.java.sun.com.

[6]	Wireless Application Protocol (WAP) version 1.0, this specification is available from WAP‘s website at http://www.wapforum.org.

[7]	vCard – The Electronic Business Card Exchange Format – Version 2.1, The Internet Mail Consortium (IMC), September 1996, http:/www.imc.org/pdi/vcard-21.doc.

[8]	vCalendar – The Electronic Calendaring and Scheduling Exchange Format – Version 1.0, The Internet Mail Consortium (IMC), September 1996

[9]	Hypertext Transfer Protocol – HTTP/1.1, IETF document RFC2068

[10]	Java Mail API version 1.0.2, available from http://www. java.sun.com

[11]	UMTS TR 22.70: “Universal Mobile Telecommunications System (UMTS); Service aspects; Provision of Services in UMTS - The Virtual Home Environment”.

[12]	UMTS TS 22.xx: “Universal Mobile Telecommunications System (UMTS); Provision of Services in UMTS - The Virtual Home Environment: Stage 1”.

[13]	ISO 639 International Standard code for the representation of name of languages

[14]	ISO 3166 International Standard code for the representation of name of countries

### 4.4.1	Capability negotiation characteristics

The generic set of capabilities which may be negotiated between the client and the server consists of the subsequently identified characteristics.:-

| Token | Characteristics |  |  | Optionality |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Client to MSE | MSE to Client |  |
| MExE-class | MExE Classmark |  |  | M | O | Classmark of the MExE device, e.g. 1, 2, etc. |
| MExE-manuf | Device | Manufacturer |  | O | O | Identification of the manufacturer, e.g. Motorola |
| MExE-model |  | Model |  | O | O | Model type, e.g. StarTac 130 |
| MExE-serial-no |  | Serial Number |  | O | O | Manufacturer’s model, e.g. 1.1.0 |
| MExE-software |  | SoftwareVersion |  | O | O | Manufacturer’s software version, IMEISV |
| MExE-resolution | Display | Size | resolution | O | O | Pixel size of the display, e.g. 200x150 etc. |
| MExE-chars |  |  | Characters | O | O | Number of lines and characters per line, e.g. 3x20 |
| MExE-colours |  | Colour/monochrome | Number of colours | O | O | The number of colours supported, e.g. 16,000 |
| MExE-greyscale |  |  | Greyscale | O | O | The number of gray scale, e.g. 2 for black and white only |
| MExE-voice | Input mode | Voice |  | O | O | Indication of whether voice capture is supported, i.e. yes/no |
| MExE-keyboard-id |  | Keyboard type | Keyboard identity | O | O | Type of keyboard supported |
| MExE-keyboard-lang |  |  | Human language | O | O | Language used on keyboard, i.e. English -UK, Swedish-Finland, Norwegian-Bokmal, German-Swiss etc. Example coding: MExE-kbdlg: EN-UK (see ref 13 nd 14) |
| MExE-keys |  |  | Number of keys | O | O | The number of keys supported by the keyboard, i.e. 102 |
| MExE-pointer |  | Pointing device | Identity | O | O | Type of pointing device, i.e. touchscreen, mouse |
| MExE-human-lang | Human Language |  |  | O | O | Preferred language in which the user wishes to work , i.e English-UK, Swedish-Finland, Norwegian-Bokmal, German-Swiss etc. Example coding: MExE-hlang: EN-UK (see ref 13 and 14) |
| MExE-formal-lang | Formal Language |  |  | O | O | Prioritised list of renditions for formal languages, e.g. HTML, WML |
| MExE-protocols | Additional Protocols |  |  | O | O | List of additionally supported protocols of the MExE device, e.g. SIM Toolkit |
| MExE-bearers | Supported Bearers |  |  | O | O | Prioritised list of preferred bearers, e.g. SMS, USSD, GPRS |
| MExE-classmark-differentiator | Difference to base Classmark |  |  | O | O | Identification of the devices’ components differing from the base Classmark definition |
| MExE-memory-type | User authorised available memory | Type of memory |  | O | O | Type of memory, e.g. RAM, flash memory, hard disk |
| MExE-memory |  | Amount of memory |  | O | O | Amount of memory authorised by the user for MExE, e.g. xxxKb |
| MExE-audio-codec | Multimedia | Audio codec type |  | O | O | Prioritised list of preferred audio codecs (cf. H.323) |
| MExE-video-codec |  | Video codec type |  | O | O | Prioritised list of preferred video codecs (cf. H.323) |
| MExE-trsf-size | Transfer limits | Size |  | O | O | Maximum size of transfers |
| MExE-trsf-time |  | Time |  | O | O | Maximum duration of transfers |
| MExE-loc-support | Location Information | Support of location information |  | O | O | How, and whether, location information is supported, e.g. No location information supported, GPRS, cell, other |
| MExE-location |  | Location information |  | O | O | Actual location information |
| MExE-user-profile | User profile |  |  | O | O | Transfer of the User Profile |
| MExE-add-chars | Additional characteristics |  |  | O | O | Support of additional characteristics, e.g. smart card reader, docking station, etc. |
| MExE-add-info | Additional Information |  |  | O | O | Manufacturer defined additional characteristics |

Table 1: Capability negotiation characteristics