ETSI SMG4 MExE	Tdoc 4M99-xxx

Helsinki, Finland

17-19.2.1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.4.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | x | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | 03.57 |
| --- | --- |

| Source: | Bosch Telecom | Date: | 12.02.1999 |
| --- | --- | --- | --- |

| Subject: | Descriptions in Capability negotiation characteristics |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | The descriptions of some of the tokens in the Capability negotiation characteristics table is unclear. |
| --- | --- |

| Clauses affected: | 4.4.1 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

### 4.4.1	Capability negotiation characteristics

The generic set of capabilities which may be negotiated between the client and the server consists of the subsequently identified characteristics.:-

| Token | Characteristics |  |  | Optionality |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MS to MSE | MSE to MS |  |
| MExE-class | MExE Classmark |  |  | M | O | Classmark of the MExE device, e.g. 1, 2, etc. Example coding: MExE-class: 1 |
| MExE-Spec | MExE specification |  |  | O | O | Version of the MExE Stage 2 standard supported by the MS; represents the first two digits of the 03.57 specification version number. Example coding: MExE-Spec: 7.0 |
| MExE-srvcs | MExE services |  |  | O | O | Identification on where information on the PLMN-provided MExE services may be found, e.g. URL Example coding: MExE-srvcs: http:://telco.mexe.com/services |
| MExE-manuf | Device | Manufacturer |  | O | O | Identification of the manufacturer, e.g. Motorola, Nokia, Ericsson, etc Example coding: MExE-manuf: Motorola |
| MExE-model |  | Model |  | O | O | Model type, e.g. 130, 8810, 688, etc. Example coding: MExE-model: 130 |
| MExE-SN |  | Serial Number |  | O | O | Manufacturer’s model, e.g. 1.1.0 etc. Example coding: MExE-SN: 1.1.0 |
| MExE-SV |  | SoftwareVersion |  | O | O | Manufacturer’s software version, IMEISV Example coding: MExE-SV: 45007867414563701 |
| MExE-res | Display | Size | resolution | O | O | Pixel size of the display, e.g. 200x150 etc. Example coding: MExE-resol: 200x150 |
| MExE-chars |  |  | Characters | O | O | Number of lines and characters per line, e.g. 3x20, 4x15 etc. Example coding: MExE-chars: 3x20 |
| MExE-clrs |  | Colour/monochrome | Number of colours | O | O | The number of colours supported, e.g. . 2, 256, 16,000 etc. Example coding: MExE-clrs: 256 |
| MExE-grey |  |  | Greyscale | O | O | The number of gray scale, e.g. 2 for black and white only Example coding: MExE-grey: 4 |
| MExE-voice | Input mode | Voice |  | O | O | Indication of whether voice capture is supported, e.g. yes/no Example coding: MExE-voice: yes |
| MExE-kbdid |  | Keyboard type | Keyboard identity | O | O | Type of keyboard supported, e.g. ME-on/off-soft2-arrow2, ME-soft2-arrow4-clr, PC-AT-102 Example coding: MExE-kbdid: ME-soft2-arrow2 |
| MExE-kbdlg |  |  | Human language | O | O | Language used on keyboard (ISO 639 [13]), i.e. EN-UK etc. Example coding: MExE-kbdlg: EN-UK |
| MExE-keys |  |  | Number of keys | O | O | The number of keys supported by the keyboard, i.e. 12, 15, 102, etc. Example coding: MExE-keys:12 |
| MExE-pntr |  | Pointing device | Identity | O | O | Type of pointing device, i.e. none, touchscreen, mouse, etc. Example coding: MExE-pntr: none |
| MExE-hlang | Human Language |  |  | O | O | Preferred language in which the user wishes to work (ISO 639 [13]), i.e EN-UK etc. Example coding: MExE-hlang: EN-UK |
| MExE-flang | Formal Language |  |  | O | O | Prioritised list of renditions for formal languages, e.g. HTML, WML, etc. Example coding: MExE-flang: WML/HTML |
| MExE-adprt | Additional Protocols |  |  | O | O | List of additionally supported protocols of the MExE device, e.g. SAT (SIM Application Toolkit) Example coding: MExE-adprt: SAT |
| MExE-bear | Supported Bearers |  |  | O | O | Prioritised list of preferred bearers, e.g. SMS, USSD, GPRS, etc. Example coding: MExE-bear: GPRS/SMS |
| MExE-cldif | Difference to base Classmark |  |  | O | O | Identification of the devices’ components differing from the base Classmark definition Example coding: MExE-cldif: Java.rmi |
| MExE-memty | User authorised available memory | Type of memory |  | O | O | Type of memory, e.g. RAM, memory, hard disk, etc. Example coding: MExE-memty: RAM |
| MExE-mem |  | Amount of memory |  | O | O | Amount of memory in Kb authorised by the user for MExE, e.g. 1, 6, 10, etc. Example coding: MExE-mem: 1 |
| MExE-audio | Multimedia | Audio codec type |  | O | O | Prioritised list of preferred audio codecs (cf. H.323) e.g. G.711, G.723 etc. Example coding: MExE-audio: G.711 |
| MExE-video |  | Video codec type |  | O | O | Prioritised list of preferred video codecs (cf. H.323), e.g. H.261, H.263 Example coding: MExE-video: H.261 |
| MExE-tsize | Transfer limits | Size |  | O | O | Maximum size of transfers in Kb, e.g. 1,2, 4, 8, etc. Example coding: MExE-tsize: 4 |
| MExE-ttime |  | Time |  | O | O | Maximum duration of transfers in minutes, eg. 1,2, 4, 8, etc. Example coding: MExE-tsize: 2 |
| MExE-loc | Location Information | Support of location information |  | O | O | Type of location information supported, if any, e.g. No, GPRS, cell, other E-OTD, TOA, etc. Example coding: MExE-loc: E-OTD |
| MExE-locin |  | Location information |  | O | O | Actual location information (as defined by type) Example coding: MExE-locin: xxxxxxxxxxxxxxx |
| MExE-uprofile | User profile |  |  | O | O | Transfer of the User Profile; individual characteristic(s) of the profile Example coding: MExE-uprof: nnnnnnnn |
| MExE-URL |  |  |  | O | O | An indirect reference to a location where the MExE MS’s characteristics may be obtained Example coding: MExE-URL: http:://name.co.com/profile |
| MExE-addch | Additional characteristics |  |  | O | O | Support of additional characteristics, e.g. none, smart card reader, docking station, etc. Example coding: MExE-addch: docking-station |
| MExE-addin | Additional Information |  |  | O | O | Manufacturer defined additional characteristics; free format Example coding: MExE-addin: XYZ |

Table 1: Capability negotiation characteristics

The support of the above characteristics between the MS and the MSE is either mandatory (marked with an M) or optional (marked with an O), as indicated in the Optionality column.  M indicates that it must be sent by that entity, and O indicates that it may be sent by that entity if it supports it.  A characteristic marked as O, does not necessarily require to be supported by the receiving entity.

Generally, the combination of user profile preferences and ME logic will determine the information sent in the capability negotiation from the MExE device to the MExE Service Environment.  As an example, for the support of location information the user’s profile controls  if and when location information may be sent to the MExE Service Environment (e..g. never sent, always sent, only after user confirmation).

The capability negotiation process shall be used by both the client and the server to permit transfer of capabilities from one to the other.  By transferring its capabilities, the client or server will support efficient use of resources both over the radio interface as well as in the client or server.  Capability negotiation shall be performed by the server prior to transfer over the radio interface to verify as far as possible the ability of the client to support any services to be downloaded.

In order to transfer the capability information between the MExE MS and the MSE, “tokens” are identified for each transferable characteristic in the HTTP request and response headers.