| TSG-T2 / SMG4 MExE 15th - 16th July 1999 Tampere, Finland | Tdoc T2X99xxx |
| --- | --- |

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 7.0.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Motorola | Date: | 12 July 1999 |
| --- | --- | --- | --- |

| Subject: | CCM Length and timestamp fields and figure update |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 | x |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | The length field in the header of a CCM must not include the length of the signature located at the end of the message. This allows a device to proceed directly to the signature without having to first parse the entire fingerprint list. This change will allow the process of decoding the CCM to be far more efficient. In order to be able to detect replay attacks, the header of a CCM must include a timestamp. Correction of the length field definition, adding a timestamp and signer info fields will ensure consistency between CCMs defined by the WAP forum and MExE. Figures 8 and 9 are redrawn in a new figure 9 with additional explanatory text to remove any possible ambiguity. |
| --- | --- |

| Clauses affected: | 8.12 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 8.12	Certificate configuration message (CCM)

The MExE device shall use the CCM to determine the third party certificates (and only the TTP certificates) that are trusted for use on the MExE MS. The CCM may be periodically fetched or downloaded to a MExE device by the Administrator to dynamically configure the trusted third party list using the mechanisms defined in section 8.15.2. The Certificate Configuration Message shall be as shown in Figure 8. This message is essentially a simplified version of a certificate revocation list to satisfy a particular use case. More complex usage requires a full certificate revocation list.

The MExE device may additionally support other means of enabling/disabling root certificates.

### 8.12.1	CCM Numbering convention

Bits are grouped into octets. The bits of an octet are shown horizontally and are numbered from 0 to 7. Multiple octets are shown vertically and are numbered from 0 to n.

### 8.12.2	CCM Order of transmission

Frames are transferred in units of octets, in ascending numerical octet order (i.e., octet 0, 1, , n-1, n). The order of bit transmission is specific to the underlying protocols used to transport the CCM.

### 8.12.3		CCM Field mapping convention

When a field is contained within a single octet, the lowest bit number of the field represents the lowest-order value. When a field spans more than one octet, the order of bit values within each octet progressively decreases as the octet number increases. In that part of the field contained in a given octet the lowest bit number represents the lowest-order value.

For example, a 16 bit number can be represented as shown in Figure 8.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| 215 | 214 | 213 | 212 | 211 | 210 | 29 | 28 |  | 1st Octet of field |
| 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |  | 2nd Octet of field |

Figure 8: Field mapping convention

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  | Octet |
| Version |  |  |  |  |  |  |  |  | 0 |
| Certificate Advice |  |  |  |  |  |  |  |  | 1 |
| Issued Timestamp . . |  |  |  |  |  |  |  | 2 – 8 |  |
| Expiry Timestamp . . |  |  |  |  |  |  |  | 9 –15 |  |
| SignerInfo |  |  |  |  |  |  |  | 16 |  |
| Listlength |  |  |  |  |  |  |  | 17 |  |
| Listlenght |  |  |  |  |  |  |  | 18 |  |
| Fingerprint 1 Hashtype |  |  |  |  |  |  |  | 19 |  |
| Hashvalue .(Variable length k according to hash type) . . |  |  |  |  |  |  |  | 21 |  |
| Fingerprint 2 Hashtype |  |  |  |  |  |  |  | k+21 |  |
| Hashvalue (Variable length m according to hash type) . . |  |  |  |  |  |  |  |  |  |
| Fingerprint 3 |  |  |  |  |  |  |  | m+k+21 |  |
| . . . |  |  |  |  |  |  |  |  |  |
| Fingerprint n . . |  |  |  |  |  |  |  |  |  |
| CCM Signature Hashtype |  |  |  |  |  |  |  | Listlength+21 |  |
| Administrator signature (Variable length according to administrator signature type) |  |  |  |  |  |  |  |  |  |

Figure 9 Format of a CCM

version = For MExE-98 the CCM format version is 0. All other values are reserved for future use.

certificateAdvice = enumerated { enable all present and future TTP certificates (0), disable all present and future TTP certificates (1), enable present list only (2),enable CCM list (3), disable CCM list (4) }.  All other values are reserved for future use.

Issue and Expiry Timestamps = Fields used to identify the issue and expiry date of the CCM.  The issue timestamp indicates a time before the current time of day (GMT) when a CCM message must be considered invalid. The expiry timestamp (GMT) identifies the time when a CCM is to be deemed no longer valid. The receiver shall use these parameters to detect a replay attack. A MExE MS maintains information on the last valid CCM message received. A replay attack is an attacker replaying a previous valid CCM message to a MS in order to change the security settings. This is particularly dangerous for CCM messages used to enable certificates. Administrators should try and  set the expiration time to be no longer than the next expected system update time of CCM information. CCM messages used to enable-all (rather than disable-all) certificates should be very short lived as the danger of these being used in a replay attack should be considered serious.

The encoding of time (GMT) shall be coded as an OCTET STRING of  seven octets in length as follows:

| Octet 0 | 1 | 2 | 3 | 4 | 5 | Octet 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Year |  | Month | Day | Hour | Minute | Second |

| Element | Size (bits) | Range |
| --- | --- | --- |
| Year | 16 | 0 – 65535 |
| Month | 8 | (1 – 12)10 |
| Day | 8 | (1 – 31)10 |
| Minute | 8 | (0 – 59)10 |
| Second | 8 | (0 – 60)10 |

For example, January 1st, 2001 00:00:30 would  be encoded as: 07 d1 01 01 00 00 1E.

SignerInfo = one octet indicating the type of signer information for this CCM. The only currently defined value is device-admin = 0. In this case, no further signer information follows as it is implicit.

listLength = The total length of the  fingerprint list not including the final CCM signature. Shall be zero when certificateAdvice = enable-all or disable-all.

hashType = enumerated { signature (0), MD5 (1), SHA-1 (2) } All other values are reserved for future use.

hashLength = The number of octets output by the selected hash type (16 for MD5 [23] and 20 for SHA-1 [24]).

The list entries shall contain certificate fingerprints in the form of hashes of the encoded signed certificates. The full hash output for the specified algorithm shall be used to generate the fingerprint. A list generator shall check to insure that no two list entries match when creating a list. For an X509v3 [26] or X9.68 (currently being drafted) certificate the fingerprint hash shall be computed over the ASN.1 encoded signed certificate object, first octet to last octet. For WTLS certificates the hash shall be computed over the signed WTLS certificate in network transmission format, first octet to last octet.

The signature type and length shall be indicated by the administrator certificate, which shall be present on the device. If no administrator certificate is on the device or the signature does not verify the message shall be rejected.

Upon receipt of a valid certificate configuration message the MExE device shall go through the trusted third party certificate list, computing fingerprints if they are not stored with the certificate, enabling or disabling each certificate according to the following conditions

- certificateAdvice is enable-all		all TTP certificates shall be enabled

- certificateAdvice is disable-all		all TTP certificates shall be disabled

- certificateAdvice is enable present list only	enable all TTP certificates currently on device, do not enable any future certificates (this option allow the list to be frozen at time of manufacture) until Administrator changes

- certificateAdvice is enable-list		if its fingerprint occurs in the CCM, it shall be enabled, otherwise it shall be disabled

- certificateAdvice is disable-list	if its fingerprint occurs in the CCM, it shall be disabled, otherwise it shall be enabled

For future releases, the setting of fine grained permissions for each certificate is expected to be supported.

An implementation shall keep track of the domain that authorised a given application. If a CCM message is received while MExE applications are currently running the implementation shall check to ensure any applications no longer in a trusted domain have their permissions re-configured appropriately and actions that are no longer permissible are terminated