| 3GPP TSG-T2 #12 Beverly Hills, USA February 12th - February 16th 2001 |  |  |  |  |  |  |  |  | T2-010071 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.057 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Administrator mechanism |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Motorola/Nokia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MEXE-ENHANC |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 28/01/2001 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-4 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The administrator mechanism requires to be updated to reflect the fact that MExE (U)SIM may now contain administrator certificates. The currently described mechanism originates from the date when it was presumed that they the administrator certificate could not be carried on the MExE-(U)SIM, and still refers back to Release 98. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The mechanism is still retained in two parts. The first part is clarified to determine who the administrator of the MExE device is, by now first checking the MExE-(U)SIM, and secondly checking the MExE device. The text is also cleaned up to remove references to some "future mechanism" as it is now supportable with MExE-(U)SIMs carrying certificates. Note that corrections to terminology (i.e. SIM, MExE ME) are corrected in a different CR. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

### 8.8.1	Determining the administrator of the MExE MS

The administrator of the MExE MS shall be determined by a two part logical process with the first part shown in the flowchart in Figure 11 "MExE administrator determination mechanism". The second part of the logical process is in Figure 12 "MExE administrator determination mechanism, for MExE-(U)SIM supporting third party certificates".

#### 8.8.1.1	Determining the administrator of the MExE device

During power-up or MExE-(U)SIM insertion event, the provisioned mechanism shall look for an administrator root public key that is stored on the MExE-(U)SIM.  If an administrator root public key cannot be found on the MExE-(U)SIM, the provisioned mechanism shall look for one on the MExE device.  This leads to the following two cases:-

- administrator root public key is absent

if the administrator root public key is absent, then the user shall automatically become the administrator of the MExE device.

- administrator root public key is present

if an administrator root public key is present, this root public key shall be used for all remote administration authentication, implying that the owner of the administrator root public key is the administrator.

Figure 11: MExE administrator determination mechanism

#### 8.8.1.2	Determining the administrator of the MExE device, for MExE-(U)SIM supporting third party certificates

The second part of the administrator determination mechanism is subsequently defined(see Figure 12 "MExE administrator mechanism, for MExE-(U)SIM supporting third party certificates),and shall be initiated after a power-up or MExE-(U)SIM insertion event is processed.

The following subclauses 8.8.1.2.1 "Administrator of the MExE device is the user" and 8.8.1.2.2 "Administrator of the MExE device is not the user" assume that Third Party certificates can be added using the MExE-SIM, however Third Party certificates may be added using a non-SIM approach(e.g. inserted at the time of manufacture, signed package download etc.).

##### 8.8.1.2.1	Administrator of the MExE MS is the user

If the administrator is the user, then a check shall be made to determine whether there is a MExE-SIM. If a MExE-SIM is present, then a check shall then be made to determine whether there is a certificate in the MExE-SIM. The second part of the administrator determination mechanism shall allow the MExE MS to determine (via a format) what type of certificate is present:

- certificate present - third party (CP-TP)

A certificate present in the MExE-SIM shall be considered by the ME as a Third Party certificate, whilst that MExE-SIM is inserted in the ME. The user shall be queried to allow or disallow the certificate as a Third Party.

- certificate present - administrator (CP-Admin)

If a temporary certificate is present in the MExE-SIM, the user shall be queried whether to allow the certificate on the MExE-SIM to take temporary control of the third party domain. By temporary control, it is meant that once the card is removed the administrator reverts back to the user administrator settings. The above mechanism implies that the previous configuration settings for the administrator shall be saved, so that they may be restored. If the user disallows the MExE-SIM certificate, the Third Party Domain shall not be able to use any of the network capabilities in the third party domain as identified in the network access section of the security Table 6 "Security domains and actions".

If an administrator certificate is not present on the MExE-SIM and the administrator is the user, the user shall continue to be the administrator and may make use of all functionality.

##### 8.8.1.2.2	Administrator of the MExE MS is not the user

If the administrator is not the user, then a check is made to determine if there is a MExE-SIM. If a MExE-SIM is present, then a check is made to see if there is a certificate in the MExE-SIM. If a certificate is present in the MExE-SIM, then a comparison is made of the certificate’s root public key on the MExE-SIM with the root public key on the ME for the following cases:

- Case (a): they are the same;

- Case (b): they are not the same, but the ME certificate is cross-certified with the MExE-SIM certificate (a cross-certificate exists on the ME);

- Case (c): they are not the same, but the ME certificate has a line of trust back to the MExE-SIM certificate domain;

- Case (d): they are not the same.

If the owner of the public key in the certificate on the MExE-SIM is to be a temporary administrator (CP-Admin), then in cases (a), (b) and (c), the temporary administrator shall be the owner of the CP-Admin root public key. In case (d), the Third Party domain shall not use any of the network capabilities in the third party domain as identified in the network access section of the security Table 6 "Security domains and actions". If the certificate is to be a Third Party, then the certificate (CP-TP) shall be verified with the CCM and based on the content and permissions of the CCM, the certificate shall be added to the Third Party list or rejected.

CR Editor's Note: above figure changed, but not shown by revision marks.  The title of the flowchart was changed, CP-TTP was changed to CP-TP, and MExE-SIM was changed to MExE-(U)SIM

Figure 12: MExE administrator determination mechanism, for MExE-(U)SIM supporting third party certificates