| 3GPP TSG-T2 /ETSI SMG4 Whistler, Canada 272th - 29th March 2001 |  |  |  |  |  |  |  |  | T2-01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.057 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | CLI capability and content negotiation (Classmark 4, non-security) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Microsoft Corporation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MEXE-EHANC |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 03/02/2001 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | Introduction of a new MExE classmark based on CLI Compact Profile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Modification to capability and content negotiation sections. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | No capability and content negotiation for Classmark 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 4.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 1: UAProf properties supported by MExE

| Mandated Properties |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Attribute | Description |  |  |  | Type | Sample |
| MexeClassmarks | List of supported MExE classmarks Note: in pre-release 4.0.0 specifications the attribute MexeClassmark (as opposed to MexeClassmarks) which was a literal (as opposed to as Literal, Bag) indicating only one MExE classmark was notified. |  |  |  | Literal (bag) | "1", "2", "3" |
| MexeSpec | The first two digits of the MExE Specification version that the device conforms to |  |  |  | Literal | "3.3", "4.1" |
| MexeSecureDomains | Indicates whether the device supports the MExE security domains |  |  |  | Boolean | "Yes", "No" |
| Recommended Properties |  |  |  |  |  |  |
| Vendor | UE vendor |  | Literal |  |  | "Lexus", "Ford" |
| Model | UE model number |  | Literal |  |  | "Mustang 90", "Q10" |
| SoftwareNumber | The number of the device specific software |  | Literal |  |  | "1.0", "2.7.0" |
| ScreenSize | The size of the device's screen in units of pixels. |  | Dimension |  |  | "160x160", "640x480" |
| ScreenSizeChar | Size of the device's screen in units of characters (based on the standard font). |  | Dimension |  |  | "12x4", "16x8" |
| ColorCapable | Whether the device display supports colour |  | Boolean |  |  | "Yes", "No" |
| AudioInputEncoder | List of audio input encoders supported by the device |  | Literal (bag) |  |  | "G.711" |
| VideoInputEncoder | List of video input encoders supported by the device |  | Literal (bag) |  |  | "MPEG-1", "MPEG-2", "H.261" |
| PointingResolution | Type of resolution of the pointing accessory supported by the device |  | Literal |  |  | "Character", "Line", "Pixel" |
| CcppAccept-Language | List of preferred document languages |  | Literal (bag) |  |  | "zh-CN" "en fr" |
| Keyboard | Type of keyboard supported by the device as an indicator of ease of text entry. |  | Literal |  |  | "Disambiguating", "Qwerty", "PhoneKeypad" |
| SupportedBearers | List of bearers supported by the device. |  | Literal (Bag) |  |  | "GPRS", "GUTS", "SMS", "CSD", "USSD" |
| JavaPlatform | List of Java platforms and profiles installed on the device |  |  | Literal (Bag) |  | "Pjava/1.1.3-compatible", "MIDP1.0-compatible", "J2SE/1.0-compatible" |
| Proposed New Properties |  |  |  |  |  |  |
| CLIPlatform | List of CLI profiles installed on the device |  |  |  | Literal (Bag) | “CLI-Compact/1.0-compatible” |

Generally, the combination of user profile and ME logic will determine the information sent in the capability negotiation from the MExE device to the MExE Service Environment.  As an example, for the support of VideoInputEncoder information the user’s profile controls  if and when VideoInputEncoder information may be sent to the MExE Service Environment (e.g. never sent, always sent, only after user confirmation).

The capability negotiation process shall be used by the client to permit transfer of capabilities from the client to the server.  By transferring its capabilities, the client will support efficient use of resources both over the radio interface as well as in the client or server.  Capability negotiation shall be performed prior to transfer over the radio interface to verify as far as possible the ability of the client to support any services to be downloaded.

In order to transfer the capability information between the MExE MS and the MSE, CC/PP method is used with the schema defined in the WAP UAProf working group.

### 4.6.2	CC/PP over WSP (Classmark 1)

In Classmark 1 the CC/PP is carried over by using CC/PP over WSP, [17].

### 4.6.3	CC/PP over HTTP (Classmark 2)

In Classmark 2 the CC/PP is carried over by using CC/PP over HTTP, [15] and optionally CC/PP over WSP, [17].

### 4.6.4	Transfer of capability negotiation information in Classmark 3

In Classmark 3 the CC/PP is carried over by using CC/PP over HTTP, [15] and optionally CC/PP over WSP, [17].

Also MIDP itself provides a simple mechanism for applications to indicate the capabilities they require. The Java Application Descriptor File (JAD), which is a file stored and downloaded separately to the application itself, contains information such as application name, version number, JAR file size, data storage requirements etc. The Application Descriptor accompanies the JAR file and can be used to ensure prior to the actual application download that the application suits the device. The JAD file is described in more details in the section 6.2.2.2.2 " MID Applications (MIDlet)".

### CC/PP over HTTP or WSP (Classmark 4)

In Classmark 4 the CC/PP is carried over by using CC/PP over HTTP,[15] and optionally CC/PP over WSP, [17]