| 3GPP TSG-T2 /ETSI SMG4 Whistler, Canada 272th - 29th March 2001 |  |  |  |  |  |  |  |  | T2-01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.057 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Classmark 4 security domains |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Microsoft Corporation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MEXE-EHANC |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 03/02/2001 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | Classmark 4 applications reduce the risk of user error by allowing aggregation of user permission requests. Such functionality requires a modification to the requirements of user access within security domains. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Addition of permission being granted on a multiple action basis. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Classmark 4 applications may require a great deal of user interaction for the permissions they require, which may result in errors. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 8.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 8.2	MExE executable permissions

Support of MExE executable permissions as detailed in this subclause is mandatory.

### 8.2.1	MExE executable permissions for operator, manufacturer and third party security domains

The following Table 6 "Security domains and actions" specifies the permissions of operator, manufacturer and third party security domains in the order of restriction.

The actions listed in the security Table 6 "Security domains and actions" are generic actions. These actions can only be performed by MExE executables via application programming interfaces (APIs) (which are intrinsically part of the MExE implementation) The security restrictions shall apply to MExE executables whether the API functionality is called directly or indirectly by the MExE executable. Explicit user permission is required for all actions by MExE executables in all domains. Types of user permission are defined in subclause 8.3 User permission types.

For Classmark 4 executables, user permission can be granted for each executable on a single or multiple action basis.

Untrusted MExE executables are not permitted access to any actions which access the phone functionality (phone functionality includes all the actions in Table 6 "Security domains and actions") except for the exceptions identified in 8.2.2 "MExE executable permissions for untrusted MExE executables".

Actions available using interfaces giving access to the phone functionality (either in existence at the time of approval of this specification or not) that are not listed in the security Table 6 "Security domains and actions" shall be categorised into one of the groups in the security Table 6 "Security domains and actions" by comparing its action against the groups in order as they are listed in the Table 6 "Security domains and actions". If an action can be categorised into a more restrictive group near the top of the table, then it shall not be again categorised into another, less restrictive, group further down in the table. E.g if a new action eventually results in forwarding a call, it shall be categorised into Network access. If the action is totally new, it shall be categorised into some of the groups by comparing its functionality to the group description below and by comparing with the list of actions listed in the table within the group.

- Device core function access includes functions, which are an essential part of the phone functionality .

- SIM smart card low level access includes functions, which allow communications at the transport service access point (send and receive application protocol data unit).

- Network security access includes all functionalities which relate to CHV, CHV2, UNBLOCK CHV and UNBLOCK CHV2 (verification, management, reading or modifying), GSM authentication, GSM ciphering.

- Network property access includes functions, which enable the management of operator-related data parameters and network settings.

- Network services access includes all functionalities which result in or need interaction via the operator´s network.

- User private data access includes all functionalities which relate to management, reading or modifying of data that the user has stored in the MS including user preferences.

- MExE security functions access includes all functionalitiess which, through an API relate to certificate handling in the MS; end to end encryption, signed content, hashing, access to public, private, secret keys stored in the MS or in a smart card.

- Application access includes the functionalities which relate to launch provisioned functionality, MExE executables, external executables (SIM tool kit application,…) usage.

- Lifecycle management includes the functionalities which are needed for installing or removing MExE executables in the MS.

- Terminal data access includes the functions which relate to accessing terminal data, i.e. not user data.

- Peripheral access includes the functionalities related to peripherals other than user interface peripherals usage through a high level software application interface.

- Input output user interface access includes the functionalities related to the user interface and user notification means usage.

Table 6: Security domains and actions

|  |  | MExE Security Domains |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Actions |  | Operator |  | Manufacturer |  |  | Third Party |  |  |
| Device core function access - Start/stop radio - Turn on/off device - Write time and/or date - Activate a user profile - Modify a user profile | No |  |  |  |  |  |  |  |  |
| Support of Core Software Download e.g. Update UE software | No |  |  |  |  | Yes |  | No |  |
| SIM smart card low level access11 - Send APDU - Slot management (power on/off, reset, port lock…) | No |  |  |  |  |  |  |  |  |
| 11 – Access to SIM is provided using more high level API as phonebook, application launching |  |  |  |  |  |  |  |  |  |
| Network Security access - Run algorithm - Verify CHV/2 or UNBLOCK CHV/2 - Activate/deactivate CHV - Modify CHV/2 | No |  |  |  |  |  |  |  |  |
| Network property access - Get IMSI - Get home network - Select network | Yes |  |  |  | No |  |  |  |  |
| Network services access - Initiate a voice/data connection 3 - Accept a voice/data connection 3 - Call forward 4 - Multiparty call 4 - Call deflection 4 - Explicit call transfer 4 - Terminate an existing connection - Hold an existing connection - Resume an existing connection - Send point-point message (e.g. SMS, USSD) 4 - Generate DTMF - Query network status - Get signal level - Get call list - QoS management |  | Yes |  |  |  |  |  |  | Yes 6 |
| 3 – A network connection may be via any supported bearer service 4 – Multiparty, deflection, and explicit call transfer shall be permitted only to numbers explicitly supplied by the user to the MExE Executable. Modification of call forward numbers stored in the network shall only be permitted to numbers explicitly supplied by the user to the operator. 6– The Third Party domain's permission to access the networking action depends on the provisioning mechanism as described in subclause 8.8.1 "Determining the administrator of the MExE UE" |  |  |  |  |  |  |  |  |  |
| User private data access 1 - Read - Write - Get properties - Delete - Get Location Information - Read stored SMS - Delete stored SMS - Modify user preferences |  | Yes2 Yes2 Yes2 Yes2 Yes2 Yes2 Yes2 Yes7 |  |  |  |  |  |  |  |
| 1 – User private data includes user files, phonebook, etc located on the MS. 2 – The user shall be able to specify data access permissions within the capabilities of the device. It is not applied to user preferences 7 – Trusted applications only have permission to modify user preferences, and not to activate or de-activate them. The user shall be able to specify for each domain, the preferences that applications in that domain can access. All other preferences shall not be accessible to that domain. The default shall be that there is no access. Single action user permission is the only type of user permission that shall be possible for changes to User Preferences. |  |  |  |  |  |  |  |  |  |
| MExE security functions access - Install a certificate for a given domain - Uninstall a certificate for a given domain - Replace a certificate for a given domain - Data encryption API - Verify a signature API - Compute a digital signature API - Hash a content API - Non repudiation API |  | Yes 5 Yes 5 Yes 5 Yes Yes Yes Yes Yes |  |  |  |  |  |  |  |
| 5 – Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate. |  |  |  |  |  |  |  |  |  |
| Application access - Get application list - Launch an application - Get application status - Stop, suspend, resume an application | Yes8 Yes8 Yes8 Yes9 |  |  |  |  |  |  |  |  |
| 8 – Device provisioned functionality access is limited to manufacturer domain. SIM tool kit application access is limited to operator domain. MExE executable access is limited to MExE executable issued by the same issuer (identify by the certificate) of launched MExE executable 9 – Access is limited to MExE executable which launch the application. But the end user, shall have a way to stop the launched application, MExE environment may stop the launched application or launched application may stop itself. |  |  |  |  |  |  |  |  |  |
| Lifecycle management - Install a MExE Executable - Uninstall a MExE executable |  | Yes |  |  |  |  |  |  |  |
| Terminal data access - Get manufacturer software version - Read time and date |  |  | Yes Yes |  |  |  |  |  |  |
| Peripheral access - Sound generation to speaker (e.g. via stream) - Set speaker volume - printer access - Monitor the power state - Change the power state - Activate/ access Serial port (RS232, Irda, Bluetooth, USB …) access - Activate/access Parallel port - Activate/access Smart card other than SIM card (Send APDU, Slot management) |  | Yes |  |  |  |  |  |  |  |
| Input output User interface access - Input device (keyboard, mouse …) - Output device (display ) - Output notification device(smart icon, sound, light, vibrator …) |  | Yes10 Yes10 Yes |  |  |  |  |  |  |  |
| 10 – Access request no user permission. |  |  |  |  |  |  |  |  |  |

The lists in the groups in Table 6 "Security domains and actions" are not exhaustive, and other actions which are of the same category shall be included in the group for the purposes of requesting user permission.

This subclause identifies the permissions for MExE executables in the 3 security domains (operator, MS manufacturer and Third Party). The permissions do not apply to untrusted MExE executables which are not permitted to execute within the domains.