| 3GPP TSG-T WG2 SWG-EMS#3 Paris, France 16th– 18th January 2002 |  |  |  |  |  |  |  |  | T2E020016 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.040 |  |  |  | CR | CRNum |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 5.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Polyphonic Sound Format for EMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Alcatel, Nec |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | TEI5 |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  |  |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The only supported sound format in EMS is a monophonic sound format. This CR proposes to add a polyphonic melody format as an extended object in EMS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | A new format type is included in the list of supported extended objects. The format of the polyphonic melody is specified in annex E. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | EMS will not support the proposed polyphonic sound format. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

3GPP TS 23.040 V5.1.0 (2001-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Terminals;

Technical realization of the Short Message Service (SMS);

(Release 5)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, GSM, SMS

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2001, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA,TTC).

All rights reserved.

...

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 01.04: "Digital cellular telecommunication system (Phase 2+); Abbreviations and acronyms".

[2]	3GPP TS 02.03: "Digital cellular telecommunication system (Phase 2+); Teleservices supported by a GSM Public Land Mobile Network (PLMN)".

[3]	3GPP TS 22.004: "General on supplementary services".

[4]	3GPP TS 22.041: "Operator determined barring".

[5]	3GPP TS 43.002: "Digital cellular telecommunication system (Phase 2+); Network architecture".

[6]	3GPP TS 23.008: "Organization of subscriber data".

[7]	3GPP TS 23.011: "Technical realization of supplementary services - General Aspects".

[8]	3GPP TS 23.015: "Technical realization of Operator Determined Barring (ODB)".

[9]	3GPP TS 23.038: "Alphabets and language-specific information".

[10]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[11]	3GPP TS 43.047: "Digital cellular telecommunication system; Example protocol stacks for interconnecting Service Centre(s) (SC) and Mobile-services Switching Centre(s) (MSC)".

[12]	3GPP TS 44.008: "Digital cellular telecommunication system (Phase 2+); Mobile radio interface layer 3 specification".

[13]	3GPP TS 24.011: "Short Message Service (SMS) support on mobile radio interface".

[14]	3GPP TS 27.005: "Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[15]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[16]	3GPP TS 51.011: "Digital cellular telecommunication system (Phase 2+); Specification of the Subscriber Identity Module - Mobile Equipment (SIM- ME) interface".

[17]	CCITT Recommendation E.164 (Blue Book): "Numbering plan for the ISDN era".

[18]	CCITT Recommendation E.163 (Blue Book): "Numbering plan for the international telephone service".

[19]	CCITT Recommendation Q.771: "Specifications of Signalling System No.7; Functional description of transaction capabilities".

[20]	CCITT Recommendation T.100 (Blue Book): "International information exchange for interactive videotex".

[21]	CCITT Recommendation T.101 (Blue Book): "International interworking for videotex services".

[22]	CCITT Recommendation X.121 (Blue Book): "International numbering plan for public data networks".

[23]	CCITT Recommendation X.400 (Blue Book): "Message handling system and service overview".

[24]	ISO/IEC10646: "Universal Multiple-Octet Coded Character Set (USC); UCS2, 16 bit coding".

[25]	3GPP TS 22.022: "Personalization of GSM ME Mobile functionality specification - Stage 1".

[26]	3GPP TS 23.042: "Compression Algorithm for Text Messaging Services".

[27]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[28]	3GPP TS 43.048: "Digital cellular telecommunications system (Phase 2+); Security Mechanisms for the SIM application toolkit; Stage 2".

[29]	3GPP TR 21.905: "3G Vocabulary".

[30]	3GPP TS 31.102: "Characteristics of the USIM application".

[31]	3GPP TS 31.101: "UICC – Terminal interface; Physical and logical characteristics".

[32]	3GPP TS 22.105: "Services and Service Capabilites".

[33]	Infrared Data Association. Specifications for Ir Mobile Communications (IrMC).
iMelody

[34]	IETF RFC 822: "Standard for the format of ARPA Internet text messages".

[35]	IETF RFC 1951: " Deflate Compressed Data Format Specification"

[36]	vCard - The Electronic Business Card", version 2.1,The Internet Mail
Consortium (IMC), September 18, 1996,
URL:http://www.imc.org/pdi/vcard-21.doc

[37] 	vCalendar - the Electronic Calendaring and Scheduling Format", version 1.0,
The Internet Mail Consortium (IMC), September 18, 1996,
URL:http://www.imc.org/pdi/vcal-10.doc

[38]				The Complete MIDI1.0 Detailed Specification
					The MIDI Manufacturers Association Los Angeles, CA

[39]				General MIDI Level 1 Sound Set
					http://www.midi.org/about-midi/gm/gm1sound.htm

[40]	ISO/IEC10646: "Universal Multiple-Octet Coded Character Set (UCS)”; UCS2, 16 bit coding.

...

9.2.3.24.10.1.11	Extended Object

The Extended Object allows an extended code range for format types. The Extended Object may extend across segment boundaries of a concatenated short message. Octets 1 through 7 of the first Extended Object IE shall be contained in a single segment. A single segment may include one or more Extended Object IEs.

If multiple SMs are concatenated and at least one of them contains an Extended Object information element, then concatenation of the SMs shall be done using the 'Concatenated short messages, 16-bit reference number', verses the 'Concatenated short messages, 8-bit reference number' information element. The re-assembly of the Extended Object segments shall be done according to the sequence number of the associated Concatenation IE.

One or more Extended Objects may be compressed using a compression algorithm as indicated in the Compression Control IE (see section 9.2.3.24.10.1.13).

An SME implementing the Extended Object IE shall be capable of interpreting an uncompressed concatenated message composed of at least min_eo_msg short messages which have been received. According to current content provider requirements and handset manufacturer constraints, variable min_eo_msg is set to 8.

The first Extended Object IE of an Extended Object contains a reference number, length, control data, type and position. The subsequent Extended Object IEs shall only contain Extended Object data as illustrated in Figure 9.2.24.10.11.

The IE length is variable.

Octet 1	Extended Object reference number
a modulo 256 counter indicating the reference number for the Extended Object. Two different Extended Objects in a single concatenated message shall have different reference numbers.

Octet 2..3	Extended Object length in number of octets (integer representation) as shown in Figure 9.2.3.24.10.1.11.

Octet 4 	Control data
 
Bit 0		Object distribution

- Object may be forwarded

- Object shall not be forwarded by SMS

Bit 1		User Prompt Indicator

0	Object shall be handled normally
1	Object shall be handled as a User Prompt (see 9.2.3.24.10.1.10)

Bit 2..7	reserved

Any reserved values shall be set to 0.

Octet 5	Extended Object Type
This octet indicates the format of the Extended Object from the table below.
If the value is reserved or if the associated format is not supported then the receiving entity shall ignore the Extend Object.

| Format Type | Format Description |
| --- | --- |
| 0x00 | Predefined sound as defined in annex E. |
| 0x01 | iMelody as defined in annex E. |
| 0x02 | Black and white bitmap as defined in annex E. |
| 0x03 | 2-bit greyscale bitmap as defined in annex E. |
| 0x04 | 6-bit colour bitmap as defined in annex E. |
| 0x05 | Predefined animation as defined in annex E. |
| 0x06 | Black and white bitmap animation as defined in annex E. |
| 0x07 | 2-bit greyscale bitmap animation as defined in annex E. |
| 0x08 | 6-bit colour bitmap animation as defined in annex E. |
| 0x09 | vCard as defined in annex E. |
| 0x0A | vCalendar as defined in annex E. |
| 0x0B | Polyphonic Melody Format as defined in annex E. |
| 0x0C..0xFF | Reserved |

Octet 6..7	Extended Object Position (integer representation)
The Extended Object Position indicates the absolute character position within the message text after which the object shall be played or displayed. The absolute character position relates to the entire text within the concatenated message, the first character is numbered character 1.

If more than one Extended Object is located at the same position then they may be played or displayed in sequence or simultaneously.

Octet 8..n	Extended Object Data
This sequence of octets is structured as illustrated in the figure below and defined annex E.   This figure illustrates the construction of a number of SMs containing a large Extended Object which crosses a SM boundary and is encoded into 2 SM TPDUs.  The figure illustrates only the User Data field of the SM (TPDUs). For a description of concatenation of SM refer to Figures 9.2.3.24 (a, b and c)

Figure 9.2.3.24.10.1.11

...

Annex E (normative):
Extended Object Format Type

# E.1	Predefined Sound

The predefined sound as integrated in the Extended Object IE is structured as follows:

Octet 8	Sound number as defined in table of section 9.2.3.24.10.3.1.

# E.2	iMelody

An iMelody object [33] can be integrated in an Extended Object IE with the following structure:

Octet 8..n	iMelody object coded according to the iMelody format [33].

# E.3		Black and white bitmap

The user-defined black and white bitmap as integrated in the Extended Object IE is structured as follows:

Octet 8	Horizontal dimension of picture
This octet shall contain the horizontal number of pixels

Octet 9	Vertical dimension of picture
This octet shall contain the vertical number of pixels.

Octet 10..n	Picture data, pixel by pixel from top left to bottom right. The picture data is encoded as a continuous sequence of bits. There shall be no fill bits at the end of each row of data, Fill bits may only be used in the last octet of the picture data if needed. The fill bits in the last octet shall be ignored. Within each octet the MSB represents the leftmost pixel. .

The colour values are encoded as follows

Bit Value		Colour
0			White
1			Black

# E.4	2-bit greyscale bitmap

The user-defined 2-bit greyscale bitmap as integrated in the Extended Object IE is structured as follows:

Octet 8	Horizontal dimension of picture
This octet shall contain the horizontal number of pixels

Octet 9	Vertical dimension of picture
This octet shall contain the vertical number of pixels.

Octet 10..n	Picture data, pixel by pixel from top left to bottom right. The picture data is encoded as a continuous sequence of bits. There shall be no fill bits at the end of each row of data, Fill bits may only be used in the last octet of the picture data. The fill bits in the last octet shall be ignored.The pair of bits at the MSB represents the leftmost pixel of the four defined in an octet.

The colour values are encoded as follows

Bit Value			Colour
00				Black
01				Dark Grey
10				Light Grey
11				White

# E.5	6-bit colour bitmap

The user-defined 6-bit colour bitmap as integrated in the Extended Object IE is structured as follows:

Octet 8	Horizontal dimension of picture
This octet shall contain the horizontal number of pixels

Octet 9	Vertical dimension of picture
This octet shall contain the vertical number of pixels.

Octet 10..n

Picture data, pixel by pixel from top left to bottom right. The picture data is encoded as a continuous sequence of bits. There shall be no fill bits at the end of each row of data, Fill bits may only be used in the last octet of the picture data. The fill bits in the last octet shall be ignored.

Each pixel colour is represented by 6-bits of data, giving a total of 64 colours. (2 bits of data define the levels of each red, green and blue). The overall pixel colour is a composite of the three RGB values.

The first pair of bits of picture data define the level of red of the topmost, leftmost pixel, the next pair of bits the level of green for this pixel, and the third pair the level of blue for the pixel. The first bit of a pair defining a colour level is the MSB. This is illustrated below.

| Octet 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| MSB Red Pixel 1 | LSB Red Pixel 1 | MSB Green Pixel 1 | LSB Green Pixel 1 | MSB Blue Pixel 1 | LSB Blue Pixel 1 | MSB Red Pixel 2 | LSB Red Pixel 2 |

| Octet 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| MSB Green Pixel 2 | LSB Green Pixel 2 | MSB Blue Pixel 2 | LSB Blue Pixel 2 | MSB Red Pixel 3 | LSB Red Pixel 3 | MSB Green Pixel 3 | LSB Green Pixel 3 |

# E.6	Predefined animation

The predefined animation as integrated in the Extended Object IE is structured as follows:

Octet 8	Animation number as defined in table of section 9.2.3.24.10.3.3.

# E.7	Black and white bitmap animation

The user-black and white animation is integrated in the Extended Object IE is structured as follows:

Octet 8	Horizontal dimension of picture
This octet shall contain the horizontal number of  pixels

Octet 9	Vertical dimension of picture.
This octet shall contain the vertical number of pixels.

Octet 10	The number of frames in the animation

Octet 11	Animation control byte

| Bits | Meaning |
| --- | --- |
| 7 – 4 | Frame display. The value (in tenths of a second) that is requested between each frame: 0000 1 tenth (i.e. 0.1s) 1111 16 tenths (i.e. 1.6 s) |
| 3 – 0 | Repeat value. The requested number of repetitions of the animation: 0000 Unlimited repetition 0001 1 repetition 1111 15 repetitions |

Octet 12..n	Contains a series of bitstreams encoding 1 bit pixel depth bitmaps as defined in F.3.  If a frame in the animation would require fill bits (as described in F.3) these shall be contained at the end of the frame such that the bit-stream for the next frame begins on an octet boundary.

# E.8	2-bit greyscale bitmap animation

The user-black and white animation is integrated in the Extended Object IE is structured as follows:

Octet 8	Horizontal dimension of picture
This octet shall contain the horizontal number of  pixels .

Octet 9	Vertical dimension of picture.
This octet shall contain the vertical number of pixels.

Octet 10	The number of frames in the animation

Octet 11	Animation control byte

| Bits | Meaning |
| --- | --- |
| 7 – 4 | Frame display. The value (in tenths of a second) that is requested between each frame: 0000 1 tenth (i.e. 0.1s) 1111 16 tenths (i.e. 1.6 s) |
| 3 – 0 | Repeat value. The requested number of repetitions of the animation: 0000 Unlimited repetition 0001 1 repetition 1111 15 repetitions |

Octet 12..n	Contains a series of bitstreams encoding 2 bit pixel depth bitmaps as defined in F.4.  If a frame in the animation would require fill bits (as described in F.4) these shall be contained at the end of the frame such that the bit-stream for the next frame begins on an octet boundary.

# E.9	6-bit colour bitmap animation

The user-black and white animation is integrated in the Extended Object IE is structured as follows:

Octet 8	Horizontal dimension of picture
This octet shall contain the horizontal number of  pixels

Octet 9	Vertical dimension of picture.
This octet shall contain the vertical number of pixels.

Octet 10	The number of frames in the animation

Octet 11	Animation control byte

| Bits | Meaning |
| --- | --- |
| 7 – 4 | Frame display. The value (in tenths of a second) that is requested between each frame: 0000 1 tenth (i.e. 0.1s) 1111 16 tenths (i.e. 1.6 s) |
| 3 – 0 | Repeat value. The requested number of repetitions of the animation: 0000 Unlimited repetition 0001 1 repetition 1111 15 repetitions |

Octet 12..n	Contains a series of bitstreams encoding 6 bit pixel depth bitmaps as defined in F.5.  If a frame in the animation would require fill bits (as described in F.5) these shall be contained at the end of the frame such that the bit-stream for the next frame begins on an octet boundary.

# E.10	vCard

A vCard object [36] can be integrated in a Extended Object IE with the following structure:

Octet 8..n	vCard object as defined in [36]. The UTF-8 encoding is used instead of the default 7-bit ASCII. For certain vCard properties, other encoding can be used by setting the CHARSET property parameter to the appropriate character set.

# E.11	vCalendar

A vCalendar object [37] can be integrated in a Extended Object IE with the following structure:

Octet 8..n	vCalendar object as defined in [37]. The UTF-8 encoding is used instead of the default 7-bit ASCII. For certain vCalendar properties, other encoding can be used by setting the CHARSET property parameter to the appropriate character set.

...

# E.13	Polyphonic sound

A polyphonic sound is composed of a header (see section E.13.1) and a data part which is specific to a profile (see section E.13.2). The format is 16-bit aligned.

### E.13.1 Polyphonic melody header

#### E.13.1.1 Header description

The header is structured as follows:

Octet 8		Absolute Time Base (integer representation , msb). The unit for the Absolute Time Base is 5 milli-seconds.

Octet 9	Bit 6,7 	Absolute Time Base (integer representation, lsb)
Bit 5		additional header  flag
			1					the header is followed by an additional header
			0					no additional header (is the default value)
Bit 0..4	Profile
			the profiles are defined in E.13.3
			00000			Basic profile
			00001				Enhanced Basic profile
			00010..11111	reserved for future use.

#### E.13.1.2 Additional header description

The presence of an additional header is indicated by the additional header flag in the file header. All the content types which may be contained by this additional header are referred to as the header option set. The header option set is profile dependent.

The additional header is a sequence of options chosen by the content provider among the header option set. The header options are coded over 16-bit words. Each additional header shall be ended with the “Additional header end” option.

The option identification is based on the header option Id coded over 4 bits in  less significant byte of each option.

The length of the option text is coded over bit 6 to bit 15 in the first 16-bit word of each option..

The general structure of a header option is as following:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| option text length |  |  |  |  |  |  |  |  |  |  |  | Option Id |  |  |  |  |  |
| option text |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The contents are defined for each profile in their respective section of header option description.

### E.13.2 Polyphonic melody data

The polyphonic melody data is a sequence composed of 2-octet and 4-octet commands and of Exclusive messages of variable length.

The command type is identified by Bits 3..0 (command Id field) of the first octet of each command. The SME may skip the command if the command type is reserved or if a reserved value has been used in the command parameters.

The size (2 or 4 byes) of the command is indicated by bit 4.

The general structure of a command is as following:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First command opcode |  |  |  |  |  |  |  |  |  |  | Size | Command Id |  |  |  |  |  |
| Second command opcode (available if size = 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The opcode of each command is defined for each profile in their respective section of command description.

The structure of the Exclusive messages is as following:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| length |  |  |  |  |  |  |  |  |  |  | 1 | 0xE |  |  |  |  |  |
| System Exclusive message |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The MIDI System Exclusive messages defined in [38] will be translated to the Exclusive message as following:

- MIDI System Exclusive message: 0xF0 DATA 0xF7

- Corresponding System Exclusive message fields: DATA

- Corresponding length field: DATA size in 16-bit words

- The DATA fields shall be padded by 0x00 in order to fix 16-bit word alignment

### E.13.3 Profile set

A profile is characterized by :
- its instruction set
- its header option set

The basic profile supports the minimal set of instructions as well as the minimum set of options within the header. Any upper profiles will have to support both the instruction set and the header option set of all the lower profiles.

The tables below give the instruction set and the header option set for all the vailable profiles:

| Instructions | Basic profile | Enhanced Basic Profile |
| --- | --- | --- |
| Note | X | X |
| Program change | X | X |
| Volume | X | X |
| RTB definition | X | X |
| Delay | X | X |
| Relative delay | X | X |
| Build pattern | -- | X |
| Execute pattern | - | X |
| Pitch bend | - | X |
| Modulation wheel | - | X |

Instruction set per profile – X: supported / -: not supported

| Header options | Basic profile | Enhanced BasicProfile |
| --- | --- | --- |
| Content description | X | X |

Header option set per profile - X: supported / -: not supported

#### The terminal equipment compliant with a given profile, is not expected to support any additonal commands (exclusive commands wich use reserved sub-identifier value or commands which use reserved identifier value in that profile) but shall skip them if not supported.

### E.13.4 Polyphonic melody: basic profile

#### E.13.4.1 Basic profile description

Both the set of instruction and the header option set of the basic profile shall be a subset of any upper  profile. The basic profile understands the minimal set of instruction as well as the minimal header option set as indicated in section E.13.3.

The content creators shall not expect that the terminal equipement supporting the Basic Profile can play more than 8 or 6 [TBD] notes of polyphony. If more notes are presented, the terminal equipment is free to implement any note stealing strategy.

#### E.13.4.2 Header option description

The header option set of the basic profile is composed by the following:

| Header option Id | Header options |
| --- | --- |
| 0x00 | Content description |
| 0x01..0xFF | reserved |

##### E.14.4.2.1 Content description option

The Content description option is used to give information on the polyphonic sound. The Content description option length  shall be a multiple of 16-bit word length.

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  |  | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option length |  |  |  |  |  |  |  |  |  |  |  |  | Option Id (0x00) |  |  |  |  |  |
| Content length #1 |  |  |  |  |  |  |  |  |  | Lt #1 |  | Content Id #1 |  |  |  |  |  |  |
| Content text #1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content text #1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content length #2 |  |  |  |  |  |  |  |  |  | Lt #2 |  | Content Id #2 |  |  |  |  |  |  |
| Content text #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content t #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Where:

Option Id				Content description option identified by the value 0x00.

Option length		specifies the length in 16-bit words of the option text.

Content Id #i	specifies the type of content #i. The number of content identifiers contained in the Content description option is given by the content number field

| Content Id | meaning |
| --- | --- |
| 0x00 | Vendor name |
| 0x01 | Carrier name |
| 0x02 | Category name |
| 0x03 | Song title |
| 0x04 | Artist name |
| 0x05 | Lyricist |
| 0x06 | Composer name |
| 0x07 | Arranger |
| 0x08 | Copyright |
| 0x09 | Creation date |
| 0x0A | Revision date |
| 0x0B | comments |
| 0x0C..0xFF | reserved |

Lt #i		specifies the Language type used for the text of Content type #i.
		0: for GSM 7 bit [???] character set
		1: for USC-2 character set [40]

Content length #i	specifies the length in 16-bit words of the texte of content type #i.

Content text #i		Text of the content type #i. The character code system is indicated by Lt #i.  Content text #I shall be padded by 0x00 in order to fix 16-bit word alignment.

#### E.13.4.3 Command description

The structure of each command type is defined in the following sections.

| Bits 3 2 1 0 (instruction Id) | instructions |
| --- | --- |
| 0000 | Note |
| 0001 | Program change |
| 0010 | Volume |
| 0011 | Delay |
| 0100 | Relative delay |
| 0101 | RTB definition |
| 0110 … 1111 | reserved |

The Real Time Base (RTB) defines the time base for  playing notes and applying relative delay commands. The RTB can be redefined anytime in the melody data with the execution of another RTB definition command.

Delay commands use the Absolute Time Base (ATB) time base.

Relative delay commands use the (RTB x ATB) time base.

Note commands use the (RTB x ATB) time base.

The play of the melody should stop when all commands have been executed and once the last delay period is over.

##### E.13.4.3.1 Note command

The extended note command (Bit 4 set to 1) is used to play a note on a particular channel. The extended note command is a 4-octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Note number |  |  |  |  |  |  | Channel |  |  |  | Size(1) | Command (0000) |  |  |  |  |
| Running time |  |  |  |  |  |  |  |  |  | Note velocity |  |  |  |  |  |  |

Where:

Command			Note Command identified by the binary value 0000.

Size					Size is 1 meaning a 4-octet command.

Channel	Channel on which the note is to be played (16 logical channels available).

Note number	Note to be played. Definition is the same as the one in the MIDI format [38] (Middle C of an 88 note piano-style keyboard has a reference value of 60).

Note velocity	Volume of the note to be played. Definition is the same as in the MIDI format [38], except that the range is reduced to 0-63. 0 is a reserved value. Value 1 represents the minimum velocity value whereas value 63 represents the maximum velocity value.

Running time	Duration of the sustain period of the note. This running time applies only to instruments supporting a sustain state. The length of time is calculated by multiplying the Running time by the RTB by the ATB.

If several Note commands are contiguous in the melody, without any insertion of a Delay command (or Extended Delay or Relative delay commands), then the contiguous notes shall be played simultaneoulsy in polyphony.

The short note command (Bit 4 set to 0) is used to play a note on a particular channel. The short note command is a 2-octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Note number |  |  |  |  |  |  | Channel |  |  |  | size(0) | Command (0000) |  |  |  |  |

The octet fields (note number and channel) must be interpreted as for the extended note. The running time and the note velocity of the short note are set to the ones of the last extended note that occured in the melody on the same channel. If no extended note occured before the occurence of a short note then the running time is set to 1 and the velocity is set to 63.

##### E.13.4.3.2Program Change command

As in MIDI, this command is used to assign a particular instrument to a specific channel. The instrument is defined by an instrument number and an ambience.

This command shall be immediately executed. It has no user perceptible running time. This command should not affect notes already being played on the specified channel.

The instrument associated to a channel can be re-assigned anytime by executing another Program Change command. The default instrument associated to a channel is a piano (ambiance classic). The Program Change command is a two octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instrument number |  |  |  | Ambience |  |  | Channel |  |  |  | size(0) | Command (0001) |  |  |  |  |

Where:

Command			Program Change Command identified by the binary value 0001

Size					Size is 0 meaning a 2-octet command.

Channel				Channel number, from 0 to 15. 
						Channel 9 is dedicated to the set of drum instruments (drumkit).
						For the drumkit, the effective instrument number is made according to the note number as in [39].

Ambience			Sound variation, from 0 to 7 as defined in the table below.

| Ambience | Type |
| --- | --- |
| 0 | Classic (default) |
| 1 | Rock |
| 2 | Pop |
| 3 | Techno |
| 4 | Disco |
| 5 | Jazz |
| 6 | Folk |
| 7 | Reserved |

Instrument number		instrument number to be associated to this channel as defined in the following table.

| Equivalent GM [39] instrument number | Instrument number | Instrument description |
| --- | --- | --- |
| 1..8 | 0 | Piano |
| 9..16 | 1 | Chromatic percussion |
| 17..24 | 2 | Organ |
| 25..32 | 3 | Guitar |
| 33..40 | 4 | Bass |
| 41..48 | 5 | Strings (violin, harp,etc) |
| 49..56 | 6 | Ensemble (orchestral, chorus) |
| 57..64 | 7 | Brass |
| 65..72 | 8 | Reed (Saxophone, clarinet, etc) |
| 73..80 | 9 | Pipe (flute, etc) |
| 81..88 | 10 | Lead Synthesiser (solo type) |
| 89..96 | 11 | Synthesiser pad (orchestral) |
| 97..104 | 12 | Synthesiser effect (special effects) |
| 105..112 | 13 | Ethnic (sitar, banjo, etc) |
| 113..120 | 14 | Percussive |
| 121..128 | 15 | Sounds effects (telephone, explosion,etc) |

This field shall be ignored when reassigning the drumkit (channel 9).

The program change command (0001) with bit 4 set to 1 is a reserved command and shall be skipped.

##### E.13.4.3.3 Volume command

This command is used to set a volume level on a specific channel. 128 different volume levels can be associated to a channel.

The initial volume value is 127 and can be changed with the Volume command. The application of the  volume command is performed according to [38].

This command shall be immediately executed. This command should affect future note as well as notes already being played. It has no user perceptible running time.

The Volume command is a 2-octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Volume |  |  |  |  |  |  | Channel |  |  |  | size(0) | Command (0010) |  |  |  |  |

Where:

Command			Volume Command identified by the binary value 0010.

Size					Size is 0 meaning a 2-octet command.

Channel				Channel number, from 0 to 15.

Volume	specifies the volume value to be applied to this channel. Binary value 0000000 is for muting the designated channel and binary value 1111111 is for the maximum volume. Intermediaries volume values are linearly interpolated from the silence and maximal volume value supported by the ME.

The volume command (0010) with bit 4 set to 1 is a reserved command and shall be skipped.

##### E.13.4.3.4 RTB definition command

This command defines or redefines the Relative Time Base (RTB). Upon execution of the RTB command, the new RTB value is used by all concerned subsequent commands (Note and Relative delay commands) until a new RTB command execution. This command should not affect notes already being played.

The initial value of the RTB is 1. The RTB command is a two octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RTB Value |  |  |  |  |  |  |  |  |  |  | size(0) | Command (0101) |  |  |  |  |

Where:

Command		RTB Command identified by the binary value 0101.

Size				Size is 0 meaning a 2-octet command.

RTB Value		Value for the RTB from the range 1…2047 (integer representation). Value 0 is reserved.

The RTB definition command (0101) with bit 4 set to 1 is a reserved command and shall be skipped.

##### E.13.4.3.4.5 Delay command

This command is used to delay the execution of the next command(s). Note that the build pattern command is not addressed by the delay command.

When the short delay command is executed, some other commands may already be under execution. The execution of the short delay command does not stop the execution of these commands. The short delay command (Bit 4 set to 0) is a two octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wait Delay |  |  |  |  |  |  |  |  |  |  | size(0) | Command (0011) |  |  |  |  |

Where:

Command			Delay Command identified by the binary value 0011

Size					Size is 0 meaning a 2-octet command.

Wait Delay	Time period to wait before executing next command (integer representation). The delay to wait is calculated by multiplying the Wait Delay by the ATB.

As for the short delay command, the extended delay command (Bit 4 set to 1) is used to delay the execution of  the next command(s).

The Extended Delay command is a 4-octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wait Delay (LSB) |  |  |  |  |  |  |  |  |  |  | Size(1) | Command (0011) |  |  |  |  |
| Wait Delay (MSB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Where:

Command			Extended Delay command identified by the binary value 0011

Size					Size is 1 meaning a 4-octet command.

Wait Delay	Time period to wait before executing next commands (integer representation). The delay to wait is calculated by multiplying the Wait Delay by the ATB.

##### E.13.4.3.6 Relative delay command

This command is used to delay the execution of the next command(s). Note that the build pattern command is not addressed by the relative delay command.

It works like the Delay command except that it is based on ATB x RTB (the Delay and Extended Delay are based on ATB only).

This command is used for special sound events (like silences)  that need to be played on exactly the same relative time base (RTB). This command may also be used for being synchronized with a specific tempo set by a RTB definition command. The relative delay command is a two octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wait Delay |  |  |  |  |  |  |  |  |  |  | size(0) | Command (0100) |  |  |  |  |

Where:

Command		Relative delay Command identified by the binary value 0100.

Size				Size is 0 meaning a 2-octet command.

Wait Delay	Time period to wait (integer representation). The delay to wait is calculated by multiplying the Wait Delay by the ATB by the RTB.

The command (0100) with bit 4 set to 1 is a reserved command and shall be skipped.

##### E.13.4.3.7 Extended command

This command is reserved to enable the addition of future commands when all the Command types from Command Id 0000 to 1110 are defined.

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UNDEFINED |  |  |  |  |  |  |  |  |  |  | size | Command (1111) |  |  |  |  |

Where:

Command		Extended Command identified by the binary value 1111.

Size				Size 0 is  meaning a 2-octet command.
					Size 1 is  meaning a 4-octet command

### E.13.5 Polyphonic melody: Enhanced Basic profile

#### E.13.5.1 Enhanced Basic profile description

The Enhanced Basic profile is fully backward compatible with the Basic profile and introduces commands to ensure pitch bend and vibrato length change. In addition, pattern notion is introduced which enable to obtain benefits in term of compression.

The content creators shall not expect that the terminal equipement supporting the Enhanced Basic Profile can play more than 8 or 6 [TBD] notes of polyphony. If more notes are presented, the terminal equipment is free to implement any note stealing strategy.

#### E.13.5.2 Header option description

The header option set is the same one as the Basic profile.

#### E.13.5.3 Command description

The structure of each command type is defined in the following sections.

| Bits 3 2 1 0 (instruction Id) | instructions |
| --- | --- |
| 0110 | Build pattern |
| 0111 | Execute pattern |
| 1000 | Modulation wheel |
| 1001 | Pitch bend |
| 1010 … 1111 | reserved |

##### E.13.5.3.1 Build pattern command

This command is used to define a recurrent command sequence identified in the ringtune. This sequence can be executed at any anytime by the use of  the execute pattern command.

A command sequence must be assigned a pattern identifier.

A command sequence definition shall begin by a build pattern command with bit #15 set to 0 and shall end with the same build pattern command except for the bit #15 which has to be set to 1.

Not that both Build pattern command and Execute pattern command are not allowed to be used inside another command sequence definition.

| 15 | 14 | 13 | 12 | 11 | 10 |  | 9 |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00000 |  |  |  |  |  | M |  | Pattern Id |  |  |  |  | size(0) | Command (0110) |  |  |  |  |

Where:

Command		Build pattern Command identified by the binary value 0110.

Size				Size is 0 meaning a 2-octet command.

Pattern Id	Pattern identifier which is assigned to an only one command sequence. This Identifier is used by the Execute pattern command.

M	Start flag which enables to indicates whether the Build pattern command addresses either the beginning or the end of a command sequence definition
			0			beginning of a command sequence definition
			1			end of a command sequence definition

##### E.13.5.3.2 Execute pattern command

This command is used to execute a recurrent command sequence identified in the ringtune. This sequence has to be previously defined and assigned a pattern identifier  by the use of the build pattern command. The Execute pattern command is a two octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000000 |  |  |  |  |  |  | Pattern Id |  |  |  |  | size(0) | Command (0111) |  |  |  |  |

Where:

Command		Execute pattern Command identified by the binary value 0111.

Size				Size is 0 meaning a 2-octet command.

Pattern Id		Pattern identifier which is assigned to an only one command sequence

Note that the execution of a pattern must be handled as if the Execute pattern command was replaced byll all the commands defined by the associated Build pattern command.

##### E.13.5.3.3 Modulation wheel command

As the Control Change Modulation in MIDI, this command is used to modify the vibrato depth of a channel. The Modulation command is a two octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vibrato depth |  |  |  |  |  |  |  | channel |  |  |  | size(0) | Command (1000) |  |  |  |  |

Where:

Command			Modulation Command identified by the binary value 1000

Size					Size is 0 meaning a 2-octet command.

Channel				Channel on which the vibrato length must be changed (16 logical channels available).

Vibrato depth	vibrato depth value.
	The default value is 0x00 (no effect).
	The range is 0x00..0x7F

[it is left for further studies that the Modulation wheel command could change the nature of the sound in the most natural (expected) way. I.e. depth of LFO; change of timbre; add more tine sound; etc..]

##### E.13.5.3.4 Pitch bend command

As in MIDI, this command is used to modify the pitch of a channel. The Pitch command is a two octet field structured as follows:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pitch |  |  |  |  |  |  |  | channel |  |  |  | size(0) | Command (1001) |  |  |  |  |

Where:

Command			Modulation Command identified by the binary value 1001

Size					Size is 0 meaning a 2-octet command.

Channel				Channel on which the pitch must be changed (16 logical channels available).

Pitch					Pitch value.
							The default value is 0x08.
							The range is 0x01..0xE

The extended Pitch bend command is structured as following:

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000000 |  |  |  |  |  |  |  | channel |  |  |  | size(1) | Command (1001) |  |  |  |  |
| Pitch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Where:

Command			Modulation Command identified by the binary value 1001

Size					Size is 1 meaning a 4-octet command.

Channel				Channel on which the pitch must be changed (16 logical channels available).

Pitch					Pitch value.
							The default value is 0x40.
							The range is 0x00..0x7F

### E.13.6 UE Conformance with polyphonic sounds

[TBD]