ETSI SMG4	TSGT2#2(99)289

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.07 |  |  |  |  | 7.2.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | TEI |
| --- | --- |

| Source: | SMG4 | Date: | 990312 |
| --- | --- | --- | --- |

| Subject: | New command for setting of Time format |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Added functionality for setting of time format for the time information presented on the ME’s display. |
| --- | --- |

| Clauses affected: | 8.xx |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

6.19	Settings time format +CSTF

Table 1: +CSTF parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CSTF=[<mode>] | +CME ERROR: <err> |
| +CSTF? | +CSTF:<mode> +CME ERROR: <err> |
| +CSTF=? | +CSTF:(list of supported <mode>s) +CME ERROR: <err> |

Description

Set command sets the time format of the time information presented to the user.

Read commands reads the current setting.

Test commands reads the supported <modes>s.

Defined values

<mode>:

- HH:MM (24 hour clock)

- HH:MM a.m./p.m.

3-7	      Manufacturer specific

Implementation

Optional