| SMG4/TSG-CN3/TSG-T2 London, 15-19 March 1999 | TSGT2#2(99)298 |
| --- | --- |

Source:	Rapportuer Lars Novak, Ericsson Mobile Communications AB

Title:	Report from the MSTE interface group

Document for:	SMG4 closing plenary

___________________________________________________________________________

# Introduction

This is the report from the MSTE interface meeting  16-17 of March held in London. The meeting was a joint meeting between  TSGT2 working group2 subworking group 2, SMG4 and TSG-CN3.

The input documents are found in section 2, the approved documents are found in section 4.

There are some documents that were not reviewed due to different circumstances, these are found in section 3. These should be reviewed at the closing plenary.

The participants of the meeting are found in chapter 6.

# Report

The documents discussed at the meeting were:

## T2-99250 Editorial Corrections to 07.10

Source: Ericsson

This document corrects errors in the remote port negotiation command chapter. An error in the first figure in chapter 5.4.6.3.9 had occurred, one column should be removed since the command can only have either one parameter or 8, and the figure shows 9. The PM bits in remote negotiation command were wrong, they should not be 1 to 18, instead 1 to 16 bits, since the PM is composed of 2 bytes (2*8=16). The last section in chapter 5.4.6.3.9 was removed since it was duplicated.

In section 5.4.6.3.11 Service Negotiation Command a type error was corrected, an “if” is changed to “of”.

Figure 12 in chapter 5.4.8.1 Flow control was corrected, the arrows marked FCS1 should be renamed to FCS3 and FCS3 should be renamed to FCS1. FCS3 reflect internal buffer management and should not come from the other device, instead it should be generated internal in the device.

Status:Agreed

## T2-99053 Syntax Error in +CHSN command

Release 96

Source: Ericsson

Every extended command has a test command (trailing =?) to test the existence of the command and to give information about the type of its subparameters”.

This document was agreed on the previous SMG4 meeting, but administrative errors had occurs, while it has to be reviewed again.

The +CHSN command violates this syntax  (without any obvious reason or benefit). As a result, an application (or user) will have to use a “trial and error” approach when setting this command. This CR introduces the normal syntax. A list of supported values is returned instead of only a single number.

Status:Agreed

## T2-99054 Syntax Error in +CHSN command

Release 97

Source: Ericsson

Same as T2-99053 but for release 97

Status:Agreed

## T2-99055 Syntax Error in +CHSN command

Release 98

Source: Ericsson

Same as T2-99053 but for release 98

Status:Agreed

## T2-99060 Alarm functionality

Source: Ericsson

This document proposes a functional addition to +CALA. The new functionality is to enable reoccurring alarms, that is, alarms that occur for instance Monday to Friday. Also a silent mode is added, that is, only an indication to the TE will be sent when the alarm expires, no indication on the MS. In addition 2 new commands were proposed,  +CALD: delete a set alarm in the MS and +CAPD: postpone an expired alarm, in some cases called “snooze”. Also a new unsoliceted result code were proposed, +CALV, which is issued when the alarm has expired.

Status:Agreed

## T2-99061 Phonebook storage

Source: Ericsson

This document proposes to add an optional parameter to the +CPBS command. This new parameter makes it easier to choose a PIN2-locked phone book memory storage by sending the PIN2 as an optional parameter.

Status:Agreed

## T2-99062 Timezone

Source: Ericsson

This document proposed 2 new AT commands and one new unsolicited result code to handle the Network Initiated Time Zone  (NITS) handling. The +CTZU “Automatic Time Zone Update” command is used to activate and deactivate the NITZ functionality in the phone. The +CTZR “Time Zone Reporting “ is used to enable indication to the TE when an update has occurred. The indication is sent as an unsolicited result code + CTZV.

Status:Agreed

## T2-99063 New command for setting the date format

Source: Ericsson

This document proposes a new command to handle date format in the phone. The command name is  +CSDF “Settings date format”. 8 different formats were defined in addition to a number of manufacturer specific formats. This command was not accepted, the following modifications has to be made:

- make it clear the formats are optional and the list is only a proposed one

- the format is applicable for the date that is presented on the MS display

- Increase the number of manufacturer specific formats to 255

- Specify that the “MMM” alternative is language specific

Status:Not Agreed

## T2-99064 New command for Silent mode

Source: Ericsson

This document proposes a new command used to silent the MS so no external sound except the loudspeaker would appear. The command is called Silence Command +CSIL.

It was not clear which sounds should be “muted”, a clarification about this was necessary. All sounds except the loudspeaker should be muted.

Status:Not Agreed

## T2-99065 New command for setting Time format

Source: Ericsson

This document proposes a new command to handle the time format on the MS display. The command is +CSTF “Settings time format”. With this command it is possible to change between 24h or am/pm presentation of the clock in the MS display.

This document was not agreed, because it was not clear if the time was presented to the used on the MS display or not. This has to be clarified.

Status:Not Agreed

## T2-99251 Additional result code for +CSSN

Source: Motorola

This document proposes to solve the following problem: In the situation where a call is already in progress and a second incoming call is diverted resulting in a network notification, there will be no way to display the message. There is an omission in the CSSU: <code 2> codes of +CSSN. This document proposes to add a new value to the <code 2> parameter identifying this case.

Status:Agreed

## T2-99252 Additional result codes for  +CLIP +CCWA

Source: Motorola

This document proposes a solution to the problem when no CLI information is available or actually withheld by the calling party to the MS and therefor could not be presented. In the present solution, there is no way to indicate this to the TE. Two new codes were proposed. This contribution was in principle agreed, although, the technical solution was not. The <number> parameter is expected to contain a number not a text. It was proposed by the meeting to change the solution to use an optional additional parameter to the commands, instead of adding the text to the <number> parameter. The commands that are concerned are the +CLIP: and the +CCWA.

Status:Not Agreed

## T2-99288 New command for Silent mode

Source: Ericsson

This document is an updated version of the T2-99064 with the requested updates. No further comments were made to the change request.

Status:Agreed

## T2-99289 New command for setting of Time format

Source: Ericsson

This document is an updated version of T2-99065 with the requested modifications. It was reviewed, no further comments were made.

Status:Agreed

## T2-99300 Additional result codes for  +CLIP +CCWA

Source: Motorola

This document is an updated version of the T2-99252. In this document the two new values were added as an extra parameter to the unsolicited result code, although, the parameter was not optional. This could not be accepted, the parameter has to be optional.

Status:Not Agreed

# Documents not reviewed

The documents listed below were not reviewed during the sub group meeting due to time limitations and have to be reviewed during the closing plenary.

## T2-99232 CR to 07.07 (release 97) to incorporate GPRS AT commands

## T2-99233 CR to 07.07 (release 98) to incorporate GPRS AT commands

## T2-99287 New command for setting of Date format

# Agreed Change Requests

| Doc number | Title | Comments |
| --- | --- | --- |
| T2-99053 | Syntax Error to +CHSN command | Release 96 |
| T2-99054 | Syntax Error to +CHSN command | Release 97 |
| T2-99055 | Syntax Error to +CHSN command | Release 98 |
| T2-99060 | Alarm funtionality |  |
| T2-99061 | Phonebook storage |  |
| T2-99250 | Corrections to 07.10 |  |
| T2-99251 | Additional result codes for +CSSN |  |
| T2-99288 | New command for silent mode | Origin: T2-99064 |
| T2-99289 | New command for setting of Time format | Origin: T2-99065 |

# All documents

| Doc number | Title | Comments |
| --- | --- | --- |
| T2-99053 | Syntax Error to +CHSN command | Release 96 |
| T2-99054 | Syntax Error to +CHSN command | Release 97 |
| T2-99055 | Syntax Error to +CHSN command | Release 98 |
| T2-99060 | Alarm funtionality |  |
| T2-99061 | Phonebook storage |  |
| T2-99062 | TimeZone |  |
| T2-99063 | New command for setting the date format |  |
| T2-99064 | New command for Silent mode |  |
| T2-99065 | New command for setting Time format |  |
| T2-99250 | Corrections to 07.10 |  |
| T2-99251 | Additional result codes for +CSSN |  |
| T2-99252 | Additional result codes for +CLIP +CCWA |  |
| T2-99288 | New command for silent mode | Origin: T2-99064 |
| T2-99289 | New command for setting of Time format | Origin: T2-99065 |
| T2-99300 | Additional result codes for +CLIP +CCWA | Origin: T2-99252 |

# Participants List

There were 20 participants from 12 companies in the group, not all were present during the whole session.

| Name | Company | Email |
| --- | --- | --- |
| Lars Novak (chairman) | Ericsson | Lars.Novak@ecs.ericsson.se |
| Yoichiro Minami | NEC | Minami@ccmail.mcc.yh.nec.co.jp |
| Eiji Iimori | Toshiba | Iimori@msra.hino.toshiba.co.jp |
| Susumu Uriya | NEC | Uriya@ccmail.mcd.yh.nec.co.jp |
| Hiroshi Kanno | Fujitsu | Kanno@mcws.ts.fujitsu.co.jp |
| Kouichi Sawai | NTT DoCoMo | Sawai@cet.yrp.nttdocomo.co.jp |
| Hidekazu Saeki | NTT DoCoMo | Saeki-h@cet.yrp-nttdocomo.co.jp |
| Toshihiro Shimizu | Matsushita Comm | Toshi.shimizu@mci.co.uk |
| Kiyohito Nagata | NTT DoCoMo | Nagata@vet.yrp.nttdocomo.co.jp |
| Yasushi Iwane | Mitsubishi | Iwane@cew.melco.co.jp |
| Hiroyuki Morimuto | Mitsubishi | Morimo@csc.melco.co.jp |
| Paul Voskar | Nokia | Paul.voskar@nmp.nokia.com |
| Esa Härmä | Nokia | Esa.harma@nmp.nokia.com |
| Carl Gustavsson | Ericsson | Carl.gustavsson@ecs.ericsson.se |
| Sten Fidjeland | Ericsson | Sten.ketil.fidjeland@ericsson.se |
| Eric Descheres | Philips | Eric.descheres@lme.pcc.philips.com |
| Marko Valo | Nokia | Marko.valo@nmp.nokia.com |
| Gunilla Bratt | Ericsson | Gunilla.Bratt@ecs.ericsson.se |
| Tomiso Kohata | Panasonic | Tommy@gm.mci.mei.co.jp |
| Duncan Fitt | Celnet | Duncan.fitt@cellnet.co.uk |