SMG4/TSG-T2							TSGT2#2(99)206

London, UK

15th - 19th March 1999

Source: Vodafone

Additional 7 bit alphabet tables for other languages in 03.38

A request has been sent to the Rapporteur of 03.38 ( Mr. Harris,  Vodafone ) for the inclusion of the Slovak language characters.

At present, any language whose characters lie outside of the GSM default alphabet set have to use UCS2 for SMS. This results in their maximum length of messages being limited to 70 characters because UCS2 is a 16 bit character set.

This puts such countries at a distinct disadvantage as their only other viable option is to use compression which is an additional level of complexity or to use concatenated SM’s which is likely to have additional tariff implications.

Recently, SMG4 introduced an extension mechanism to 03.38 to allow characters of international significance to be included in an extension table ( e.g. the Euro currency symbol.)

It would be possible to assign a value in the extension table to point to one of 128 further extension tables each of which would apply to a particular language character set. Each of these tables may require one code value to act as an escape mechanism back to the 7 bit default alphabet table. The assignment of that escape character , if necessary, should be left to the appropriate language.

The overhead for accessing a language table would be 3 characters but it is likely that message encoding and decoding processes would remain in a particular language table for that SM.

This results in a maximum SM message length of  157 characters compared to 70 UCS2 characters for those countries to whom this mechanism would apply.

SMG4 are invited to discuss this proposal.