| 3GPP TSG-T WG2#3 Yokohama, Japan, 19-21 April 1999 | TSGT2#3(99)354 |
| --- | --- |

Agenda Item:	7

Source:	Mannesmann Mobilfunk, Panasonic

Title:	MExE Capability Negotiation

Document for:	Decision

___________________________________________________________________________

During the alignment of the capability negotiation properties with the WAP Forum it appeared that some of the properties proposed by MExE were deleted. This contribution provides examples of use for capability negotiation properties which should therefore be retained.

Keyboard Type

The realization of the Virtual Home Environment, VHE, includes the adaptation and personalisation of the User Interface to the preferences of the user, to the capabilities of the actually used terminal, and to the provision of new services (incl. the roaming case). Especially for the Network Operator / Service Provider it is important to create a close link to the customer and to be able to fulfil specific customer wishes. The knowledge about the used terminals and the keyboard layout  is an prerequisite to select and transfer the service/application which is most suited.

Serial Number/ Software Version

For Operation and  Maintenance, Customer Care and Service Management purposes an unequivocal terminal identification and software status recognition is necessary.

During the years of GSM operation it became very clear that the successful implementation of new GSM services and features is quite often hindered by outdated software in the terminals of the actual customer base. Examples for difficulties are eg. in the area of Supplementary Services and SMS Cell Broadcast, but also network features like frequency hopping.

Supported Bearers

The Network Operator / Service Provider shall be able to choose the MExE executable the fits best to the terminal capabilities (in terms of supported bearers) and the available resources in the network. It is therefore useful to be able to negotiate the appropriate terminal bearer capabilities. In addition the user shall be able to express his wishes in terms of preferred bearer services (udi, SMS, USSD, packet, ...) this shall be taken into consideration by the Network Operator / Service Provider.

It is proposed to leave the following properties in the capability negotiation table of the MExE specification (03.57):

- Serial Number

- Software Version

- Keyboard type

- Supported bearers

Home Environment in UMTS