ETSI SMG4	TSGT2#2(99)430

Yokohama, 19-21 April, 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 02.57 |  | Version |  | 7.0.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | x | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | 02.57, MExE |
| --- | --- |

| Source: | SMG10/Vodafone | Date: |  |
| --- | --- | --- | --- |

| Subject: | Security section |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | X |
| only shall be | D Editorial modification | X |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Complete replacement of security section, following review by SMG10 and discussion between SMG10 and SMG4. Changes term “security level” to “security domain” Changes term “applications, applets and content” to “MExE executables” |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

# 11	Security

This clause consists of:

- a sub-clause giving the principles behind security for MExE.  These are not requirements as such but the principles behind the requirements

- a sub-clause specifying specific requirements that MExE implementations must adhere to

- a sub-clause specifying the security domain classifications for MExE executables

## 11.1	Security Principles

- The ME and the data therein are the property of the user.  The user is also responsible for the payment of chargeable events involving her MS, and will be seen as the party responsible for any events (whether chargeable or not) involving her MS.  Therefore the user shall have full control over all chargeable and non-chargeable events initiated by her MS (“event” includes responses made by the MS to external events, e.g. the acceptance by the MS of an incoming call).  This control can be exercised either by the giving of explicit permission at the time of the event or by the giving of implicit permission to the events by the agreement to an event schedule listed clearly in a user profile.

- The user shall be able to request the logging of specific network events initiated by MExE MS applications/applets.

- The privacy of user data in the MS is of paramount importance.

- The SIM and operator controlled areas within the terminal are the property of the network operator.  The network operator shall therefore have full control over access to the SIM and operator controlled area The operator shall also have full control over data, excluding personal user data, transmitted to or from the SIM and the operator controlled terminal area and all events initiated by the SIM or operator controlled area (“event” includes responses made to external events, e.g. the response to a command sent from the ME).

- As the user cannot know the capabilities of any MExE executables transferred from a MExE service environment before transfer, the MS MExE environment shall ensure that transferred MExE executables cannot compromise the above principles.

## 11.2	Security Requirements

- For MExE executables of security operator, manufacturer and user trusted domains , as defined in clause 11.3, it shall be possible to authenticate the identity of the body that authorised the application, applet or content.

- There shall be a secure, unforgable means for assigning the security domains defined in section 11.3 to the MExE executables transferable from the MExE service environment.

- The certification of authorisation associated with MExE executables transferable from the MExE service environment shall be transferred with the certified material.

- The MExE MS shall be able to verify the security domain, as defined in section 11.3, of MExE executables transferred from the MExE service environment.

- The verification process in the MS itself shall not compromise the security of the functionality and content in the MS

- Transferred material that fails verification shall not be installed and shall be deleted by the terminal as soon as possible.

- MExE executables that cannot be verified due to the absence of required verification information in the MS, shall be considered as untrusted  material, as defined in section 11.3.

- The events that MExE executables are given permission by the user to initiate shall be securely recorded in the user profile.

- There shall be mechanisms within the MExE MS for ensuring that applications cannot have access to MS functionality and content beyond that allowed by their security domain, as defined in section 11.3.

- It shall be possible to for the user to downgrade MExE executables of operator, manufacturer or user trusted domain status to untrusted status, at installation or at any other time.

- The MExE MS shall be able to detect if MExE executables transferred from the MExE service environment have been modified since they were assigned a security level.

- MExE executables shall not be transferred to a MExE MS without the explicit permission of the MS user immediately prior to transfer or implicit permission via the user profile.

- Applications and applets transferred to a MExE MS shall not be able to initiate events without the explicit permission of the MS user immediately prior to event initiation or implicit permission via the user profile.

- The user profile data for transfer and event initiation cannot be changed without the explicit agreement of the user.

- The user shall be able to abort or suspend any on-going call that has been set up automatically by an application.

- The integrity of the SIM and existing GSM security mechanisms shall not be compromised by the introduction of MExE services.

- The user shall be able to request the logging of specific network events initiated by MExE MS applications/applets.

- MExE MS applications/applets shall not be able to send command RUN GSM ALGORITHM to the SIM.

## 11.3	Security domain classifications

The security domain of MExE executables shall be graded according to the measure of authorisation which they have been designated. The following 3 (the “sandbox” in which untrusted MExE executables runs is not considered to be a domain) domains shall be supported for MExE executables:

-	MExE Security Operator Domain (used by the HPLMN operator);

MExE executables designated at this security domain have been authorised by the network operator (i.e. HPLMN),

-	MExE Security Manufacturer Domain  (system MExE executables);

MExE executables designated at this security domain have been authorised by the MExE MS manufacturer.

-	MExE Security User Trusted Domain (trusted applications, applets and content);

MExE executablesMExE executables designated at this security domain have been written by user trusted software developers and verified as user trusted domain material (but not with regard to their content) via organisations such as certification authorities.

-	MExE Security Untrusted (untrusted applications, applets and content);

Untrusted MExE executables have not been supplied with an associated authorisation, or the authorisation cannot be verified due to the absence of required verification information in the MS.MExE.

The common execution environment on the MExE MS shall provide a comprehensive set of security mechanisms.

With the transferring and execution of applications, applets and content to the MExE MS (via the GSM/UMTS network or other means), secure firewalls to the basic functionality of the MExE MS shall be provided. Transferred MExE applications, applets and content shall be secure and verified on the MExE MS. The secure area of the application, applet and content will be used to verify both the integrity of the transferred MExE applications, applets and content, as well as identifying and authenticating the Security Level and certification assigned to it. The security of content may be optional, as determined by the sender of the information. Strict rules are required in the MExE MS execution environment in terms of detecting and discarding rogue applets.

The security level of applications, applets and content shall be securely contained within the applications, applets and content and be verified for its integrity in the MExE MS as part of the security procedures.

The security level of applications, applets and content shall be graded according to the measure of authorisation which they have been designated. The following 4 levels of shall be supported for MExE applications, applets and content:

-	MExE Security Level 1 (used by the HPLMN operator);

MExE applications, applets and content designated at this security level have been authorised by the network operator (i.e. HPLMN), with access to GSM security areas and functions.

-	MExE Security Level 2 (system applications, applets and content);

MExE applications, applets and content designated at this security level have access authorisation on the ME or smartcard, to enhance the MExE MS’s capabilites (i.e. upgrade the MExE MS software/firmware). This level of authorisation is normally ascribed to the MExE MS manufacturer.

-	MExE Security Level 3 (trusted applications, applets and content);

MExE applications, applets and content designated at this security level have been authorised to perform functions (such as create files, or read files) on the MExE MS, and have access rights to the MExE MS’s capabilities as defined by this level of user-authorised certification. The applications’, applets’ and content’s access rights associated with the certification may be determined by the user.

-	MExE Security Level 4 (untrusted applications, applets and content);

MExE applications, applets and content designated at this security level have restricted access authorisation on the ME or SIM, and do not have certification permitting access to the MExE MS’s capabilities. The applications’, applets’ and content’s access rights have not been supplied with an associated certification determined by the user, or the certification has not been successfully verified.

Multiple Security Levels may be assigned to applications, applets and content, and there is no implied hierarchy or inclusion of Security Levels. MExE Stage 2 shall further elaborate the capabilities of each Security Level.

The MExE subscriber shall be able to control whether applications, applets and content of specific Security Levels may be transferred to the MExE MS.

Applications and applets shall be able to designate whether their attributed file space is accessible by other applications and applets (i.e. application “A” which maintains a personal financial information in a file shall identify whether it may only be accessed by itself, or by other applications).

Within the constraints of the defined Security Levels, applications and applets (either on or off the MExE MS) shall not be able to originate calls, unless they have been authorised by the user.

With MExE’s capability to transfer applications and applets from a network server to the MExE MS, the risk of compromising the integrity of the MExE MS’s security must be prevented. Security requirements are therefore identified for both the ME and the SIM. Security mechanisms from the MExE MS to the accessed servers shall and have access to a range security functions to authenticate both the MExE MS and server; such authentication may be performed by the MExE MS, an intermediate entity, and/or the server. MExE shall continue to support the inherent security and integrity of the GSM network.

The integrity of the SIM and existing GSM security mechanisms shall not be compromised with the introduction of MExE services.