| 3GPP TSG-T2#4 / ETSI SMG4 Miami, US, 14-16 June 1999 |  |  |  |  |  | TSGT2#4(99)461 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST No : |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |
| Technical Specification GSM / UMTS: |  |  |  | 03.38 |  |  | Version: |  |  | 7.1.0 |  |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | x |  |  | without presentation ("non-strategic") |  |  |  |  |  |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3a.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE |
| --- | --- |

| Source: | Ericsson, Logica Aldiscon, Nokia, Unwired Planet | Date: | 30/04/1999 |
| --- | --- | --- | --- |

| Subject: | Data Coding Scheme for WAP over USSD and CB |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | x |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Current GSM 03.38 does not allow the indication of WAP specific messages, nor User Data Header, over USSD or Cell Broadcast. By reserving one Reserved Coding Group in Data Coding Scheme for network originated WAP specific messages, the terminal would know to use that WAP specific coding has been used for the message. |
| --- | --- |

| Clauses affected: | Sections 2 and 5 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

# 1	Scope

This TS defines the language-specific requirements for GSM. These are specific codepoints required by the Short Message Service (SMS) specifications which in turn are used not only for SMS (GSM 03.40, 03.41) but also for Unstructured Data (GSM 02.90) and may additionally be used for Man Machine Interface (MMI) (GSM 02.30).

The specification for the Data Circuit terminating Equipment/Data Terminal Equipment (DCE/DTE) interface (GSM 07.05 [8]) will also use the codes specified herein for the transfer of SMS data to an external terminal.

# 2	Normative references

References may be made to:

a)	specific versions of publications (identified by date of publication, edition number, version number, etc.), in which case, subsequent revisions to the referenced document do not apply; or

b)	all versions up to and including the identified version (identified by "up to and including" before the version identity); or

c)	all versions subsequent to and including the identified version (identified by "onwards" following the version identity); or

d)	publications without mention of a specific version, in which case the latest version applies.

A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

[1]	GSM 01.04: "Digital cellular telecommunication system (Phase 2+); Abbreviations and acronyms".

[2]	GSM 02.30: "Digital cellular telecommunication system (Phase 2+); Man-Machine Interface (MMI) of the Mobile Station (MS)".

[3]	GSM 03.90: "Digital cellular telecommunication system (Phase 2+); Unstructured supplementary services operation - Stage 2".

[4]	GSM 03.40: "Digital cellular telecommunication system (Phase 2+); Technical realization of the Short Message Service (SMS) Point to Point (PP)".

[5]	GSM 03.41: "Digital cellular telecommunication system (Phase 2+); Technical realization of Short Message Service Cell Broadcast (SMSCB)".

[6]	GSM 04.11: "Digital cellular telecommunication system (Phase 2+); Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[7]	GSM 04.12: "Digital cellular telecommunication system (Phase 2+); Short Message Service Cell Broadcast (SMSCB) support on the mobile radio interface".

[8]	GSM 07.05: "Digital cellular telecommunication system (Phase 2+); Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[10]	ISO/IEC10646: "Universal Multiple-Octet Coded Character Set (UCS)”; UCS2, 16 bit coding.

[11]	GSM 04.90: "Digital cellular telecommunication system (Phase 2+); Unstructured supplementary services operation - Stage 3".

[12]	ISO 639 “Code for the representation of names of languages”

[13]	GSM 03.42: "Digital cellular telecommunication system (Phase 2+); Compression algorithm for text messaging services”.

[14]	GSM 03.40: "Digital cellular telecommunication system (Phase 2+); Technical realization of the Short Message Service (SMS) Point to Point (PP)".

[15]	"Wireless Datagram Protocol Specification", Wireless Application Protocol Forum Ltd.

# 5	Cell Broadcast Data Coding Scheme

The Cell Broadcast Data Coding Scheme indicates the intended handling of the message at the MS, the alphabet/coding, and the language (when applicable). Any reserved codings shall be assumed to be the GSM default alphabet (the same as codepoint 00001111) by a receiving entity. The octet is used according to a coding group which is indicated in bits 7..4. The octet is then coded as follows:

| Coding Group Bits 7..4 | Use of bits 3..0 |
| --- | --- |
| 0000 | Language using the default alphabet |
|  | Bits 3..0 indicate the language: |
|  | 0000 German |
|  | 0001 English |
|  | 0010 Italian |
|  | 0011 French |
|  | 0100 Spanish |
|  | 0101 Dutch |
|  | 0110 Swedish |
|  | 0111 Danish |
|  | 1000 Portuguese |
|  | 1001 Finnish |
|  | 1010 Norwegian |
|  | 1011 Greek |
|  | 1100 Turkish |
|  | 1101Hungarian 1110 Polish |
|  | 1111 Language unspecified |
| 0001 | 0000 Default alphabet; message preceded by language indication. The first 3 characters of the message are a two-character representation of the language encoded according to ISO 639 [12], followed by a CR character. The CR character is then followed by 90 characters of text. A Pre-Phase 2+ MS will overwrite the start of the message up to the CR and present only the text. 0001 UCS2; message preceded by language indication The message starts with a two 7-bit default alphabet character representation of the language encoded according to ISO 639 [12]. This is padded to the octet boundary with two bits set to 0 and then followed by 40 characters of UCS2-encoded message. An MS not supporting UCS2 coding will present the two character language identifier followed by improperly interpreted user data. 0010..1111 Reserved for European languages |
| 0010.. | 0000 Czech 0001 .. 1111 Reserved for European Languages using the default alphabet, with unspecified handling at the MS |
| 0011 | 0000..1111 Reserved for European Languages using the default alphabet, with unspecified handling at the MS |
| (continued) |  |

(concluded)

| 01xx | General Data Coding indication Bits 5..0 indicate the following: |
| --- | --- |
|  | Bit 5, if set to 0, indicates the text is uncompressed Bit 5, if set to 1, indicates the text is compressed using the GSM standard compressing algorithm. ( see GSM TS 03.42 ) |
|  | Bit 4, if set to 0, indicates that bits 1 to 0 are reserved and have no message class meaning Bit 4, if set to 1, indicates that bits 1 to 0 have a message class meaning: |
|  | Bit 1 Bit 0 Message Class: |
|  | 0 0 Class 0 |
|  | 0 1 Class 1 Default meaning: ME-specific. |
|  | 1 0 Class 2 SIM specific message. |
|  | 1 1 Class 3 Default meaning: TE-specific (see GSM TS 07.05 [8]) |
|  | Bits 3 and 2 indicate the alphabet being used, as follows: |
|  | Bit 3 Bit 2 Alphabet: |
|  | 0 0 Default alphabet |
|  | 0 1 8 bit data |
|  | 1 0 USC2 (16 bit) [10] |
|  | 1 1 Reserved |
| Coding Group Bits 7..4 | Use of bits 3..0 |
| 1000..1101 | Reserved coding groups |
| 1110 | Reserved for WAP [15] |
| 1111 | Data coding / message handling |
|  | Bit 3 is reserved, set to 0. |
|  | Bit 2 Message coding: |
|  | 0 Default alphabet |
|  | 1 8 bit data |
|  | Bit 1 Bit 0 Message Class: |
|  | 0 0 No message class. |
|  | 0 1 Class 1 user defined. |
|  | 1 0 Class 2 user defined. |
|  | 1 1 Class 3 |
|  | default meaning: TE specific (see GSM TS 07.05 [8]) |

These codings may also be used for Unstructured SS Data and MMI/display purposes.

See GSM 04.90 [11] for specific coding values applicable to Unstructured SS Data for MS originated USSD messages and MS terminated USSD messages. USSD messages using the default alphabet are coded with the 7-bit alphabet given in subclause 6.2.1. The message can then consist of up to 182 user characters.

Cell Broadcast messages using the default alphabet are coded with the 7-bit alphabet given in subclause 6.2.1. The message then consists of 93 user characters.

If the 7 bit default alphabet extension mechanism is used then the number of displayable characters will reduce by one for every instance where the 7 bit default alphabet extension table is usedCell Broadcast messages using 8-bit data have user-defined coding, and will be 82 octets in length.

UCS2 alphabet indicates that the message is coded in UCS2 [10]. The General notes specified in subclause 6.1.1 override any contrary specification in UCS2, so for example even in UCS2 a <CR> character will cause the MS to return to the beginning of the current line and overwrite any existing text with the characters which follow the <CR>. Messages encoded in UCS2 consist of 41 characters.

Class 1 and Class 2 messages may be routed by the ME to user-defined destinations, but the user may override any default meaning and select their own routing.

Class 3 messages will normally be selected for transfer to a TE, in cases where a ME supports an SMS/CBS interface to a TE, and the TE requests "TE-specific" cell broadcast messages (see GSM 07.05 [8]). The user may be able to override the default meaning and select their own routing.