ETSI SMG 	TSGT2#4(99) 496

Miami

11/6-13/6-99

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.07 |  | Version: |  |  | 7.2.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | EDGE circuit switched data |
| --- | --- |

| Source: | Ericsson | Date: | 1999-05-26 |
| --- | --- | --- | --- |

| Subject: | ECSD additions |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | The introduction of ECSD requires additions to the +CHSD and +CHSC command in order to cater for the new channel codings and the new 56/64k air interface user rates. |
| --- | --- |

| Clauses affected: | 6.12, 6.15 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 6.12	HSCSD device parameters +CHSD

Table 1: +CHSD action command syntax

| Command | Possible response(s) |
| --- | --- |
| +CHSD | +CHSD: <mclass>,<maxRx>,<maxTx>,<sum>,<codings> +CME ERROR: <err> |
| +CHSD=? |  |

Description

Execution command returns information about HSCSD features (refer GSM 02.34 [29]) supported by the ME/TA. Refer subclause 9.2 for possible <err> values.

Defined values

<mclass>: integer type; multislot class

<maxRx>: integer type; maximum number of receive timeslots that ME can use

<maxTx>: integer type; maximum number of transmit timeslots that ME can use

<sum>: integer type; total number of receive and transmit timeslots that ME can use at the same time (per TDMA frame). The following applies in a HSCSD call: 1  (receive slots) + (transmit slots)  <sum>

<codings> is a sum of integers each representing a supported channel coding (e.g. value 5 indicates that 4.8k and 9.6k channel codings are supported):

1	4.8k full rate data traffic channel

4	9.6k full rate data traffic channel

8	14.4k full rate data traffic channel

16	28.8k full rate data traffic channel  (only possible when 14.4k is supported)

32	32.0k full rate data traffic channel  (only possible in a two-timeslot configuration)

64	43.2k full rate data traffic channel  (only possible when 14.4k is supported)

Implementation

Mandatory when HSCSD implemented.

## 6.13	HSCSD transparent call configuration +CHST

Table 2: +CHST parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CHST=[<wRx>[,<codings>]] |  |
| +CHST? | +CHST: <wRx>,<codings> |
| +CHST=? |  |

Description

Set command controls parameters for transparent HSCSD calls. Changing them during a call does not affect the current call.

Defined values

<wRx>: integer type; wanted amount of receive timeslots. Default value 0 indicates that TA shall calculate a proper value from currently selected fixed network user rate (<speed> subparameter from +CBST command) and <codings>

<codings>: a sum of integers each representing a channel coding that is accepted for transparent HSCSD calls. Default value 0 indicates that all supported codings are accepted (refer +CHSD command for other values)

Implementation

Mandatory when transparent HSCSD implemented.

## 6.14	HSCSD non-transparent call configuration +CHSN

Table 3: +CHSN parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CHSN=[<wAiur>[,<wRx>[,<topRx> [,<codings>]]]] |  |
| +CHSN? | +CHSN: <wAiur>,<wRx>,<topRx>,<codings> |
| +CHSN=? | +CHSN: <maxAiur>,<modify> |

Description

Set command controls parameters for non-transparent HSCSD calls. Changing <topRx> or <codings> value during a call does not affect the current call. Changing of <wAiur> or <wRx> affects the current call only if <topRx> was non-zero when call was established.

Defined values

<wAiur>: integer type; wanted air interface user rate. Default value 0 indicates that TA shall calculate a proper value from currently selected fixed network user rate (<speed> subparameter from +CBST command), <codings>, and <wRx> (or <maxRx> from +CHSD command if <wRx>=0). Other values:

1	9600 bps

2	14400 bps

3	19200 bps

4	28800 bps

5	38400 bps

6	43200 bps

7	57600 bps

<wRx>: integer type; wanted amount of receive timeslots. Default value 0 indicates that TA shall calculate a proper value from currently selected <wAiur> and <codings>

<topRx>: integer type; top value for <wRx> that user is going to request during the next established non-transparent HSCSD call. Default value 0 indicates that user is not going to change <wAiur>/<wRx> during the next call

<codings>: a sum of integers each representing a channel coding that is accepted for non-transparent HSCSD calls. Default value 0 indicates that all supported codings are accepted (refer +CHSD command for other values)

<maxAiur>: integer type; maximum value for <wAiur> (assuming that all supported channel codings are accepted and maximum number of timeslots are used)

<modify>:

0	<wAiur>/<wRx> modification during call is not supported by ME/TA (<topRx> accepts only 0)

1	<wAiur>/<wRx> modification during call is supported by ME/TA

Implementation

Mandatory when non-transparent HSCSD implemented.

## 6.15	HSCSD current call parameters +CHSC

Table 4: +CHSC action command syntax

| Command | Possible response(s) |
| --- | --- |
| +CHSC | +CHSC: <rx>,<tx>,<aiur>,<coding> |
| +CHSC=? |  |

Description

Execution command returns information about current HSCSD call. If no HSCSD call is active, all parameters returned shall equal zero. (It is manufacturer specific whether non-zero information is returned in case of an active normal single-slot data call.)

Defined values

<rx>: integer type; number of receive timeslots currently in use

<tx>: integer type; number of transmit timeslots currently in use

<aiur>: integer type; current air interface user rate (in case of transparent service this equals fixed network user rate) (refer +CHSN command for possible values).  For the two-timeslot ECSD bit transparent configuration the following additional values apply:

8	56000 bps

9	64000 bps

<coding>: current channel coding (refer +CHSD command for possible values)

Implementation

Optional.