| TSG-T2 / SMG4 MExE 14th-16th June 1999 Miami, USA | Tdoc T2-99544 |
| --- | --- |

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.10.1 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Motorola | Date: |  |
| --- | --- | --- | --- |

| Subject: | Limited network access for untrusted applications |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Introduction of limited network access to support WAP devices. |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 8.2		MExE executable permissions

The following table 3 specifies the permissions of each security domain.

The actions listed in the security table 3 are generic actions. Application programming interfaces (APIs) which are intrinsically part of the MExE implementation may directly or indirectly cause these actions to be performed. When these actions occur, whether directly or indirectly called by such an API, the security restrictions shall apply.

Explicit user permission is required for all actions by MExE executables in all domains.  Types of user permission are defined in clause 8.3

File access is not permitted for untrusted MExE executables, except that untrusted Java applications can access files only in the application’s own directory. Untrusted MExE executables are not permitted access to any actions which access the phone functionality (phone functionality includes all the actions in the table 3) except for this one qualified exception.

Actions available using interfaces giving access to the phone functionality (either in existence at the time of approval of this specification or not) that are not listed or not of the same category as a group in the security table 3 can be accessed by all trusted MExE executables. The actions cannot be accessed by untrusted MExE executables.

|  |  | MExE Security Domains |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Actions |  |  | Operator | Manufacturer |  | Trusted Third Party |  |
| User private data access 1 - Read - Write - Get properties - Delete - Get Location Information - Read stored SMS - Delete stored SMS |  | Yes 2 |  |  |  |  |
| Network access - Initiate a voice/data connection 3 - Accept a voice/data connection 3 - Call forward 4 - Multiparty call 4 - Call deflection 4 - Explicit call transfer 4 - Terminate an existing connection - Hold an existing connection - Resume an existing connection - Send point-point message (e.g. SMS, USSD) 4 - Generate DTMF - Query network status - Get signal level - Get call list |  | Yes |  |  |  | Yes 6 |
| Lifecycle management - Install a MExE Executable - Uninstall a MExE executable |  | Yes |  |  |  |  |
| Miscellaneous functions - Get manufacturer software version |  | Yes |  |  |  |  |
| Certificate management - Install a certificate for a given domain - Uninstall a certificate for a given domain - Replace a certificate for a given domain |  | Yes 5 |  |  |  |  |
| Audio access - Sound generation to speaker (e.g. via stream) - Select Melody - Set speaker volume - Get melody list |  | Yes |  |  |  |  |
| Power management - Monitor the power state - Change the power state |  | Yes |  |  |  |  |
| Network selection/operator data access - Get IMSI - Get home network - Select network | Yes |  |  | No |  |  |
| Device core function access - Start/stop radio - Turn on/off device | No |  |  |  |  |  |
| User profile management - Activate a user profile - Modify a user profile | No |  |  |  |  |  |
| User preference management Modify user preferences | Yes7 |  |  |  |  |  |
| Legend for above footnotes:- 1 – User private data includes user files, phonebook, etc located on the MS. 2 – The user shall be able to specify data access permissions within the capabilities of the device. 3 – A network connection may be via any supported bearer service 4 – Multiparty, deflection, and explicit call transfer shall be permitted only to numbers explicitly supplied by the user to the MExE Executable. Modification of call forward numbers stored in the network shall only be permitted to numbers explicitly supplied by the user to the operator. 5 – Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate. 6– The Trusted Third Party domain's permission to access the networking action depends on the provisioning mechanism as described in section 8.15 7 – Trusted applications only have permission to modify user preferences, and not to actiivate or de-activate them. The user shall be able to specify for each domain, the preferences that applications in that domain can access. All other preferences shall not be accessible to that domain. The default shall be that there is no access. |  |  |  |  |  |  |

Table 3: Security domains and actions

The lists in the groups in the table 3 are not exhaustive, and other actions which are of the same category shall be included in the group for the purposes of requesting user permission.

### 8.2.1	MExE executable permissions for untrusted applications

Clause 8.2 identifies the permissions for MExE executables in the 3 domains (operator, MS manufacturer and Trusted Third Party).  The permissions do not apply to untrusted applications which are not permitted to execute within the domains.

In order to facilitate untrusted applications having some limited access to MExE MS functionality beyond their very limited privileges, the following specific network access permissions in Table 3 are extended to untrusted applications:-

- Initiate a voice/data connection

Untrusted applications shall be able to make calls under the following conditions.

In addition to an untrusted application possibly displaying the number to be called to the user, the number to be called shall be presented to the user for permission by a provisioned application of the MExE MS and not by the application itself.  (This facility would support, for example, "click to dial" button/links in an untrusted application, and a MExE MS provisioned application then represents the number to the user for confirmation.)

- Generate DTMF

Untrusted applications shall be able to generate DTMF tones under the following conditions.

An untrusted application is only permitted to send DTMF tones in a currently active call.  The request to generate DTMF tones in the currently active call, shall result in the characters which the tones represent being presented to the user for permission by a provisioned application of the MExE MS.

The untrusted applications permitted to use the above facilities shall be applications the user has downloaded himself, and not be applications that have been pushed to the user.  Applications/applets on the MExE MS due to the user having visited a particular web site are considered to be applications that the user had downloaded himself.

Untrusted applications shall not be permitted access to any other permissions identified in Table 3.