ETSI SMG4/ 3GPP T2	T2-99662

Helsinki

6-9 September 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.10 |  | Version: |  |  | 6.3.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | x |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | TEI |
| --- | --- |

| Source: | Nokia | Date: | 27/08/1999 |
| --- | --- | --- | --- |

| Subject: | Clarification of CR bit |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | When control messages are sent on channel 0 there are two fields which contains a CR bit: - CR bit on field "Address" - CR bit on field "type". The use of the CR bit is different for both fields, and the document does not specify such difference. This can lead to a misunderstanding. |
| --- | --- |

| Clauses affected: | 5.4.6.2 Operating procedures |
| --- | --- |

| Other specs | Other releases of same spec | x |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications | x |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

#### 5.4.6.2	Operating procedures

Messages always exist in pairs; a command message and a corresponding response message. If the C/R bit is set to 1 the message is a command, if it is set to 0 the message is a response. A response message has the same T bits as the command that provoked it.

If a command does not produce a response within a time T2 the command may be sent again up to N2 times. If no response is received on the N2 transmissions, the multiplexer control channel should be considered faulty and an alarm raised. Resolution of the error situation is implementation dependent.

Note: Notice that when UIH frames are used to convey information on DLCI 0 there are at least two different fields that contain a CR bit, and the bits are set of different form. The CR bit in the Type field shall be set as it is stated above, while the CR bit in the Address field (see subclause 5.2.1.2)  shall be set as it is described in subclause 5.4.3.1.

5.4.6.3	Message Type and Actions

##### 5.4.6.3.1 	DLC parameter negotiation (PN)

This procedure is optional. If this command is not supported, default values are applied to each DLC.

Before a DLC is set up using the mechanism in subclause 5.4.1, the TE and MS must agree on the parameters to be used for that DLC. These parameters are determined by parameter negotiation.

The parameter negotiation uses the following type field octet:

| Bit | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EA | C/R | 0 | 0 | 0 | 0 | 0 | 1 |

The length field octet contains the value 8 and there follow eight value octets. The value octets contain the information in Table 1.

Table 1: Parameter Negotiation

| Value Octet | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D1 | D2 | D3 | D4 | D5 | D6 | 0 | 0 |
| 2 | I1 | I2 | I3 | I4 | CL1 | CL2 | CL3 | CL4 |
| 3 | P1 | P2 | P3 | P4 | P5 | P6 | 0 | 0 |
| 4 | T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 |
| 5 | N1 | N2 | N3 | N4 | N5 | N6 | N7 | N8 |
| 6 | N9 | N10 | N11 | N12 | N13 | N14 | N15 | N16 |
| 7 | NA1 | NA2 | NA3 | NA4 | NA5 | NA6 | NA7 | NA8 |
| 8 | K1 | K2 | K3 | 0 | 0 | 0 | 0 | 0 |

The various fields are coded as follows:

The D-bits define the DLCI that the other information refers to; Bit D1 is the least significant.

The I-bits define the type of frames used for carrying information in the particular DLC - See Table 2.

Table 2: Meaning of I-bits

| Meaning | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- |
| Use UIH frames | 0 | 0 | 0 | 0 |
| Use UI frames | 1 | 0 | 0 | 0 |
| Use I frames (note) | 0 | 1 | 0 | 0 |