3GPP TSG T WG2, SWG1 (MExE)	Tdoc T2-99728

Helsinki, Findland

6 – 10 September, 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 23.057 |  | Version |  | 1.3.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  | x |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Alcatel Business System | Date: | 23/08/99 |
| --- | --- | --- | --- |

| Subject: | Addition of actions in security domains and actions table |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | x |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Action access to trusted application is limited to action listed in security table. So we need to add several action access - Input / Output User interface - User notification - Date/clock - Serial port - Printer - Smart card - External application - GSM and other security function |
| --- | --- |

| Clauses affected: | 3.2, 8.2 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: | Release 98 |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | - A melody is a user notification as a vibrator activation, a light flash. - Melody selection is a user preference, so it is cover by user preference category. - User notification associated to an event is a user preference. - WAP is providing a library to sign a content. - Java allow to launch an other application, WAP would like create a library to launch an application (SIM tool kit …) - SIM smart card access (send APU) is forbidden for all application what ever the application supported by SIM card (for example WIM application) |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 3.2	Abbreviations

For the purposes of this TS the following abbreviations apply:

API	Application Programming Interface

APDU	Application protocol data unit

CA	Certification Authority

CC/PP	Composite Capability/Preference Profiles

CGI	Common Gateway Interface

CCM	Certificate Configuration Message

CP-Admin	Certificate Present (in the MExE SIM) - Administrator

CP-TP	Certificate Present (in the MExE SIM) - Third Party

DHCP	Dynamic Host Configuration Protocol

GSM	Global System for Mobile Communication

HTTP	HyperText Transfer Protocol

HTTPS	HyperText Transport Protocol Secure (https is http/1.1 over SSL, i.e. port 443)

IP	Internet Protocol

JNDI	Java Naming Directory Interface

JTAPI	Java Telephony Application Programming Interface

JAR file	Java Archive File

MSE	MExE Service Environment

OCF	OpenCard Framework

RDF	Resource Description Format

RFC	Request For Comments

SAP	Service Access Point

SMS	Short Message Service

TLS	Transport Layer Security

TP	Third Party

UDP	User Datagram Protocol

UI	User Interface

URL	Uniform Resource Locator

URI	Uniform Resource Identifier

USSD	Unstructured Supplementary Service Data

WAE	Wireless Application Environment

WAP	Wireless Application Protocol

WDP	Wireless Datagram Protocol

WSP	Wireless Session Protocol

WTA	Wireless Telephony Applications

WTAI	Wireless Telephony Applications Interface

WTLS	Wireless Transport Layer Security

WTP	Wireless Transaction Protocol

WWW	World Wide Web

Further abbreviations are given in GSM 02.57 (MExE stage 1) [2] and GSM 01.04 [1].

## 8.2	MExE executable permissions

The following table 3 specifies the permissions of operator, manufacturer and third party security domains.

The actions listed in the security table 3 are generic actions. Application programming interfaces (APIs) which are intrinsically part of the MExE implementation may directly or indirectly cause these actions to be performed. When these actions occur, whether directly or indirectly called by such an API, the security restrictions shall apply.

Explicit user permission is required for all actions by MExE executables in all domains.  Types of user permission are defined in clause 8.3

File access is not permitted for untrusted MExE executables, except that untrusted Java applications can access files only in the application’s own directory. Untrusted MExE executables are not permitted access to any actions which access the phone functionality (phone functionality includes all the actions in the table 3) except for this one qualified exception, and as further identified in 8.2.1.

Actions available using interfaces giving access to the phone functionality (either in existence at the time of approval of this specification or not) that are not listed or not of the same category as a group in the security table 3 can be accessed by all trusted MExE executables.

Table 3: Security domains and actions

|  |  | MExE Security Domains |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Actions |  |  | Operator | Manufacturer |  | Third Party |  |
| User private data access 1 - Read - Write - Get properties - Delete - Get Location Information - Read stored SMS - Delete stored SMS |  | Yes 2 |  |  |  |  |
| Network access - Initiate a voice/data connection 3 - Accept a voice/data connection 3 - Call forward 4 - Multiparty call 4 - Call deflection 4 - Explicit call transfer 4 - Terminate an existing connection - Hold an existing connection - Resume an existing connection - Send point-point message (e.g. SMS, USSD) 4 - Generate DTMF - Query network status - Get signal level - Get call list |  | Yes |  |  |  | Yes 6 |
| Lifecycle management - Install a MExE Executable - Uninstall a MExE executable |  | Yes |  |  |  |  |
| Terminal data - Get manufacturer software version - Read time and date - Write time and date |  | Yes Yes No |  |  |  |  |
| Certificate management - Install a certificate for a given domain - Uninstall a certificate for a given domain - Replace a certificate for a given domain |  | Yes 5 |  |  |  |  |
| GSM Security function | No |  |  |  |  |  |
| Security function other than GSM - Data encryption - Verify a signature - Compute a digital signature - Hash a content - Non repudiation |  | Yes Author note : the need of this category should be confirmed |  |  |  |  |
| Input output User interface - Input device (keyboard, mouse …) - Output device (display ) - Output notification device(smart icon, sound, light, vibrator …) |  | Yes10 Yes10 Yes |  |  |  |  |
| Audio access - Sound generation to speaker (e.g. via stream) - Set speaker volume |  | Yes |  |  |  |  |
| Power management - Monitor the power state - Change the power state |  | Yes |  |  |  |  |
| Network selection/operator data access - Get IMSI - Get home network - Select network | Yes |  |  | No |  |  |
| Device core function access - Start/stop radio - Turn on/off device | No |  |  |  |  |  |
| Printer access | Yes |  |  |  |  |  |
| External port low level access12 - Serial port (RS232, Irda, Bluetooth, USB …) - Parallel port | No/Yes Author note : access status stay open |  |  |  |  |  |
| SIM smart card low level access11 - Send APDU - Slot management (power on/off, reset, port lock…) | No |  |  |  |  |  |
| Smart card other than SIM card low level access - Send APDU - Slot management (power on/off, reset, port lock …) | Yes/No Author note : access status stay open |  |  |  |  |  |
| Application access - Get application list - Launch an application - Get application status - Stop, suspend, resume an application | Yes8 Yes8 Yes8 Yes9 |  |  |  |  |  |
| User profile management - Activate a user profile - Modify a user profile | No |  |  |  |  |  |
| User preference management Modify user preferences | Yes7 |  |  |  |  |  |
| Legend for above footnotes:- 1 – User private data includes user files, phonebook, etc located on the MS. 2 – The user shall be able to specify data access permissions within the capabilities of the device. 3 – A network connection may be via any supported bearer service 4 – Multiparty, deflection, and explicit call transfer shall be permitted only to numbers explicitly supplied by the user to the MExE Executable. Modification of call forward numbers stored in the network shall only be permitted to numbers explicitly supplied by the user to the operator. 5 – Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate. 6– The Third Party domain's permission to access the networking action depends on the provisioning mechanism as described in section 8.15 7 – Trusted applications only have permission to modify user preferences, and not to activate or de-activate them. The user shall be able to specify for each domain, the preferences that applications in that domain can access. All other preferences shall not be accessible to that domain. The default shall be that there is no access. Single action user permission is the only type of user permission that shall be possible for changes to User Preferences. 8 – Device native application access is limited to manufacturer domain. SIM tool kit application access is limited to operator domain. MExE application access is limited to MExE application issued by the same issuer (identify by the certificate) of launched application 9 – Access is limited to MExE application which launch the application. But the end user, shall have a way to stop the launched application, himself, MExE environment may stop the launched application or launched application may stop itself. 10 – Access request no user permission. 11 – Access to SIM is provided using more high level API as phonebook, application lauching 12 – Access to external port is provided by more high level API as data user synchronisation, localisation information |  |  |  |  |  |  |