| 3GPP TSG-T2 #5 / ETSI SMG4 Helsinki, FINLAND, 6-9 September 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TSGT2#5(99)672 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 27.007 |  |  |  |  | CR |  |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.1.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submision to TSG |  |  |  |  | #5 |  |  |  | for approval |  |  |  |  |  | x |  |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  |  | ME |  |  |  |  |  |  |  |  | x |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | SMG4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 08/09/99 |  |  |  |  |
| Subject: |  |  | Time Zone |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | Technical Enhancements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | Added functionality to set up time zone settings and reporting. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | 8.xx, Annex B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

8.x	Automatic Time Zone Update +CTZU

Table 1: +CTZU parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CTZU=<onoff> | +CME ERROR: <err> |
| +CTZU? | +CTZU: <onoff> +CME ERROR: <err> |
| +CTZU=? | +CTZU: (list of supported <onoff>s) +CME ERROR: <err> |

Description

Set command enables and disables automatic time zone update via NITZ. If setting fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

Read command returns the current settings in the ME.

Test command returns supported on- and off-values.

Defined values

<onoff>: integer type value indicating:

0 – Disable automatic time zone update via NITZ (default).

1 – Enable automatic time zone update via NITZ.

Implementation

Optional.

8.z	Time Zone Reporting +CTZR

Table 2: +CTZR parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CTZR=<onoff> | +CME ERROR: <err> |
| +CTZR? | +CTZR: <onoff> +CME ERROR: <err> |
| +CTZR=? | +CTZR: (list of supported <onoff>s) +CME ERROR: <err> |

Description

This set command enables and disables the time zone change event reporting. If the reporting is enables the ME returns the unsolicited result code +CTZV: <tz> whenever the time zone is changed. If setting fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

Read command returns the current reporting settings in the ME.

Test command returns supported <onoff>-values.

NOTE: The Time Zone reporting is not affected by the Automatic Time Zone setting command, +CTZU.

Defined values

<onoff>: integer type value indicating:

0 – disable time zone change event reporting (default).

1 – Enable time zone change event reporting.

Implementation

Optional.

Annex B (normative):
Summary of result codes

V.25ter [14] result codes which can be used in GSM and codes defined in this ETS:

Table B.1: Result codes

| Verbose result code (V.25ter command V1 set) | Numeric (V0 set) | Type | Description |
| --- | --- | --- | --- |
| +CTZV: <tz> | as verbose | unsolicited | refer subclause 8.z |
| +CCCM: <ccm> | as verbose | unsolicited | refer subclause 7.15 $(AT R97)$ |
| +CCWA: <number>,<type> ,<class>[,<alpha>] | as verbose | unsolicited | refer subclause 7.11 |
| +CCWV | as verbose | unsolicited | refer subclause 8.28 |
| +CDEV: <elem>,<text> | as verbose | unsolicited | refer subclause 8.10 |
| +CIEV: <ind>,<value> | as verbose | unsolicited | refer subclause 8.10 |
| +CKEV: <key>,<press> | as verbose | unsolicited | refer subclause 8.10 |
| +CLAV: <code> | as verbose | unsolicited | refer subclause 8. |
| +CLIP: <number> ,<type>[,<subaddr> ,<satype>[,<alpha>]] | as verbose | unsolicited | refer subclause 7.6 |
| +CME ERROR: <err> | as verbose | final | refer subclause 9.2 |
| +COLP: <number> ,<type>[,<subaddr> ,<satype>[,<alpha>]] | as verbose | intermediate | refer subclause 7.8 |
| +CR: <type> | as verbose | intermediate | refer subclause 6.8 |
| +CREG: <stat>[,<lac> ,<ci>] | as verbose | unsolicited | refer subclause 7.2 |
| +CRING: <type> | as verbose | unsolicited | refer subclause 6.11 |
| +CSSI: <code1> [,<index>] | as verbose | intermediate | refer subclause 7.16 |
| +CSSU: <code2> [,<index>[,<number>, <type>[,<subaddr>, <satype>]]] | as verbose | unsolicited | refer subclause 7.16 |
| +CUSD: <m>[,<str>,<dcs>] | as verbose | unsolicited | refer subclause 7.14 |
| +DR: <type> | as verbose | intermediate | refer subclause 6.13 |
| +ILRR: <rate> | as verbose | intermediate | refer subclause 4.3 |
| BUSY | 6 | final | busy signal detected |
| CONNECT | 1 | intermediate | connection has been established |
| CONNECT <text> | manufacturer specific | intermediate | as CONNECT but manufacturer specific <text> gives additional information (e.g. connection data rate) |
| ERROR | 4 | final | command not accepted |
| NO ANSWER | 7 | final | connection completion timeout |
| NO CARRIER | 3 | final | connection terminated |
| NO DIALTONE | 5 | final | no dialtone detected |
| OK | 0 | final | acknowledges execution of a command line |
| RING | 2 | unsolicited | incoming call signal from network |