Liaison statement

To:	TSG CN1, TSG CN2, TSG T2

Cc:

From:	3GPP TSG S2

# Title:	Multimedia Call Control for UMTS R 99

TSG S2 would like to inform that TSG S2 has specified the following:

- H.323 shall be the multimedia call control protocol for the PS domain in UMTS R99.

Thus the revised principles for the support of multimedia in UMTS are (from 23.121):

- P1) GSM/UMTS shall enable the provisioning of multimedia services and multivendor interworking between UE and network.

- P2) Basic voice and PDP-context establishment shall be based on GSM CC/SM respectively.

- P3) Handover and roaming to and from GSM shall be supported provided GSM is capable of supporting the ongoing media service.

- P4) Ideas, concepts and procedures developed by other fora e.g. other standards bodies such as ITU, IETF etc. shall be included or referenced in GSM/UMTS when found suitable.

- P5) To ensure multi-vendor inter-working and UE roaming, a single standardised multimedia protocol for CS domain and a single standardised multimedia protocol for PS domain shall be  selected for GSM / UMTS R99. This does not preclude the selection of other protocols by UMTS in the future.

- H.323 shall be the multimedia call control model for the PS domain in UMTS R99.

- P6) For multimedia services the standardized multimedia protocol shall be run transparently via a PDP-context or a circuit-switched connection established using GSM SM/CC . This allows transparent hand-over and roaming between GSM and UMTS provided that GSM supports the QoS requirements.