| 3GPP/SMG Meeting TSG-T2#7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | T2-991027 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ystad, SWEDEN, 22-26 Nov 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 23.057 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 1.5.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | BTCellnet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 22 November 99 |  |  |  |  |  |
| Subject: |  |  | Inclusion of QoS in MExE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | SWG 1 (MExE) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | MExE QoS requirements are not yet reflected in MExE stage 2 specification, this CR adds QoS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 2.1, 3.2, 4.12, 9. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.1	Normative references

[1]	GSM 01.04: “Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms”.

[2]	GSM 02.57: "Digital cellular telecommunications system (Phase 2+);MExE Stage 1 Description"

[3]	Personal Java 1.1.1, Sun Microsystems http://java.sun.com/products/personaljava/spec-1-1-1/index.html.

[4]	JavaPhone API version 0.9, http://java.sun.com/products/javaphone/.

[5]	JTAPI 1.2, Sun Microsystems http://www.java.sun.com.

[6]	Wireless Application Protocol (WAP) version 1.1 http://www.wapforum.org.

[7]	vCard – The Electronic Business Card Exchange Format – Version 2.1, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/vcard-21.doc.

[8]	vCalendar – The Electronic Calendaring and Scheduling Exchange Format – Version 1.0, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/

[9]	Hypertext Transfer Protocol – HTTP/1.1, IETF document RFC2068, http://www.w3.org/Protocols/rfc2068/rfc2068

[10]	Java Mail API version 1.0.2, http://www.java.sun.com

[11]	UMTS TR 22.170: “Universal Mobile Telecommunications System (UMTS); Service aspects; Provision of Services in UMTS - The Virtual Home Environment”.

[12]	UMTS TS 22.121: “Universal Mobile Telecommunications System (UMTS); Provision of Services in UMTS - The Virtual Home Environment: Stage 1”.

[13]	ISO 639 International Standard - codes for the representation of language names

[14]	UMTS TS 22.101: “Universal Mobile Telecommunications System (UMTS); Service Aspects; Service Principles”.

[15]	CC/PP Exchange Protocol based on HTTP Extension Framework; W3C http://www.w3.org/TR/NOTE-CCPPexchange

[16]	Composite Capability/Preference Profiles (CC/PP):A user side framework for content negotiation;  Available at W3C web pages.

[17]	UAProf  Specification http://www.wapforum.org

[18]	JDK 1.1 security http://www.javasoft.com/products/jdk/1.1/docs/guide/security/index.html

[19]	Java 2 security http://www.javasoft.com/products/jdk/1.2/docs/guide/security/index.html

[20]	Java security tutorial http://java.sun.com/docs/books/tutorial/security1.2/overview/index.html

[21]	OCF 1.1.: “Smartcard API specified by OpenCard Consortium http://www.opencard.org

[22]	RFC 1738 Uniform Resource Locators (URL) http://www.w3.org/pub/WWW/Addressing/rfc1738.txt

[23]	The MD5 Message Digest Algorithm", Rivest, R., RFC 1321, April 1992.  URL: ftp://ftp.isi.edu/in-notes/rfc1321.txt

[24]	ISO/IEC 10118-3 1996, Information technology - Security techniques - Hash-functions - Part 3: Dedicated hash-functions

[25]	IETF RFC 2368: The mailto URL scheme

[26]	ITU-T X.509: Information technology – Open Systems Interconnection – The Directory: Authentication framework

[27]	GSM 11.11: "Digital cellular telecommunications system (Phase 2+); Specification of the Subscriber Identity Module – Mobile Equipment (SIM-ME) interface.

[28]	UMTS TS 23.107: '3rd Generation Partnership Project; Technical Specification Group Services and system Aspects QoS Concept and Architecture (3G TR 23.107)

## 3.2	Abbreviations

For the purposes of this TS the following abbreviations apply:

API	Application Programming Interface

APDU	Application protocol data unit

CA	Certification Authority

CC/PP	Composite Capability/Preference Profiles

CGI	Common Gateway Interface

CCM	Certificate Configuration Message

CP-Admin	Certificate Present (in the MExE SIM) - Administrator

CP-TP	Certificate Present (in the MExE SIM) - Third Party

Diff-serv	Differentiated Services

DHCP	Dynamic Host Configuration Protocol

GSM	Global System for Mobile Communication

GPRS	General Packet Radio Service

HTTP	HyperText Transfer Protocol

HTTPS	HyperText Transport Protocol Secure (https is http/1.1 over SSL, i.e. port 443)

IETF	Internet Engineering Task Force

IP	Internet Protocol

IPv4	IP version 4

IPv6	IP version 6

JNDI	Java Naming Directory Interface

JTAPI	Java Telephony Application Programming Interface

JAR file	Java Archive File

MMI	Man-Machine Interface

MSE	MExE Service Environment

OCF	OpenCard Framework

QoS	Quality of Service

RDF	Resource Description Format

RFC	Request For Comments

SAP	Service Access Point

SMS	Short Message Service

TLS	Transport Layer Security

TOS	Type of Service

TP	Third Party

UDP	User Datagram Protocol

UE	User Equipment

UI	User Interface

UMTS	Universal Mobile Telecommunications System

URL	Uniform Resource Locator

URI	Uniform Resource Identifier

USSD	Unstructured Supplementary Service Data

WAE	Wireless Application Environment

WAP	Wireless Application Protocol

WDP	Wireless Datagram Protocol

WSP	Wireless Session Protocol

WTA	Wireless Telephony Applications

WTAI	Wireless Telephony Applications Interface

WTLS	Wireless Transport Layer Security

WTP	Wireless Transaction Protocol

WWW	World Wide Web

# 4	Generic MExE aspects

This section defines the common aspects of all MExE compliant devices, independent of MExE technology.

### 4.12		Quality of Service

Quality of Service (QoS) [28] is seen by the end user as a measure of the amount of network resources given to an application by the underlying network.  The network may employ a number of QoS mechanisms, but the end user / application is not involved in these.  The end user / application requires an interface into the network QoS through a visible set of standard parameters.

A QoS aware application will request a QoS from the network at the beginning of a QoS session.  The network should attempt negotiate and maintain this level of QoS.  Changes in the level of QoS provided should be notified to the end user / application.  An application / user may have several QoS streams open simultaneously.

The MExE application should be able to dynamically request a change in the level of QoS.  The network is free to reject a QoS modification request, upon which the end user / application must be notified and decide whether to continue the session, to attempt another re-negotiation, or terminate the QoS session.

The QoS subscription parameters are stored in the network, they identify the maximum permissible QoS that a user may negotiate with the network.  Several QoS subscriptions are possible for one user.

# 9	Quality of Service

QoS aware applications may be executing on the MExE device.  To ensure correct operation with the QoS provisioning of the bearer network(s), a MExE QoS manager and associated API's are required – see figure xxx.  Non QoS aware applications will continue to operate with a best effort QoS, provided by the network.

Figure xxx – MExE Terminal QoS manager elements.

## 9.1	MExE QoS Support

A QoS manager is provided between the application and the network.  To interface this, an API to the MExE application is provided and another API to the network, see figure xxx.  The method of QoS support may vary between network implementations.  The MExE QoS functions should accommodate standard methods of end to end QoS provisioning – eg differentiated services (Diff-serv).  For differentiated services data, each IP header contains QoS information, which is interpreted by the IP nodes and routers, it is used to make hop by hop resource allocation decisions.

The MExE device should at least support basic QoS operations :-

- There should be adequate mapping between the QoS requirements of the MExE executable and the network layer.

- MExE executables shall be able to indicate and interpret QoS values of the UMTS network,

- MExE executables shall be able to modify the network QoS dynamically.

- MExE executables shall be able to respond to changes in the provided QoS level.

- The end user shall be able to manage the QoS directly, without requiring the intervention of the QoS API.

MExE introduces two new elements to cater for QoS – the MExE QoS manager and the QoS API.  If the existing network control API does not have QoS capabilities, then this may be achieved through the MExE QoS manager.

## 9.2	QoS API

The QoS API provides the MExE executable with an interface to the network QoS control.  It does not require the MExE executable to have any knowledge of the underlying network, or how QoS is implemented in the network.

The QoS API shall provide the MExE executable with a standard set of parameters – identified in subclause 'QoS API parameters'.

## 9.3	MExE QoS Manager

The MExE QoS manager is responsible for:

- Managing the QoS streams for MExE executables

- Notification of the negotiated and delivered QoS to the MExE executable

- Traffic control according to QoS

The MExE QoS manager may provide QoS signalling (eg Diff-serv) to the network, if the network API does not do so.

The MExE QoS manager shall translate between the QoS parameters of the application and those of the network.

The MExE QoS manager shall monitor the QoS delivered by the network and manage QoS requests between the application and network.

The MExE QoS Manager shall be informed of the requested QoS and minimum QoS for each application.  It may attempt to re-negotiate the QoS if it falls below a threshold.  If a suitable algorithm is in place, the QoS may be allowed to fall below the threshold for a limited period, as the service may not always require 100% network availability.

Information regarding the level of QoS available within the network may be requested by the QoS manager.

The MExE QoS manager does not need to know the subscribed QoS, this is held within the network and used to validate a requested QoS level.

The end user may also access the MExE QoS manager directly, through the MMI.

## 9.4	Network Control API

The network control API shall provide the QoS manager with access to the network specific QoS control (defined for GPRS/UMTS  in 24.007 and 24.008).

The MExE QoS manager may perform some QoS control, if it is not provided in the network control.

## 9.5	QoS API Parameters

The QoS parameters of the MExE QoS manager shall be consistent with those of external network elements (eg IP routers).

The FLOWSPEC parameters, defined by the IETF Integrated Services Working Group, provide the QoS information required by QoS capable network elements.  Table xxx shows the set of Flowspec parameters.

- Flowspec parameters shall be used, for the MExE QoS API, between the MExE executable and the QoS manager.

- Flowspec parameters shall be passed to the network control functions through the Network Control API.

| Parameter | Units | Type |
| --- | --- | --- |
| Token Bucket Rate | bytes /sec | 32-bit IEEE floating point number |
| Token Bucket Size | bytes | 32-bit IEEE floating point number |
| Peak Data Rate | bytes/sec | 32-bit IEEE floating point number |
| Minimum Policed Unit | bytes | 32-bit integer |
| Maximum Packet Size | bytes | 32-bit integer |
| Latency | micro secs | 32-bit integer |
| Delay Variation | micro secs | 32-bit integer |
| Service Type |  | service type |

Table xxx – Flowspec parameters

As a minimum the following three Flowspec parameters shall be used in the MExE QoS functions

- Token Bucket Rate

- Token Bucket Size

- Peak Data Rate

If the MExE executable does not provide a full set of QoS FLOWSPEC parameters, then the MExE QoS manager shall the provide QoS parameters based on information available to it, see subclause 'Sources of UMTS Bearer Service Parameters'.

## 9.6	Sources of UMTS Bearer Service Parameters

A set of QoS parameters (QoS profile) specify the service provided to the user by the UMTS network.  At UMTS bearer service establishment or modification different QoS profiles have to be taken into account.  This is based on:-

- The UE capabilities

- The UE or the TE within the terminating network

- A QoS profile in the UMTS subscription (describes the upper limits)

- Default QoS profile (of the user or network)

- A Network specific QoS profile characterising for example the current resource availability or other network capabilities.

## 9.7	QoS Subscription

The QoS subscription is held within the network.  The subscription is used as the maximum attainable QoS for a session.  For security and the prevention of damage to the network, the user cannot directly modify the subscription data.

## 9.8	QoS Streams

Several applications may be executing in the MExE terminal, each with a different QoS requirement.  Also, one application may operate several QoS streams, each with different parameter settings.  The QoS manager within the MExE terminal should be able to deal with each stream independently.

## 9.9	QoS Security

Only the user or applications using a QoS stream should be able to modify the QoS level of that stream.

QoS privileges may be identified in the security table.

## 9.10 Change of Bearer

Subsequent to a change of bearer or intersystem handover the level of QoS previously negotiated shall be maintained, as close as possible to the original.