| 3GPP/SMG Meeting TSG-T2#7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | T2-991032 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ystad, SWEDEN, 22-26 Nov 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 23.057 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 1.5.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | BTCellnet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 22 November 99 |  |  |  |  |  |
| Subject: |  |  | Inclusion of QoS in MExE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | SWG 1 (MExE) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | MExE QoS requirements are not yet reflected in MExE stage 2 specification, this CR adds QoS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 2.1, 3.2, 4.12, 9. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.1	Normative references

[1]	GSM 01.04: “Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms”.

[2]	GSM 02.57: "Digital cellular telecommunications system (Phase 2+);MExE Stage 1 Description"

[3]	Personal Java 1.1.1, Sun Microsystems http://java.sun.com/products/personaljava/spec-1-1-1/index.html.

[4]	JavaPhone API version 0.9, http://java.sun.com/products/javaphone/.

[5]	JTAPI 1.2, Sun Microsystems http://www.java.sun.com.

[6]	Wireless Application Protocol (WAP) version 1.1 http://www.wapforum.org.

[7]	vCard – The Electronic Business Card Exchange Format – Version 2.1, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/vcard-21.doc.

[8]	vCalendar – The Electronic Calendaring and Scheduling Exchange Format – Version 1.0, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/

[9]	Hypertext Transfer Protocol – HTTP/1.1, IETF document RFC2068, http://www.w3.org/Protocols/rfc2068/rfc2068

[10]	Java Mail API version 1.0.2, http://www.java.sun.com

[11]	UMTS TR 22.170: “Universal Mobile Telecommunications System (UMTS); Service aspects; Provision of Services in UMTS - The Virtual Home Environment”.

[12]	UMTS TS 22.121: “Universal Mobile Telecommunications System (UMTS); Provision of Services in UMTS - The Virtual Home Environment: Stage 1”.

[13]	ISO 639 International Standard - codes for the representation of language names

[14]	UMTS TS 22.101: “Universal Mobile Telecommunications System (UMTS); Service Aspects; Service Principles”.

[15]	CC/PP Exchange Protocol based on HTTP Extension Framework; W3C http://www.w3.org/TR/NOTE-CCPPexchange

[16]	Composite Capability/Preference Profiles (CC/PP):A user side framework for content negotiation;  Available at W3C web pages.

[17]	UAProf  Specification http://www.wapforum.org

[18]	JDK 1.1 security http://www.javasoft.com/products/jdk/1.1/docs/guide/security/index.html

[19]	Java 2 security http://www.javasoft.com/products/jdk/1.2/docs/guide/security/index.html

[20]	Java security tutorial http://java.sun.com/docs/books/tutorial/security1.2/overview/index.html

[21]	OCF 1.1.: “Smartcard API specified by OpenCard Consortium http://www.opencard.org

[22]	RFC 1738 Uniform Resource Locators (URL) http://www.w3.org/pub/WWW/Addressing/rfc1738.txt

[23]	The MD5 Message Digest Algorithm", Rivest, R., RFC 1321, April 1992.  URL: ftp://ftp.isi.edu/in-notes/rfc1321.txt

[24]	ISO/IEC 10118-3 1996, Information technology - Security techniques - Hash-functions - Part 3: Dedicated hash-functions

[25]	IETF RFC 2368: The mailto URL scheme

[26]	ITU-T X.509: Information technology – Open Systems Interconnection – The Directory: Authentication framework

[27]	GSM 11.11: "Digital cellular telecommunications system (Phase 2+); Specification of the Subscriber Identity Module – Mobile Equipment (SIM-ME) interface.

[28]	3G TS 23.107: '3rd Generation Partnership Project; Technical Specification Group Services and system Aspects QoS Concept and Architecture (3G TS 23.107)'

[29]	3GPP TS 24.007: '3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile radio interface signalling layer 3; General Aspects (3G TS 24.007)'

[30]	3GPP TS 24.008; '3rd Generation Partnership Project; Universal Mobile Telecommunications System; Mobile radio interface layer 3 specification, Core Network Protocols – Stage 3 (TS 24.008)'

[31]	3GPP TS 23.060, '3rd Generation Partnership Project; Technical Specification Group Core Network; Digital cellular telecommunications system (Phase 2+); General Packet Radio Service (GPRS); Service Description; Stage 2 (3G TS 23.060)'

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of this TS the following definitions apply:

administrator: The administrator of the MExE MS is the entity which has the control of the third party trusted domain, and all resources associated with the domain. The administrator of the device could be the user, the operator, the service provider, or a third party as designated by the owner of the device.

certificate: An entity that contains the issuer's public key, identification of the issuer, identification of the signer, and possibly other relevant information. Also, a certificate contains a signed hash of the contents. The signer can be a 3rd. party other than the issuer.

fine grain: Refers to the capabilities of the Java security system to allow applications, sections of code or Java classes to be assigned permissions to perform a specific set of privileged operations.  The smallest programming element that can be given permission attributes is a Java class [19]

key pair: Key pairs are matching private and public keys. If a block of data is encrypted using the private key, the public key from the pair can be used to decrypt it. The private key is never divulged to any other party, but the public key is available, e.g. in a certificate.

personal certificate: This is a certificate loaded by the user or a user application which is limited to the application that it is intended for, and is not a MExE Certificate. E.g. an e-mail application could load certificates for its usage. Personal certificates are out of scope for MExE.

phonebook: A phonebook is a dataset of personal or entity attributes. The simplest form is a set of name-number pairs as supported by GSM SIMs.

MExE: MExE (Mobile station application Execution Environment) is defined in detail in this document, but the scope of MExE does not include the operating system, or the manufacturer’s execution environment.

MExE certificate: This is a certificate used in the realisation of  MExE security domains. A MExE Certificate can be used to verify downloaded MExE executables. Use of the word "certificate" in this document implies a MExE certificate. Other varieties of certificate will be explicitly qualified as a e.g. "Personal Certificate".

MExE executable: An executable is an applet, application, or executable content, which conforms to the MExE specification and may execute on the ME.

MExE Java VM: This is a standard Java virtual machine used to execute MExE Java applets and applications

MExE native library: This is a downloaded native library that can be accessed by MExE executables

MExE-SIM: A SIM that is capable of storing a security certificate that is accessible using standard mechanisms..

owner: An owner of the MExE MS.  An owner could be a user, operator (e.g. where the MS is obtained as part of a subscription and the cost of the MS is subsidised), service provider, or a third party (e.g. the MS is owned by the user’s company and this company wishes to control how the MS is used)

power up event: An abstract event that occurs when the MExE MS is cold started (i.e. switched on).

quality of service (QoS) profile: A QoS profile is associated with each session created.  The QoS profile defines the performance expectations placed on the bearer network.  Multiple QoS sessions may exist, each with a different QoS profile.  A QoS profile comprises a number of parameters, a PLMN might support only a limited subset of the possible QoS profiles.

subscribed QoS: The QoS profile subscription parameters are held in the HLR [31].  An end user may have several QoS subscriptions.  The network will not grant a QoS greater than that subscribed.  For security and the prevention of damage to the network, the end user cannot directly modify the QoS subscription profile data.

requested QoS: A QoS profile is requested at the beginning of a session.  QoS modification requests are also possible during the lifetime of a session.

negotiated QoS: In response to a QoS request, the network shall negotiate each QoS attribute to a level that is in accordance with the available network resources.  After QoS negotiation, the bearer network shall always attempt to provide adequate resources to support all of the negotiated QoS profiles.

delivered QoS: The measured QoS over the lifetime of the session.

best effort QoS: The bearer network delivers best effort QoS where it cannot or will not deliver any guaranteed QoS.  The best effort QoS is the lowest of all QoS levels.

sandbox: A sandbox is a safe area to run Java code. Untrusted Java code executing in a sandbox has access to only certain resources [18]

service: A service (which may consist of an application or applet, and its related content) is a set of functions offered to a user by an organisation, and may be performed on the MExE MS and/or remotely

service name: An identifier associated with a service, which could be a string, a fully qualified Java class name, a unique URI or other identifier.

session: The period between the launching of a MExE executable and its execution termination. A WAP-session is established between the mobile and the WAP Gateway. The duration of a WAP-session can range from a second to years. The WAP-session can be associated with a particular subscription in the WAP Gateway.

signature: "Signing" is the process of encrypting a hash of the data using a private key. If the signature can be decrypted using the public key, then the signature is valid.

signed JAR file: Archives of Java classes or data that contain signatures that also include a way to identify the signer in the manifest. (The Manifest contains a file which has attributes defined in it.)

user: The user of the MExE MS

Further definitions specific to MExE are in GSM given in 02.57 (MExE stage 1) [2].

## 3.2	Abbreviations

For the purposes of this TS the following abbreviations apply:

API	Application Programming Interface

APDU	Application protocol data unit

CA	Certification Authority

CC/PP	Composite Capability/Preference Profiles

CGI	Common Gateway Interface

CCM	Certificate Configuration Message

CP-Admin	Certificate Present (in the MExE SIM) - Administrator

CP-TP	Certificate Present (in the MExE SIM) - Third Party

Diff-serv	Differentiated Services

DHCP	Dynamic Host Configuration Protocol

GSM	Global System for Mobile Communication

GPRS	General Packet Radio Service

HTTP	HyperText Transfer Protocol

HTTPS	HyperText Transport Protocol Secure (https is http/1.1 over SSL, i.e. port 443)

IETF	Internet Engineering Task Force

IP	Internet Protocol

JNDI	Java Naming Directory Interface

JTAPI	Java Telephony Application Programming Interface

JAR file	Java Archive File

MMI	Man-Machine Interface

MSE	MExE Service Environment

OCF	OpenCard Framework

QoS	Quality of Service

RDF	Resource Description Format

RFC	Request For Comments

SAP	Service Access Point

SMS	Short Message Service

TLS	Transport Layer Security

TP	Third Party

UDP	User Datagram Protocol

UE	User Equipment

UI	User Interface

UMTS	Universal Mobile Telecommunications System

URL	Uniform Resource Locator

URI	Uniform Resource Identifier

USSD	Unstructured Supplementary Service Data

WAE	Wireless Application Environment

WAP	Wireless Application Protocol

WDP	Wireless Datagram Protocol

WSP	Wireless Session Protocol

WTA	Wireless Telephony Applications

WTAI	Wireless Telephony Applications Interface

WTLS	Wireless Transport Layer Security

WTP	Wireless Transaction Protocol

WWW	World Wide Web

# 4	Generic MExE aspects

This section defines the common aspects of all MExE compliant devices, independent of MExE technology.

### 4.12		Quality of Service

### Quality of Service (QoS) [28] is seen by the end user as a measure of the amount of network resources given to an application by the underlying network.  The network may employ a number of QoS mechanisms, but the end user /  MExE executable is not involved in these.  The end user /  MExE executable requires an interface into the network QoS through a visible set of standard parameters.

A QoS aware  MExE executable may request a QoS from the network at the beginning of a QoS session. Changes in the level of QoS provided shall be notified to the end user / MExE executable.  An end user may request a change in the QoS through the MExE MS MMI.  A MExE executable  may have several QoS streams open simultaneously.

The MExE executable shall be able to dynamically request a change in the level of QoS at connection setup request or subsequently during the connection.  The  end user / MExE executable may receive a rejection to a QoS modification request, upon which the end user / MExE executable must be notified.

The end user's service level QoS subscription parameters are stored in the network, they identify the maximum permissible QoS that a user may negotiate with the network.  Several QoS subscriptions may be possible for one user.  MExE is neither aware nor able to determine or modify the end user's service level QoS subscriptions.

For MExE devices supporting bearers defined by QoS, the MExE execution environment shall support QoS management.  QoS management may be available directly to the MExE executables themselves, or to the MExE environment.

# 9	Quality of Service

QoS aware MExE executables may be executing on the MExE device.  To ensure correct operation with the QoS provisioning of the bearer network(s) the associated API's and the MExE QoS manager shall be supported by MExE MS supporting bearers defined by QoS– see figure xxx.  Non QoS aware MExE executables shall operate with the defined QoS by the user or the network.

Figure xxx – Logical MExE Terminal QoS manager elements

## 9.1	MExE QoS Support

A MExE QoS manager exists between the MExE executable and the Network Control API.  To interface this, an API to the MExE executable is provided and another API to the network, see figure xxx.  The MExE QoS functions accommodates standard methods of end to end QoS provisioning – e.g. differentiated services (Diff-serv).

For MExE devices supporting bearers defined by QoS, the MExE device shall  support the following basic QoS operations :-

- a mapping between the QoS requirements of the MExE executable and the network layer

- MExE executables shall be able to indicate and interpret QoS values of the network

- MExE executables shall be able to modify the QoS dynamically

- MExE executables shall be able to react to changes in the provided QoS

- The end user shall be able to manage the QoS directly via the MMI.

MExE introduces two new elements to cater for QoS – the MExE QoS manager and the QoS API.  The MExE QoS manager shall handle the fact that the network may not have QoS capabilities.

## 9.2	MExE QoS Manager

The MExE QoS manager is responsible for:

- Managing the QoS streams for MExE executables

- Notification of the negotiated and delivered QoS to the end user / MExE executable

The MExE QoS manager shall support the MExE QoS API according to the bearer supported by the device, and provide functions such as:-

- insert additional QoS signalling parameters (e.g. Diff-serv)

- add the functionality of the MExE QoS API at best effort, if the network does not support it directly

- translate between the QoS parameters from the MExE executable and those of the network

- monitor the QoS delivered by the network and manage QoS requests between the MExE executable and the network

- be informed of the requested QoS and minimum QoS for each MExE executable

- attempt to re-negotiate the QoS if it falls below a threshold.

The MExE QoS manager may request information from the network regarding the QoS available.

The MExE QoS manager does not need to know the end user's subscribed QoS, this is held within the network and used to validate a requested QoS level.

The MExE QoS manager may also be  accessed through the device's MMI.

## 9.3	Network Control API

The network control API shall provide the QoS manager with access to the network specific QoS control (defined for GPRS/UMTS in [29] and [30].

The MExE QoS manager may perform some QoS control, if it is not provided in the network control.

## 9.4	QoS API

The QoS API provides the MExE executable with an interface to the QoS management.  It does not require the MExE executable to have any knowledge of the underlying network, or how QoS is implemented in the network.

The QoS API shall provide the MExE executable with a standard set of parameters .  Refer to [28] for details of these parameters.

Table xxx shows the set of example parameters.

| Parameter | Units | Type |
| --- | --- | --- |
| Token Bucket Rate | bytes /sec | 32-bit IEEE floating point number |
| Token Bucket Size | bytes | 32-bit IEEE floating point number |
| Peak Data Rate | bytes/sec | 32-bit IEEE floating point number |
| Minimum Policed Unit | bytes | 32-bit integer |
| Maximum Packet Size | bytes | 32-bit integer |
| Latency | micro secs | 32-bit integer |
| Delay Variation | micro secs | 32-bit integer |
| Service Type |  | service type |

Table xxx – Example parameters

As a minimum the following three parameters shall be supported by the MExE QoS manager:-

- Token Bucket Rate

- Token Bucket Size

- Peak Data Rate

If the MExE executable does not provide a full set of QoS parameters, then the MExE QoS manager shall the provide QoS parameters based on information available to it (e.g. from the MMI settings), see subclause 'Sources of UMTS Bearer Service Parameters'.

## 9.5	Sources of Bearer Service Parameters

A set of QoS parameters (QoS profile) specify the service provided to the user by the network.  At bearer service establishment or modification different QoS profiles have to be taken into account.  This is based on:-

- The UE capabilities

- The UE or the TE within the terminating network

- A QoS profile in the QoS subscription (describes the upper limits)

- Default QoS profile (of the user or network)

- A Network specific QoS profile characterising for example the current resource availability or other network capabilities.

## 9.6	QoS Subscription

The QoS subscription is held within the network.  The subscription is used as the maximum attainable QoS for a session.  For security and the prevention of damage to the network, the user cannot directly modify the subscription data.

## 9.7	QoS Streams

Several MExE executables may be executing in the MExE device, each with a different QoS requirement.  Also, a MExE executable may operate several QoS streams, each with different parameter settings.  The MExE QoS manager within the MExE device shall be able to deal with each stream independently.

## 9.8	QoS Security

Only the end user, MExE executable or the network using a QoS stream should be able to modify the QoS of that stream.

The FLOWSPEC parameters, defined by the IETF Integrated Services Working Group, provide the QoS information required by QoS capable network elements.