| 3GPP TSG-T3 API SWG meeting #14 Tdoc T3-020532 Paris, France, 17-19 Sept. 2002 | Tdoc T3a020324 |
| --- | --- |
| 3GPP TSG-T3 meeting #25 Tdoc T3-020532 Maastricht, Netherlands, 5-8 Nov. 2002 | Tdoc T3-020xxx |

Title:	Discussion document on

Source:	T3 API SWG / EP SCP WG3

Document:	For discussion

Contact Person:

#### Name:	Paul JOLIVET	Sebastian HANS

E-mail:	jolivet@docomo.fr

Attachements:	None

Following the discussion raised during last T3 plenary meeting by T3 API, Gemplus and G&D about portability of 2G Java Card applets on a 3G platform, a joint session was organised between T3 API SWG and EP SCP WG3.

The current situation regarding the API on the card is as follows:

- SIM API is standardised to run a SAT application on a 2G environment (mono-first level application card)

- There is a requirement for a UICC API specification to run SAT/USAT application on a 2G/3G environment (multiple first level application card).

This introduces the following cases :

- the SIM API is not present on the UICC : an applet using the SIM API would then have to be modified/re-compiled in order to be loaded and executed on the UICC

- the SIM API and the UICC API are present on the UICC : an applet using the SIM API can then be loaded without any modification on the UICC (binary compatibility) but the interworking of the two APIs have to be specified (e.g. should the menus registered by a SIM API applet be displayed when a USIM application is active, should a SIM API applet be triggered when a USIM application is active, etc.)

It is foreseen that a possible way forward is

The different packages are expecting to be :

| SIM API (Rel-5, Rel-6 ?) |  |
| --- | --- |
| sim.access |  |
| sim.toolkit |  |
| UICC API (Rel-6) |  |
| uicc.access |  |
| uicc.toolkit |  |
| uicc.sim.access | SIM specific constants |
| uicc.sim.toolkit |  |
| uicc.usim.access | USIM specific constants |
| uicc.usim.toolkit |  |