TSG-Terminals Working Group 3 (USIM) meeting #4	TSGT3#4(99)129

Tokyo 19th - 21th April, 1999	(replaces T3-99115)

Source:	Rapporteur of USIM characteristics (Nippon Ericsson)

Title:	Modified Proposal for USIM version management

___________________________________________________________________________

1.	Introduction

This document presents the modified proposal for the EF for the USIM version management agreed in T3 plenary on April 19, 1999. The original proposal is in TSGT3#4(99)114. It was agreed to change the name of EF to ‘Application profile’.

2.	Modified proposal

Modified proposal is as follows.

It should be noted that EF is put under each DFUSIM.

- EFAPPI (Application profile indication)

This EF contains an indication concerning the application (USIM) profile.

| Identifier: 'xxxx' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: X byte |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Application profile |  |  | M |  | 1 byte |
| 2-X | RFU |  |  | O |  | X-1 byte |

-	Application profile

Coding:

'00' : The USIM based on the version of the present document approved by TSG-T in April 1999.

All other codings are reserved for specification by 3GPP.