| 3GPP TSG-T3 Miami, June, 14th to 16th, 1999 | Document T3-99178 Page 1 of 6 |
| --- | --- |

Source: T-Mobil

Title: Proposed text on authentication command for 33.102

## 3.3	Symbols

||	Concatenation

Å	Exclusive or

f1	Message authentication function used to compute MAC

f2	Message authentication function used to compute RES and XRES

f3	Key generating function used to compute CK

f4	Key generating function used to compute IK

## 3.4	Abbreviations

Clause numbering depends on applicability. Abbreviations should be ordered alphabetically.

AK	Anonymity key

AUTN	Authentication token

CK	Cipher key

CS	Circuit switched

HE	Home Environment

IK	Integrity key

K	USIM Individual key

KSI	Key Set Identifier

MAC	Message Authentication Code

MAC	The message authentication code included in AUTN, computed using f1

RAND	Random challenge

SQN	Sequence number

XRES	Expected response

# X	USIM commands

## X.1	AUTHENTICATE

### X.1.1	Command description

This function fulfils two purposes:

- authentication,

- resynchronisation.

#### X.1.1.1 Authentication

The function is used during the procedure for authenticating the USIM to its HE and vice versa. In addition, a cipher key and an integrity key are calculated. For the execution of the command the card uses the subscriber authentication key K, which is stored in the USIM.

The USIM first computes the anonymity key AK = f5K (RAND) and retrieves the sequence number SQN = (SQN Å AK) Å AK.

Next the USIM computes XMAC = f1K (SQN || RAND || MODE) and compares this with MAC which is included in AUTN. If they are different, the USIM aborts the function.

Next the USIM verifies the freshness of the received sequence number SQN: For each mode the USIM keeps track of one internal counter: SQNMS/CS for authentications initiated by the circuit switched core network nodes, and SQNMS/PS for authentications initiated by the packet switched core network nodes. To verify the freshness of the sequence number SQN, the USIM compares SQN (calculated as decribed above) with SQNMS/MODE. If SQN > SQNMS/MODE the sequence number is considered as fresh and the USIM subsequently sets SQNMS/MODE  to SQN.

If the USIM considers the sequence numbers not fresh, the USIM aborts the function.

If the sequence number is considered fresh however, the USIM computes RES = f2K (RAND) and includes this parameter in the command response. Finally the USIM computes the cipher key CK = f3K (RAND) and the integrity key IK = f4K (RAND). Note that if this is more efficient, RES, CK and IK could also be computed earlier at any time after receiving RAND.

The function is related to a particular USIM and shall not be executable unless the USIM or any sub-directory  has been selected as the Current Directory and a successful CHV1 verification procedure has been performed (see 11.3.1).

Input:

-	RAND, AUTN (AUTN := SQN Å AK || MODE || MAC).

Output:

-	RES, CK, IK.

#### X.1.1.1 Resynchronisation

The function allows for resynchronisation of the SQNMS/CS and  SQNMS/PS counters in the HLR/AuC. The function returns a RAND and AUTN pair where SQN the value of the SQNMS/CS or SQNMS/PS counters stored in the SIM, depending on the indicated mode. AUTN is computed as AUTN := SQN Å AK || MODE || MAC.

NOTE: The USIM has several ways to produce RAND || AUTN. Either it stores and returns the latest received RAND || AUTN pair, or it only stores the received RAND and re-computes AUTN, or it generates a RAND and computes the corresponding AUTN.

Input:

-	None.

Output:

-	RAND, AUTN (AUTN := SQN Å AK || MODE || MAC).

### 12.1.2	Command parameters and data

| Code | Value |
| --- | --- |
| CLA | As specified in ... |
| INS | '88' |
| P1 | '01' |
| P2 | See table ... |
| Lc | See below |
| Data | See below |
| Le | See below |

Parameter P2 specifies the mode and whether authentication or resynchronisation has to be performed:

-	'01' = Authentication, circuit switched;

-	'02' = Authentication, packet switched;

-	'11' = Resynchronisation, circuit switched;

-	'12' = Resynchronisation, packet switched.

Case 1: P2='01' or '02':

Command parameters/data:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | Length of RAND (L1) | 1 |
| 2 – (L1+1) | RAND | L1 |
| (L1+2) | Length of AUTN (L2) | 1 |
| (L1+3) – (L1+L2+2) | AUTN | L2 |

Response parameters/data:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | Length of RES (L3) | 1 |
| 2 – (L3+1) | RES | L3 |
| (L3+2) | Length of CK (L4) | 1 |
| (L3+3) – (L3+L4+2) | CK | L4 |
| (L3+L4+3) | Length of IK (L5) | 1 |
| (L3+L4+4) – (L3+L4+L5+3) | IK | L5 |

The most significant bit of RES is coded on bit 8 of byte 2. The most significant bit of CK is coded on bit 8 of byte (L3+3). The most significant bit of IK is coded on bit 8 of byte (L3+L4+4).

Case 2: P2='11' or '12':

Response parameters/data:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | Length of RAND (L1) | 1 |
| 2 – (L1+1) | RAND | L1 |
| (L1+2) | Length of AUTN (L2) | 1 |
| (L1+3) – (L1+L2+2) | AUTN | L2 |

## Y	Status Conditions Returned by the UICC

Status of the card after processing of the command is coded in the status bytes SW1 and SW2. This subclause specifies coding of the status bytes in the following tables.

### Y.1	Security management

| SW1 | SW2 | Error description |
| --- | --- | --- |
| '98' | '62' | - Authentication failure, incorrect MAC |
| '98' | '64' | - Authentication failure, sequence number error |

### Y.2	Status Words of the Commands

The following table shows for each command the possible status conditions returned (marked by an asterisk *). Status conditions of GSM and USIM applications are on the left and right sides of the table, respectively.

Table 12.9: Commands and status words

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AUTHENTICATE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 91 XX |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 9F XX 61 XX# |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 93 00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 92 0X |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 65 81 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 94 00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 94 02 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 94 04 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 94 08 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 98 02 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 69 82 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 98 08 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 98 10 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 98 40 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 98 50 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 98 62 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 98 64 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 67 XX |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 6B XX |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6D XX |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 6E XX |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 6F XX |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 62 81 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 62 83 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 62 82 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 62 84 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 62 00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 63 CX |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 69 81 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 69 84 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 69 85 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 69 86 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6A 81 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6A 82 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6A 83 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6A 84 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6A 85 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 6A 86 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6A 87 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | * |  |  |  |  |  |  | 6A 88 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6C XX |

# Z	Application Protocol

### Z.1	Authentication

The ME selects a USIM and uses the Authenticate function (see 8.16) in authentication mode. The response  is sent to the ME (in case of the T=0 protocol when requested by a subsequent GET RESPONSE command).

### Z.2	Resynchronisation

The ME selects a USIM and uses the Autnenticate function (see 8.xx) in resynchronisation mode. The response  is sent to the ME (in case of the T=0 protocol when requested by a subsequent GET RESPONSE command).