3GPP TSG-T3		Document T3-99181

Miami, June, 14th to 16th, 1999

Source: T3

Phone Book Implementation

# Introduction

This paper is a working document within TSG-T3 to be elaborated until the next meeting (T3#7) in Lund. This paper shall be a basis for the decision about the phone book implementation.

Currently three (3) proposals are under discussion:

- T3-99152	Proposal for Implementation of ‘Phone Book’ in USIM (Nokia)

- T3-99167	Phone book management with ISO/IEC 7816-7	 (Gemplus)

- T3-99171	Discussion document on phone book	 (Schlumberger)

The proposals from Nokia and Schlumberger are based on in index file structure, the Gemplus proposal is based on a subset of a SCQL (ISO/IEC 7816 part7) database.

The owners of the proposals are kindly requested to elaborate the issues listed below in this document.

Of course any other source for inputs is welcome.

As the document comprising all inputs is to be distributed to TSG-T3 members on June 29 1999, the
deadline for the input delivery is June 25.

All inputs are to be sent to the 31.102 rapporteur Mr. Kobayashi
(e-mail: makoto.kobayashi2@nrj.ericsson.se)

# Technical Evaluation of the proposals

The following items were identified for evaluation. A summary of the results shall be the table in Annex 1.

- Card Memory

1.1	Memory Optimization
E.g. in case that less second names than Phone Book entries are required, is it possible to allocate only the memory which is actually used.

1.2	Memory Consumption for Phone Book Data

What is the amount of memory occupied by the Phone Book for the following examples:

- Case 1: xxxx ADN, yyy second name, zzz groupings, ... TBD
- Case 2: xxxx ADN, yyy second name, zzz groupings, ... TBD

1.3	Memory Consumption for Phone Book Code (e.g. Operating System, SCQL engine,..)

- Access to data linked to entries in the ‘Incoming / Outgoing Call’ list

E.g. is a direct link to the according Phone Book entry stored in this list. Or has e.g. a search operation on the Phone Book to be performed (always unambigous?).

- Adding of new features / elements to the Phone Book

- Complexity of the implementation (card and mobile)

- Access time to Phone Book entries

- Error Recovery

- Backward Compatibility

# General ISSUES

General aspects, comments, concerns should be summarized to get the whole picture of implications, such as e.g.

- IPR issues

- Schedule for Phase 1

- not being ready for future evolutions not been foreseen today (too reluctant when introducing new functionality)

- ....

# ANNEX 1

|  | NOKIA Proposal | GEMPLUS Proposal | SCHLUMBERGER Proposal |
| --- | --- | --- | --- |
| 1.1 |  |  |  |
| 1.2 |  |  |  |
| 1.3 |  |  |  |
| 2 |  |  |  |
| ... |  |  |  |