3GPP TSG-T3 (USIM) #11	Tdoc T3-99408

8- 10 December 1999

Sofia Antipolis, France

# Implementation of synchronisation functionality

Source: Ericsson / Schlumberger

### PID

The phonebook identifier is used to determine whether the accessed phonebook is the same as the previously accessed phonebook or if it is a new unknown phonebook (might be the case that there is one phonebook under DF-telecom and one phonebook residing in an USIM-application). If the PID is unknown, a full synchronization of the phonebook will follow.

The PID is also used to regenerate the UID’s and reset the CC to prevent them from running out of range. When the UID´s or the CC has reached its maximum value, a new PID is generated. This leads to a scenario where neither the CC nor the UID’s will run out of range and 2 bytes/record are enough.

### UID

A UID for each record is used to uniquely identify a record and to be able to keep track of the records in the phonebook. The UID value is assigned by the terminal when a new entry is created in the phonebook. The UID shall remain on the UICC, in EF-UID, until the PID is regenerated. This means that when a record is deleted from the phonebook, the content of the linked information (eg ADN, E-MAIL,..) are set to the personalisation value 'FF…FF'. But the UID-value of the deleted record (value of ADN record is 'FF…FF') shall not be used when a new record is added to the phonebook until the PID is regenerated, but it shall be set to a new value.

2 bytes shall be enough to give each record a unique identifier and the values of the UID can never run out of range when they are used together with the concept of a PID, as described above.

### CC

The change counter (CC) is placed on the UICC and shall be used to detect changes made in the phonebook in the UICC. The concept of having a CC makes it possible to update the phonebook on the UICC in different terminals but still be able detect the changes (changes between different manufacturers and/or 2:nd and 3:rd generation of terminals also possible). The length of the change-counter must be 2 bytes i.e. the CC shall be able to have larger values than number of records in the phonebook.
The value of the change counter is updated by the terminal every time a record in the phonebook is changed.

### Initialisation and update mechanism for the PID

#### EFPID (Phone Book Identifier)

The phone book identifier (PID) is used by the terminal to verify if the currently used phone book is the same as the one used in the previous session.

The PID is generated and given an initial value during the personalisation stage according to:

- For a phonebook residing in DF-telecom:

PID = ICCid (10bytes) + 4 bytes (in EF-PID-V)

Fixed Part PIDF 	Variable Part PIDV

- For a phonebook residing in an USIM application:

PID= (ICCid XOR AID)(10 last bytes of this operation) + 4 bytes

Fixed Part PIDF		Variable Part PIDV

The PID shall be regenerated by the terminal if one of the following situations applies:

- the values of the UIDs have run out of range

- the whole phone book has been reset/deleted

- the value of the CC has run out of range

Table 10a: Structure of  EFPID-V

| Identifier: '4FXX' |  | Structure: transparent |  |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SFI: optional |  |  |  |  |  |  |  |
| File size: 4 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN1 UPDATE PIN1 DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |  |
| Bytes | Description |  |  |  | M/O |  | Length |
| 1 to 4 | Phone Book Identifier (PID) PIDV |  |  |  | M |  | 4 bytes |

-	PID: Unique Identifier of Phone Book

Content: Number to unambiguously identify the phone book for synchronisation purposes

To be able to detect if the PID needs to be regenerated (i.e. the PIDV shall be changed) the following test must be made by the terminal before for each update of either the CC or the assignment of a new UID:

Each time the terminal has to increment the value of the UID the following test is needed:

If UID =FFFF then

{Increment PIDV mod FFFFFFFF ;

Regenerate new UIDs “ the terminal reads all the records of the phonebook anyway”

}

Each time the terminal has to increment the value of CC the following test is needed:

If CC =FFFF then

{Increment PIDV mod FFFFFFFF ;

CC=0001

}

If the phonebook is deleted then will the terminal change the PIDV according to:

Incrementing the 4bytes modulus FFFFFFFF

### Initialisation and update mechanism for UID

All UID´s will be set to 0000 Hex (e.g. empty) during the personalisation stage.

##### EFUID (Unique Identifier)

For each phone book entry a unique identifier (UID) is assigned by the terminal when the new entry is created. This value unambiguously identifies the phone book record. The value of the UID does not change as long as the value of the PID remains the same. If/when the PID is regenerated, all UID´s for the records in the phonebook shall be assigned new values starting from 1. The new value of the UID for each record shall then be kept until the PID is regenerated again.

Table 10: Structure of  EFUID

| Identifier: '4F21' |  | Structure: linear fixed |  |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SFI: optional |  |  |  |  |  |  |  |
| Record length: 2 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN1 UPDATE PIN1 DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |  |
| Bytes | Description |  |  |  | M/O |  | Length |
| 1 to 2 | Unique Identifier (UID) of Phone Book Entry |  |  |  | M |  | 2 bytes |

-	Unique Identifier of Phone Book Entry

Content: Number to unambiguously identify the phone book entry for synchronisation purposes

Initialisation and update mechanism for CC

The CC will be set to 0000 Hex (e.g. empty) during the personalisation stage.

#### EFCC (Change Counter)

The change counter (CC) is used to indicate recent changes made to phone book entries.

Every update/deletion of an existing phone book entry or addition of a new phone book entry causes the terminal to increment the CC.

Table 10b: Structure of  EFCC

| Identifier: '4FXX' |  | Structure: transparent |  |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SFI: Mandatory |  |  |  |  |  |  |  |
| File size: 2 bytes |  |  |  | Update activity: high |  |  |  |
| Access Conditions: READ PIN1 UPDATE PIN1 DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |  |
| Bytes | Description |  |  |  | M/O |  | Length |
| 1 to 2 | Change Counter (CC) of Phone Book |  |  |  | M |  | 2 bytes |

-	Change Counter of Phone Book

Content: Indicates recent change(s) to phone book entries for synchronisation purposes

The terminal updates the value of the CC for each change/update made to the phone book and the card can not differ between a deletion/an update of a record.

When the 3G UICC has been inserted into a 2G terminal and a record in the phone book has been updated, a flag in the entry control information in the EFPBC is set from 0 to 1 by the card. When/if the UICC later is inserted into a 3G terminal again, the terminal shall check the flag in EFPBC and if this flag is set update the CC. A set flag in EFPBC results in a full synchronisation of the phone book (if synchronisation is requested).