| 3GPP T3 Meeting #15 San Diego, USA, 16 - 18 August, 2000 | Tdoc T3-00 0330 |
| --- | --- |

TSG-RAN Working Group 2 (Radio L2 and Radio L3) 		R2-001541

Paris, France, 3 - 7 July 2000

Source:	TSG-RAN WG2

To:		TSG-T WG3

Cc:		TSG-SA WG3

Title: 		Response to LS (T3-99304) on Parameters to be stored in the USIM

Contact:	Marko Mahkonen, Nokia

Email: Marko.Mahkonen@nokia.com

Tel.: +358 40 768 2650

TSG-RAN WG2 would like to thank TSG-T WG3 for requesting information about network related parameters to be stored in the USIM.

For the time being, TSG-RAN WG2 has identified the following information that is relevant for the Access Stratum to store in the USIM. This information can be found in future as an Annex in the TS 25.331.

### Ciphering information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cipher key for each CN domain | MP | <1 to maxCNDomains> |  | Cipher key is described in 33.102. |
| > Old CK | MP |  | Bitstring (128) |  |
| > New CK | MP |  | Bitstring (128) |  |
| Integrity key for each CN domain | MP | <1 to maxCNDomains> |  | Integrity key is described in 33.102. |
| > Old IK | MP |  | Bitstring (128) |  |
| > New IK | MP |  | Bitstring (128) |  |
| START value for each CN domain | MP | <1 to maxCNDomains> |  | START value is described in 33.102. |
| > Old START | MP |  | Bitstring (20) |  |
| > New START | MP |  | Bitstring (20) |  |
| KSI, Key set identifier for each CN domain | MP | <1 to maxCNDomains> |  | Key set identifier is described in 33.102. |
| > Old KSI | MP |  | Bitstring (3) |  |
| > New KSI | MP |  | Bitstring (3) |  |

### Frequency information

#### Neighbour cell list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FDD cell list | OP | <1 to maxFDDFreqList> |  |  |
| >UARFCN uplink (Nu) | MP |  | Integer(0..16383) | [25.101] |
| >UARFCN downlink (Nd) | OP |  | Integer(0 .. 16383) | [25.101] If IE not present, default duplex distance of 190 MHz shall be used. |
| > Primary scrambling code | MP | <1 to maxFDDFreqCellList> | Primary CPICH info 10.3.6.51 |  |
| TDD cell list | OP | <1 to maxTDDFreqList> |  |  |
| >UARFCN (Nt) | MP |  | Integer(0 .. 16383) | [25.102] |
| > Cell parameters ID | MP | <1 to maxTDDFreqCellList> | Integer (0..127) | The Cell parameters ID is described in 25.223. |
| GSM Neighbour cell list | OP |  |  |  |
| >GSM neighbour cell info | MP | <1 to maxGSMCellList> |  |  |
| >> BSIC | MP |  |  |  |
| >> BCCH ARFCN | MP |  |  |  |

### Multiplicity values and type constraint values

| Constant | Explanation | Value |
| --- | --- | --- |
| Ciphering information |  |  |
| maxCNDomains | Maximum number of CN domains | 4 |
| Frequency information |  |  |
| maxFDDFreqList | Maximum number of FDD carrier frequencies to be stored in USIM | 4 |
| maxTDDFreqList | Maximum number of TDD carrier frequencies to be stored in USIM | 4 |
| maxFDDFreqCellList | Maximum number of neighbouring FDD cells on one carrier to be stored in USIM | 8 |
| maxTDDFreqCellList | Maximum number of neighbouring TDD cells on one carrier to be stored in USIM | 8 |
| maxGSMCellList | Maximum number of GSM cells to be stored in USIM | 8 |