TSG-T3 (USIM) meeting #4	Tdoc T3-99109

17-19 th of April, Tokyo

Japan

# Proposal for Implementation of "Phone Book" in USIM

Source: Nokia

Based on the discussions during previous TSG-T3 meetings the following structure is proposed as an extension to the existing ADN data field in the GSM SIM. This new feature in the USIM is called Phone Book. The new structure is compatible with the ADN data field in the GSM SIM. The second generation terminal is only capable of reading the original ADN data field, assuming that the EF identifier is the same. In addition the ADN feature is extended to cater for more than 254 entries, which means that the phone book contains two sets of ADN files with associated additional EFs. Each of the ADN files is associated with an index administration file. The index administration file contains an equal number of records as the AND file and the AND file and the index administration file are linked together based on the record identifier.

In order to support the additional information required the index file concept is introduced. This concept allows data related to one record in the original ADN data field to be extended. In order to save memory in the UICC pointers are used to indicate the location of a record in a file that is part of the Phone Book entry. The pointers are stored in the Index administration file. The content of a sample index file is shown Table 1.

Table 1. Index File Structure

| Identifier: 'XXXX' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record Length: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ Original EF UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 - 2 | EF Identifier of Index Administration EF |  |  | M |  | 2 bytes |
| 2 - 4 | EF Identifier of master EF |  |  | M |  | 2 bytes |
| 4 – 6 | EF Identifier of first Extension EF |  |  | M |  | 2 bytes |
| 6 - X | EF Identifier of additional Extension EF |  |  | O |  | 2 bytes |

The administration index file is located at the same level as the other EFs that is part of the Phone Book. The EF identifier of the Index file can be retrieved by selecting one of the EFs belonging to the Phone Book and perform a STATUS command. In the status response the EF identifier of the Index file is indicated as shown in table 2.

Table 2 Response parameters/data in case of an EF:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 - 2 | RFU | 2 |
| 3 - 4 | File size (for transparent EF: the length of the body part of the EF) (for linear fixed or cyclic EF: record length multiplied by the number of records of the EF) | 2 |
| 5 - 6 | File ID | 2 |
| 7 | Type of file (see 9.3) | 1 |
| 8 | see detail 3 | 1 |
| 9 - 11 | Access conditions (see 9.3) | 3 |
| 12 | File status (see 9.3) | 1 |
| 13 | Length of the following data (byte 14 to the end) | 1 |
| 14 | Structure of EF (see 9.3) | 1 |
| 15 | Length of a record (see detail 4) | 1 |
| 16-17 | Index file ID if an Index file is attached to the selected EF | 2 |
| 18 and following | RFU | - |

# Public Phone Book EF structure

The EF structure related to the public phone book is located under DFTELECOM. A USIM specific phone book may exist for application specific entries. The application specific phoine book is protected by the application CHV1. The application specific phone book is a copy of the file structure of that specified for the public phone book under DFTELECOM.

## Phone Book Index file EFPBI

The phone book structure is presented in the phone book index file. The index file contains one record for each EFADN. The amount of bytes in each record is dendent upon what EFs are associated with the phone book. If the phone book contains more than 254 ADN records two or more EFADN exists. The index file structure is as shown in table 3. The phone book may contain all or part of the optional files. The information provided by the optional files are defined by their IDs. This means that the ME can construct the phone book out of the information provided in the index file record even if not all optional files are provided by a phone book.

Table 3 Phone Book Index file EFPBI structure

| Identifier: '6FXX' |  | Structure: linear fixed |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record Length: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ EFADN UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 - 2 | EF Identifier of Index Administration EF (EFIAP) |  |  | M |  | 2 bytes |
| 2 - 4 | EF Identifier of master EF (EFADN) |  |  | M |  | 2 bytes |
| 4 - 6 | EF Identifier of first Extension EF (EFEXT1) |  |  | M |  | 2 bytes |
| 6 - 8 | EF Identifier of Control EF (EFPBC) |  |  | M |  | 2 bytes |
| 8 - 10 | EF Identifier Additional Number (EFANR) |  |  | O |  | 2 bytes |
| 10 - 12 | EF Identifier Second Name (EFSNE) |  |  | O |  | 2 bytes |

## Index Administration file EFIAP

The EF contains the pointers to the different records in the files that are part of the phone book. Each index administration EF record indicates the record locations for one phone book entry. The structure of EFIAP is shown in table 4. The index administration file contains the same amount of records as EFADN.

The value 'FF' is an invalid record number and is used  in any location in the index adminstration file record to indicate that no corresponding record in the indicated file is available e.g. if no additional number information is stored for the phone book entry byte 4 in the index administration file is set to 'FF'.

The content of the index administartion file is set to 'FF' at the personalisation stage.

Table 4 Index administration file EFIAP structure

| Identifier: '6FXX' |  | Structure: linear fixed |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record Length: 5 bytes |  |  | Update activity: high |  |  |  |
| Access Conditions: READ EFADN UPDATE EFADN INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Pointer to record in EFADN |  |  | M |  | 1 byte |
| 2 | Pointer to record in EFEXT1 |  |  | M |  | 1 byte |
| 3 | Pointer to record in EFPBC |  |  | M |  | 1 byte |
| 4 | Pointer to record in EFANR |  |  | O |  | 1 byte |
| 5 | Pointer to record in EFSNE |  |  | O |  | 1 byte |

## Abbriviated Dialling Numbers EFADN

The phone book is an extension of the existing ADN feature in the GSM SIM. The existing ADN feature contains the EFADN and EFEXT1. These two data fields are mandatory in phone book for GSM compatibility. The content of the EFADN is as described in table 3. The usage of the extension file EFEXT1 is compatible with GSM. This means that at the end of the record in EFADN a pointer to the corresponding entry in EFEXT1 can be found. The same pointer value can be found in each record in the in index administration file.

Table 3 Structure of EFADN

| Identifier: '6F3A' |  | Structure: linear fixed |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: X+14 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE CHV1 INVALIDATE CHV2 REHABILITATE CHV2 |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | Alpha Identifier |  |  | O |  | X bytes |
| X+1 | Length of BCD number/SSC contents |  |  | M |  | 1 byte |
| X+2 | TON and NPI |  |  | M |  | 1 byte |
| X+3 to X+12 | Dialing Number/SSC String |  |  | M |  | 10 bytes |
| X+13 | Capability/Configuration Identifier |  |  | M |  | 1 byte |
| X+14 | Extension1 Record Identifier |  |  | M |  | 1 byte |

## EFEXT1 (Extension1)

This EF contains extension data of an ADN/SSC, an MSISDN, or an LND. Extension data is caused by:

-	an ADN/SSC (MSISDN, LND) which is greater than the 20 digit capacity of the ADN/SSC (MSISDN, LND) Elementary File or where common digits are required to follow an ADN/SSC string of less than 20 digits. The remainder is stored in this EF as a record, which is identified by a specified identification byte inside the ADN/SSC (MSISDN, LND) Elementary File. The EXT1 record in this case is specified as additional data;

-	an associated called party subaddress. The EXT1 record in this case is specified as subaddress data.

Table 4 Structure of  EFEXT1

| Identifier: '6FXX' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 13 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE CHV1 INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Record type |  |  | M |  | 1 byte |
| 2 to 12 | Extension data |  |  | M |  | 11 bytes |
| 13 | Identifier |  |  | M |  | 1 byte |

-	Record type

Contents: type of the record

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Called Party Subaddress |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Additional data |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |

b3-b8 are reserved and set to 0;

a bit set to 1 identifies the type of record;

only one type can be set;

'00' indicates the type "unknown".

The following example of coding means that the type of extension data is "additional data":

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  |

-	Extension data

Contents: Additional data or Called Party Subaddress depending on record type.

Coding:

Case 1, Extension1 record is additional data:

The first byte of the extension data gives the number of bytes of the remainder of ADN/SSC (respectively MSISDN, LND). The coding of remaining bytes is BCD, according to the coding of ADN/SSC (MSISDN, LND). Unused nibbles at the end have to be set to 'F'. It is possible if the number of additional digits exceeds the capacity of the additional record to chain another record inside the EXT1 Elementary File by the identifier in byte 13.

Case 2, Extension1 record is Called Party Subaddress:

The subaddress data contains information as defined for this purpose in GSM 04.08 [15]. All information defined in GSM 04.08, except the information element identifier, shall be stored in the SIM. The length of this subaddress data can be up to 22 bytes. In those cases where two extension records are needed, these records are chained by the identifier field. The extension record containing the first part of the called party subaddress points to the record which contains the second part of the subaddress.

-	Identifier

Contents: identifier of the next extension record to enable storage of information longer than 11 bytes.

Coding: record number of next record. 'FF' identifies the end of the chain.

Example of a chain of extension records being associated to an ADN/SSC. The extension1 record identifier (Byte 14+X) of ADN/SSC  is set to 3.

In this example ADN/SSC is associated to additional data (record 3) and a called party subaddress whose length is more than 11 bytes (records 6 and 5).

## Phone book control EFPBC

This EF contains control information related to each entry in the phone book. This EF contains as many records as the EFADN that it is associated with. Each record in EFPBC points to a record in EFADN. Control information like grouping and hidden entry is indicated in this EF.

The structure of the file is linear fixed and a record may contain several TLV objects. The usage of TLV objects allows for different type of control information to be provided in EFPBC. The tag indicates what kind of information is supplied in the value data field. The Tag value 'FF' is reserved to indicate that no further TLV objects are defined in the data part of the record. The content of the remaining part of the record following Tag 'FF' is set to 'FF'.

The contents of EFPBC is linked to the associated EFADN record by means of a pointer to the record in the index administartion file.

If the grouping tag is indicated in a record in EFPBC the contents of the data field following the tag is the grouping information that is assigned to the phone book entry. If no grouping TLV object is indicated in EFPBC no grouping information is associated with the phone book entry.

If the hidden tag is indicated in a record in EFPBC the contents of the phone book entry shall not be displayed to the user unless the hidden code has been verified. The hidden code is contained in the TLV object. The terminal shall verify the hidden code provided in the TLV object before displaying the contents of the phone book entry that is indicated as hidden by the TLV object in EFPBC.

Table 5 Tag definition for EFPBC

| Tag Value | TAG Description |
| --- | --- |
| '10' | Grouping |
| '11' | Hidden |
| 'FF' | Empty |

Table 6 Structure of EFPBC

| Identifier: '6FXX' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: X+2 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ EFADN UPDATE EFADN INVALIDATE CHV2 REHABILITATE CHV2 |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Index administration file Record Identifier |  |  | M |  | 1 byte |
| 2-X+2 | TLV object strings |  |  | M |  | X+1 bytes |

## Additional Number EFANR

Several Phone numbers can be attached to one EFADN record. The additional number information is provided in an elementary file. The EF structure is linear fixed and the record has a format containing several TLV objects. Each TLV object contains a number. The Tag indicates what type of number is indicated in the value field of the TLV object. This EF if present contains as many records as EFADN. This structure allows several types of numbers to be stored in the same record. Tags associated with EFANR are defined in table 7.

The coding rules for the EFADN dialling number/SSC string applies for the EFSNE TLV object value field. The Tag value 'FF' is reserved to indicate that no further TLV objects are defined in the data part of the record. The content of the data part of the record following Tag 'FF' is set to 'FF'.

Table 7. Tag definitions for EFANR

| Tag Value | TAG Description |
| --- | --- |
| '01' | Private |
| '02' | Business |
| '03' | Fax |
| '04' | Work |
| '05' | Home |
| 'FF' | Empty |

The structure of EFANR is shown in table 8

Table 8. Structure of EFANR

| Identifier: '6FXX' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: X+2 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ EFADN UPDATE EFADN INVALIDATE CHV2 REHABILITATE CHV2 |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Index administration file Record Identifier |  |  | M |  | 1 byte |
| 2-X+2 | TLV object strings |  |  | M |  | X+1 bytes |

## Second Name EFSNE

The phone book also contains the option of a second name entry. The second name entry is associated with the ADN record through the pointer in the index administration file. The amount of second name entries may be less than or equal to the amount of records in EFADN. The structure of the EFSNE (Second NamE) is identical to that of EFADN but the actual number information is missing. The amount of EFADN records may exceed 254 and are then split into two data fields. In order to link the second name record to the correct EFADN entry the control information in a second name record contains a pointer to the index file record identifier from where the correct EFADN can be found.

Table 4: Structure of  EFSNE

| Identifier: '6FXX' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: X+2 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ EFADN UPDATE EFADN INVALIDATE CHV2 REHABILITATE CHV2 |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | Alpha Identifier of Second Name |  |  | M |  | X bytes |
| X+1 | Index file Record Identifier |  |  | M |  | 1 byte |
| X+2 | Index administration file Record Identifier |  |  | M |  | 1 byte |

# Application Specific Phone Book Files

## Incoming Call Information EFICI

This EF is application related and located within the USIM application. The incoming call information can be linked to the phone book stored under DFTELECOM or to the local phone book within the USIM. The EFICI contains the information related to incoming calls.

The time of the call and duration of the call is stored in this EF. This EF can also contain an alpha identifier that may be supplied with the incoming call. This EF is of cyclic structure and contains a limited amount of entries. When the EF is full the next entry overwrites the first entry in the file.

If CLI is supported and the incoming phone number matches a number stored in the phone book the incoming call information is linked to the corresponding information in the phone book. If the incoming call matches an entry in the phone book the link is established but the information is not displayed by the ME if the key for the secret entry has not been verified. The ME shall not ask for the secret key to be entered at this point.

The public part of the phone book is located in DFTELECOM where as the application specific phone book is located within the USIM. In order to link the correct phone book and the correct entry to the incoming call information and indication of where the phone book is located must be provided. The indication of the phone book location is through EFDIR. Each record in EFICI contains a reference pointer to the application in EFDIR. The location of DFTELECOM is specified in the EFDIR as well as the location of the USIM application.

As the phone book may consist of more than one EFADN with associated information the record in EFICI must be linked to the correct EFADN and the correct record in EFADN. Linking is achieved by referring to the appropriate index file and the record number in the index file (this gives the correct EFADN).

Each record in the index file contains a list of EFs associated with the EFADN and the file ID of the EFADN itself (this gives the EFADN location). Each record in the EFICI also contains a pointer to the record in the in EFADN (this gives the location of the record in EFADN).

This  allows the usage of one EFICI to be associated with several EFADN files. The structure of EFICI is shown in table 6. The number content is coded using the rules for EFADN .

Table 9 Structure of EFICI

| Identifier: 'XXXX' |  | Structure: Cyclic |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: X+14 bytes |  |  | Update activity: high |  |  |  |
| Access Conditions: READ CHV1 UPDATE CHV1 INVALIDATE CHV2 REHABILITATE CHV2 |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | Alpha Identifier |  |  | O |  | X bytes |
| X+1 | Length of BCD number/SSC contents |  |  | M |  | 1 byte |
| X+2 to X+11 | Incoming Call Number/SSC String |  |  | M |  | 10 bytes |
| X+12 | Pointer to application in EFDIR |  |  | M |  | 1 byte |
| X+13 | Pointer to Phone book index file record ID |  |  | M |  | 1 byte |
| X+14 | Pointer to index administration file record ID |  |  | M |  | 1 byte |

## Outgoing Call Information EFOCI

This EF is application related and located within the USIM application. The outgoing call information can be linked to the phone book stored under DFTELECOM or to the local phone book within the USIM. The EFOCI contains the information related to outgoing calls.

The time of the call and duration of the call is stored in this EF. This EF is of cyclic structure and contains a limited amount of entries. When the EF is full the next entry overwrites the first entry in the file.

If the dialled phone number matches a number stored in the phone book the outgoing call information is linked to the corresponding information in the phone book. If no match is found in the phone book the pointers to the Pointer to Phone book index file record ID and Pointer to index administration file record ID are set to 'FF'. The dialled number may match with a secret entry in the phone book. If the dialled number matches a secret entry in the phone book the link is established but the information related to the phone book entry is not displayed by the ME, if the secret key has  not been verified. The ME shall not perform secret key verification at this point.

The public part of the phone book is located in DFTELECOM where as the application specific phone book is located within the USIM. In order to link the correct phone book and the correct entry to the incoming call information and indication of where the phone book is located must be provided. The indication of the phone book location is through EFDIR. Each record in EFOCI contains a reference pointer to the application in EFDIR. The location of DFTELECOM is specified in the EFDIR as well as the location of the USIM application.

As the phone book may consist of more than one EFADN with associated information the record in EFICI must be linked to the correct EFADN and the correct record in EFADN. Linking is achieved by referring to the appropriate index file and the record number in the index file (this gives the correct EFADN).

Each record in the index file contains a list of EFs associated with the EFADN and the file ID of the EFADN itself (this gives the EFADN location). Each record in the EFOCI also contains a pointer to the record in the in EFADN (this gives the location of the record in EFADN).

This allows the usage of one EFICI to be associated with several EFADN files. The structure of EFICI is shown in table 6. The number content is coded using the rules for EFADN .

Table 10 Structure of EFOCI

| Identifier: 'XXXX' |  | Structure: Cyclic |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 14 bytes |  |  | Update activity: high |  |  |  |
| Access Conditions: READ CHV1 UPDATE CHV1 INVALIDATE CHV2 REHABILITATE CHV2 |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Length of BCD number/SSC contents |  |  | M |  | 1 byte |
| 2 to X+11 | Outgoing Call Number/SSC String |  |  | M |  | 10 bytes |
| 12 | Pointer to application in EFDIR |  |  | M |  | 1 byte |
| 13 | Pointer to Phone book index file record ID |  |  | M |  | 1 byte |
| 14 | Pointer to index administration file record ID |  |  | M |  | 1 byte |

## Accumulated Call Counter EFACC

This EF contains the an accumulated call counter value for the current call and previous calls. The EF is USIM specific and resides within the USIM application. The structure of EFACC is shown in table

Table 11 Structure of EFACC

| Identifier: '6FXX' |  | Structure: cyclic |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  | Update activity: high |  |  |  |
| Access Conditions: READ CHV1 UPDATE CHV1/CHV2 (fixed during administrative management) INCREASE CHV1 INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 - 3 | Accumulated count of units |  |  | M |  | 3 bytes |

Coding:

First byte:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 223 |  | 222 |  | 221 |  | 220 |  | 219 |  | 218 |  | 217 |  | 216 |  |  |

Second byte:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 215 |  | 214 |  | 213 |  | 212 |  | 211 |  | 210 |  | 29 |  | 28 |  |  |

Third byte:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 27 |  | 26 |  | 25 |  | 24 |  | 23 |  | 22 |  | 21 |  | 20 |  |  |

For instance, '00' '00' '30' represents 25+24.