TD <>

3GPP 21.11 V1.5.0 (1999-04)

Technical Report

Third Generation Partnership Project (3GPP);

USIM and IC Card Requirements

3GPP

Reference

<WORKITEM> (070000c3.PDF)

Keywords

<keyword[, keyword]>

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 3GPP 1999.

All rights reserved.

Contents

Intellectual Property Rights

Foreword

1	Scope

2	References

2.1	Normative references

2.2	Informative references

3	Definitions, symbols and abbreviations

3.1	Definitions

3.2	Symbols

3.3	Abbreviations

4	General Requirements

5	Security Requirements

5.1	File access conditions

5.2	User authentication

5.3	User data stored in ME

5.4	Authentication

5.5	Data integrity of signalling elements

5.6	User identity confidentiality

5.7	Length of security parameters

6	Logical issues

6.1	Application selection

6.2	Simultaneous access

7	Service Requirements

7.1	User profiles

7.2	Data transfer

7.3	Application execution environment

7.4	Profile exchange

7.5	Version identification

8	Physical Characteristics

8.1	Dimensions

8.2	Contacts

9	Electrical characteristics and transmission protocols

9.1	Power consumption indication

10	Contents of the Elementary Files

10.1	USIM information storage requirements

10.2	Phone Book

10.2.1	Support of two names fields per entry

10.2.2	Support of multiple phone numbers per entry

10.2.3	Support of email address

10.2.4	Support of user definable groupings

10.2.5	Support of hidden entries

10.2.6	Number of entries

10.2.7	Mode of alerting

10.3	Storage of call details

11	3GPP/GSM interworking

11.1	GSM subscribers in a 3GPP network

11.2	3GPP subscribers in a GSM network

History

# Intellectual Property Rights

# Foreword

# 1	Scope

This document defines the requirements of the USIM (Universal Subscriber Identity Module) and the IC card for 3GPP (UICC). These are derived from the service and security requirements defined in 3GPP 22.00 [1] and 22.01 [2]. The USIM is a 3GPP application on an IC card. It inter-operates with a 3GPP terminal and provides access to 3GPP services.

This document is intended to serve as a basis for the specification of the USIM and the UICC, and the interface to the 3GPP terminal.

# 2	References

References may be made to:

a)	specific versions of publications (identified by date of publication, edition number, version number, etc.), in which case, subsequent revisions to the referenced document do not apply; or

b)	all versions up to and including the identified version (identified by "up to and including" before the version identity); or

c)	all versions subsequent to and including the identified version (identified by "onwards" following the version identity); or

d)	publications without mention of a specific version, in which case the latest version applies.

A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

## 2.1	Normative references

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.

-	A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

[1]	3GPP 22.00: "Universal Mobile Telecommunications System (UMTS); UMTS phase 1".

[2]	3GPP 22.01: "Universal Mobile Telecommunications System (UMTS); UMTS service aspects; Service principles".

[3]	ISO/IEC 7816-3 (1997): "Identification cards - Integrated circuit(s) cards with contacts, Part 3: Electronic signals and transmission protocols".

[4]	ISO/IEC 7816-4 (1995): "Identification cards - Integrated circuit(s) cards with contacts, Part 4: Interindustry commands for interchange".

[5]	ISO/IEC 7816-5 (1994): "Identification cards - Integrated circuit(s) cards with contacts, Part 5: Numbering system and registration procedure for application identifiers".

[6]	ETSI EG 201 220: "Integrated Circuits Cards (ICC); ETSI numbering system for telecommunication; Application providers (AID)".

[7]	GSM 11.11: "Digital cellular telecommunications system (Phase 2+); Specification of the Subscriber Identity Module - Mobile Equipment (SIM - ME) interface".

[8]	GSM 11.12 (ETS 300 641): "Digital cellular telecommunications system (Phase 2); Specification of the 3 Volt Subscriber Identity Module - Mobile Equipment (SIM - ME) interface".

[9]	GSM 11.14: "Digital cellular telecommunications system (Phase 2+); Specification of the SIM Application Toolkit for the Subscriber Identity Module - Mobile Equipment (SIM - ME) interface".

[10]	GSM 11.18: "Digital cellular telecommunications system (Phase 2+); Specification of the 1.8 Volt Subscriber Identity Module - Mobile Equipment (SIM - ME) interface".

[11]	3GPP S3.03 "3G security: Security Architecture". (note: S3.03 is a temporary number that is likely to change).

## 2.2	Informative references

[20]	GSM 02.48: "Digital cellular telecommunications system (Phase 2+); Security Mechanisms for the SIM application toolkit; Stage 1".

[21]	GSM 03.48: "Digital cellular telecommunications system (Phase 2+); Security Mechanisms for the SIM application toolkit; Stage 2".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following definitions apply:

UICC:	A removable IC card containing a USIM.

USIM: 	A 3GPP application on an IC card.

## 3.2	Symbols

Vpp	Programming voltage

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ADN	Abbreviated Dialling Number

ATR	Answer To Reset

DF	Dedicated File

EF	Elementary File

FFS	For Further Study

ICC	Integrated Circuit Card

IK	Integrity Key

IMUI	International Mobile User Identity

ME	Mobile Equipment

MF	Master File

PIN	Personal Identification Number

PPS	Protocol and Parameter Selection

SIM	Subscriber Identity Module

UIA	3GPP Integrity Algorithm

USIM	Universal Subscriber Identity Module

# 4	General Requirements

The UICC shall be a removable module containing a USIM. The USIM shall contain an identity which unambiguously identifies a subscriber.

For access to 3GPP services, a UICC containing a valid USIM shall be present at all times, other than for emergency calls.

The specifications shall support the security requirements as defined in S3.03 [11].

The USIM shall provide storage for subscription and subscriber related information.

The UICC/USIM may also contain applications which use the features defined in the SIM Application Toolkit specification GSM 11.14 [9].

# 5	Security Requirements

The USIM shall be used to provide security features. If the UICC is removed from the 3GPP terminal, the service shall be terminated immediately. The functions of the USIM include authenticating itself to the network and vice versa, authenticating the user and providing additional security functions as defined by 3GPP TSG-SA WG3.

The USIM shall be unambiguously identified, also in the case of pre-paid subscriptions.

Means shall be provided to prevent fraudulent use of stolen IC Cards.

It shall not be possible to access data intended for USIM internal use, e.g. authentication keys.

Further details of the following requirements are given in S3.03 [11].

## 5.1	File access conditions

Actions, such as READ, UPDATE on UICC data shall be controlled by access conditions. These shall be satisfied prior to the action being performed.

Since a UICC may contain multiple (3GPP and non-3GPP) applications, a flexible method of controlling file access shall be provided.

## 5.2	User authentication

The USIM shall support means to authenticate the user, to provide, for example, protection against the use of stolen cards. For the USIM, authentication shall be performed by the verification of a numeric PIN of four (4) to eight (8) decimal digits.

A function to disable user authentication may exist which may be inhibited by the application provider, in which case the user shall always use the PIN. Otherwise, the user may decide whether or not to make use of the user authentication function. If disabled, the user authentication function remains disabled until the user specifically re-enables it.

Following correct PIN presentation, the ME may perform functions and actions on USIM data, which are protected by the relevant access condition.

If an incorrect PIN is entered, an indication shall be given to the user. After three (3) consecutive incorrect entries the relevant PIN is blocked, i.e. functions and actions on data protected by the access condition shall no longer be possible, even if between attempts the UICC has been removed, the USIM has been deselected or the ME has been switched off. Once a PIN is blocked, further PIN verifications shall be denied.

The USIM shall support a mechanism for unblocking a blocked PIN. Unblocking of a PIN is performed by using the relevant PIN Unblocking Key.

PINs, but not Unblock PINS, shall be changeable by the user following correct entry of either the current PIN or Unblock PIN.

The Unblock PIN shall consist of eight (8) decimal digits and shall not be changeable by the user. If an incorrect Unblock PIN is presented, an indication shall be given to the user. After ten (10) consecutive incorrect entries, the Unblock PIN shall be blocked, even if between attempts the UICC has been removed, the USIM has been deselected or the ME has been switched off. Unblocking of a blocked PIN shall not be possible.

It shall not be possible to read PINs or Unblock PINs.

## 5.3	User data stored in ME

Subject to the exception below, all user related information transferred into the ME during network operations shall be deleted from the ME after removal of the UICC, deselection of the USIM, deactivation of the ME, or following an electrical reset of the UICC. [This includes any data that was transferred to the ME by SIM Application Toolkit commands. FFS]

User related security codes such as PIN and Unblock PIN may only be stored by the ME during the procedures involving such a code and shall be discarded by the ME immediately after completion of the procedure.

Optionally, an ME may retain some less security-critical data at UICC removal, USIM deselection or ME switch-off. Such data are SMS, ADN/SSC, FDN/SSC, LND. These data, when stored in the ME, shall only be readable/retrievable if the same USIM is reactivated (as determined by the IMUI). If the IMUI is retained in the ME for this purpose, it shall be stored securely and shall not be able to be read out.

## 5.4	Authentication

A means shall be specified to mutually authenticate the USIM and the network by showing knowledge of a secret key K which is shared between and available only to the USIM and in the user's Home Environment. The method is composed of a challenge/response protocol identical to the GSM user authentication and key establishment protocol combined with a sequence number-based one-pass protocol for network authentication.

## 5.5	Data integrity of signalling elements

Some signalling information elements are considered sensitive and must be integrity protected. An integrity function shall be applied on certain signalling information elements transmitted between the ME and the network.

The 3GPP Integrity Algorithm (UIA) shall be implemented in the USIM.

The UIA shall be used with an Integrity Key (IK) to compute a message authentication code for a given message. The setting of IK is triggered by the authentication procedure.

## 5.6	User identity confidentiality

A mechanism shall be specified to provide user identity confidentiality by means of a temporary identity. If a temporary identity is not available in the serving network, a means of encrypting the permanent user identity (IMUI) with a group key may be used. The requirement for this mechanism is still under study by TSG-SA WG3.

## 5.7	Length of security parameters

In order to allow for enhancements of the security level in 3GPP, the following requirements shall be covered:

-	all security-related parameters for UMTS shall be accompanied by a length indicator;

-	the USIM shall support variable-length security parameters.

If the USIM supports the GSM security mechanisms in addition to 3GPP security, fixed length security parameters according to GSM 11.11 [7] shall be supported in addition.

# 6	Logical issues

## 6.1	Application selection

In a multiapplication environment, a flexible application selection method is required. The application identifier defined in ISO/IEC 7816-5 [5] and EG 201 220 [6] should be used for application selection. Direct application selection and the EFDIR concept of ISO/IEC 7816-4 [4] shall be followed.

## 6.2	Simultaneous access

A mechanism shall be specified for simultaneous access to several files or applications.

# 7	Service Requirements

## 7.1	User profiles

Each USIM shall contain at least one user profile [FFS].

## 7.2	Data transfer

A mechanism allowing highly secure transfer of applications and/or associated data to/from the UICC/USIM shall be specified in line with the requirements in 3GPP 22.01 [2]. This requires a secure transfer mechanism. GSM 02.48 [20] and GSM 03.48 [21] could be considered here, however this is limited to the case where the application to be downloaded runs in the context of an existing subscription. The security requirements in the case where, for instance, a new USIM or other application has to be downloaded, requires further study.

It is envisaged that in early USIM specifications, the transfer of subscription-related applications (e.g. SIM application toolkit applications) will be specified. The generic application download (e.g. download of a new USIM) is not likely to be included in these early specifications.

Application creation comprises file creation and other administrative operations on the, as well as negotiation of code type or language.

## 7.3	Application execution environment

An application execution environment may exist on the UICC/USIM which includes the functionality defined in GSM 11.14 [9].

## 7.4	Profile exchange

A mechanism for the ME, the USIM and the network to exchange service capabilities shall be specified. The following exchange of service capabilities may occur:

-	ME services capabilities may be provided to the USIM/UICC;

-	USIM/UICC services capabilities may be provided to the ME (and thus potentially to the network);

-	network services capabilities may be provided to the USIM/UICC via the ME.

Editors note: This requirement needs to be ratified by the TSG-SA1 (services) group.

## 7.5	Version identification

A means for identification of the version of the USIM shall be provided.

# 8	Physical Characteristics

## 8.1	Dimensions

The ID-1 and Plug-in format used for the GSM SIM shall be adopted. A third format, smaller than the Plug-in format, is for further study. If a new format is defined, a means shall be specified in order to prevent an incorrect insertion of the card into the ME.

## 8.2	Contacts

The UICC shall not provide any connection to the VPP contact. The contact shall be provided on the UICC. The ME may support the VPP contact in the reader. The ME shall not have this contact connected; neither to ground nor to the UICC supply voltage.

NOTE:	According to ISO/IEC 7816-3 [3] the VPP contact is RFU for ICCs operating at 3V.

# 9	Electrical characteristics and transmission protocols

Electronic signals and transmission protocols shall be in accordance with ISO/IEC 7816-3 [3] unless specified otherwise.

The electrical specifications shall at least cover the 1.8V and 3V voltage ranges as specified in GSM 11.12 [8] and GSM 11.18 [10]. Lower voltages may be added in the future. 3GPP terminals shall not support 5V on the ME-UICC interface. Both ME and UICC shall support operational class indication as defined in ISO/IEC 7816-3 [3]. Both ME and UICC shall support at least two voltage classes.

Both UICC and ME shall support PPS as defined in ISO/IEC 7816-3 [3] with at least the values defined in GSM 11.11 [7].

The ME shall have the capabilities of initiating a warm reset as defined in ISO/IEC 7816-3 [3]. The UICC shall support warm reset as defined in ISO/IEC 7816-3 [3].

NOTE: 	The warm reset is used during a session when there is a need to restart the USIM due to internal modifications of data caused by user actions or network data downloading.

The UICC may indicate in the ATR to the warm reset that the specific mode is entered automatically, using the parameters that were used prior to the warm reset. In case of a cold reset, the UICC shall enter the negotiable mode.

In addition to the T=0 protocol which is mandatory for the UICC and the ME, the T=1 protocol shall be mandatory for the ME. It is optional for the UICC.

The speed enhancement as specified in GSM 11.11 [7] shall be supported by both the ME and the UICC. Higher interface bit rates than those specified in GSM 11.11 [7] should be considered.

## 9.1	Power consumption indication

Power consumption figures are to be revised based on the need for more secure authentication algorithms, utilising crypto co-processors. In order to be compatible with the GSM specifications, the UICC shall meet the power consumption specifications set in GSM 11.12 [8] and GSM 11.18 [10] during the ATR. The USIM status information shall contain power consumption information, which is related to the operational class indicated in the ATR and the operating frequency indicated for running the authentication algorithm.

NOTE:	The power consumption figure may differ between different applications on the UICC; thus a particular ME may support some applications in a card and reject others, depending on the power consumption values.

The ME may reject the USIM if it can not supply the current indicated in the status information and if this current is above the maximum value defined for use by the UICC for running the USIM.

# 10	Contents of the Elementary Files

## 10.1	USIM information storage requirements

The USIM shall contain information elements for 3GPP network operations. The USIM may contain information elements related to the subscriber, 3GPP services and home environment or service provider related information.

The UICC shall provide storage capability for the following:

-	UICC related information:

-	IC card identification: a number uniquely identifying the UICC and the card issuer;

-	Preferred language(s);

-	Directory of applications.

-	 USIM related information:

-	Administrative information: indicates mode of operation of the USIM, e.g. normal, type approval;

-	USIM service table: indicates which optional services are provided by the USIM;

-	IMUI;

-	Language indication;

-	Location information;

-	Cipher key (Kc) and cipher key sequence number;

-	Access control class(es);

-	Forbidden PLMNs;

-	Phase identification;

-	Ciphering Key for GPRS;

-	GPRS location information;

-	Cell Broadcast related information;

-	Emergency call codes;

-	Phone numbers (ADN, FDN, SDN);

-	Short messages and related parameters;

-	Capability and Configuration parameters;

-	HPLMN search period [FFS];

-	BCCH information: list of carrier frequencies to be used for cell selection [FFS].

-	 Information accessible to the USIM and other applications:

-	ADN.

In addition, the USIM shall manage and provide storage for the following information in accordance with the security requirements of clause 5:

-	PIN;

-	PIN enabled/disabled indicator;

-	PIN error counter;

-	Unblock PIN;

-	Unblock PIN error counter;

-	Data integrity keys;

-	Subscriber authentication keys.

## 10.2	Phone Book

The Phone Book feature is based on the ADN functionality as defined in GSM 11.11 [7]. Additional features are identified in the following subclauses. A Phone Book entry consists of a record in an ADN file and, optionally, additional records which are placed in different EFs. In the latter case, a mechanism shall be defined to link all records in the same Phone Book entry. These features shall be supported by the ME while their support by the USIM is optional.

### 10.2.1	Support of two name fields per entry

The support of two name fields per entry shall be specified to allow, for example, for two different representations of the same name, for example, in Japanese and English.

### 10.2.2	Support of multiple phone numbers per entry

The support of multiple phone numbers per entry shall be specified, for example office, home, fax, mobile or pager. In addition to that, information for identifying those attributes are needed.

### 10.2.3	Support of email address

The support of email addresses linked to Phone Book entries shall be specified. In addition to that, information for identifying these addresses is needed.

### 10.2.4	Support of user definable groupings

The specification shall support the grouping of Phone Book entries into groups defined by the user, for example, business and private.

### 10.2.5	Support of hidden entries

The specification shall support means of marking Phone Book entries as "hidden".

### 10.2.6	Number of entries

The specification shall support storage of at least 500 entries.

### 10.2.7	Mode of alerting

[FFS]

## 10.3	Storage of call details

The specification shall support provision of storage for call detail information. The call detail information consists of the following attributes:

-	mobile terminated calls:

calling party number, date and time, calling party's name and status of call (i.e. answered or missed), duration and [FFS] charge;

-	mobile originated calls:

called party number, date and time, called party's name, duration and [FFS] charge;

-	accumulated duration of preceding calls, separately for mobile originated and mobile terminated calls;

-	accumulated charge information of preceding calls [FFS].

Call detail attributes are optional. A value to mark them as "undefined" shall be available.

NOTE 1:	The calling/called party's name may be available from the Phone Book.

NOTE 2:	The storage of multiparty call information is FFS.

# 11	3GPP/GSM interworking

## 11.1	GSM subscribers in a 3GPP network

3GPP 22.01 [2]: "UMTS shall provide some mechanisms which permit pre UMTS subscribers to roam easily onto UMTS and access the services."

3GPP 22.00 [1]: "The UMTS mobile terminal shall support phase 2 and phase 2+ GSM SIMs as access modules to UMTS networks. The services that can be provided in this case may be limited to GSM like services provided within that UMTS network. It shall be up to the UMTS network operator whether or not to accept the use of GSM SIM as access modules in its network".

## 11.2	3GPP subscribers in a GSM network

The following requirement is made in 3GPP 22.01 [2]: "UMTS shall provide some mechanisms which permit UMTS subscribers to roam easily onto pre-UMTS systems and access the services."

This may be achieved by providing all mandatory elements as defined in GSM 11.11 [7] in the UICC.

The specification shall allow the UICC to be used with a dual mode (GSM/ 3GPP) ME and a GSM ME for the provision of GSM service.

NOTE:	This does not, for example, preclude the support of the PDC application on a UICC.

# History

| Document history |  |  |
| --- | --- | --- |
| 0.1.0 | 2 October, 1998 | Produced by the rapporteur for consideration at SMG9 UMTS #5 (6-7 October, 1998) |
| 0.2.0 | 26 October, 1998 | Inclusion of material agreed at SMG9 UMTS #5 (6 - 7 October, 1998) in tdoc 98u044. Abbreviations and references section also expanded. |
| 0.3.0 | 5 November, 1998 | Inclusion of material agreed at SMG9 UMTS #6 (4 - 5 November, 1998) |
| 0.4.0 | 22 December, 1998 | Inclusion of material agreed at SMG9 UMTS #7 (15 – 16 December, 1998) |
| 1.0.0 | 21 January, 1999 | Agreed at SMG9 #17 (18 - 22 January, 1999) for submission to 3GPP T3 |
| 1.1.0 | 27 January, 1999 | Modifications as agreed at 3GPP TSG-T#1 (25 - 27 January, 1999) |
| 1.2.0 | 19 February, 1999 | Modifications as agreed at 3GPP TSG-T#2 (17 - 19 February, 1999) |
| 1.3.0 | April, 1999 | Modifications as agreed at 3GPP TSG-T#3 (16 - 18 March, 1999) |
| 1.4.0 | 21 April, 1999 | Changes as agreed by a splinter group during TSG-T3 #4 (19-21 April, 1999) |
| 1.5.0 | 21 April, 1999 | Version agreed at TSG-T3 #4 (19-21 April, 1999) for submission to TSG-T |