# Proposal for General Authentication Command to be incorporated in 31.101 and specific coding to be incorporated in 31.102

Source: Nokia

### AUTHENTICATE

Table y1 AUTHENTICATE Command APDU

| Code | Value |
| --- | --- |
| CLA | As specified in ux.y.z |
| INS | As specified in ux.y.z |
| P1 | Authentication algorithm indication |
| P2 | Command processing and reference of secret |
| Lc | Length of subsequent data field |
| Data | Command Data |
| Le | Length of response data |

Parameter P1 specifies the authentication algorithm:

Table y2  Coding of the algorithm control P1

| Coding b8-b1 | Meaning |
| --- | --- |
| '00000000' | Default Authentication Algorithm (specified by the application), no information given |
| '0XX---------' | '00' (all other values RFU |
| '0--XXXXX' | Authentication algorithm X (specified by the application), applicable globally |
| '1------------' | Authentication algorithm X (specified by the application), applicable to the selected DF/Application |

P2 specifies the authentication secret as follows:

Table y3 Coding of the reference control P2

| Coding b8-b1 | Meaning |
| --- | --- |
| '00000000' | No information given |
| '0-----------' | Global reference data (e.g. MF specific/Application independent key |
| '-XX-----' | '00' (All other values application specific) |
| '1------------' | Specific reference data (e.g. DF specific/application depenent key |
| '---XXXXX' | Reference /Number of secret |

If P1, P2= '00', The default application authentication is indicated. The format, type and parameters of the command data field are proprietary to the application. The command data field is interpreted correctly by a terminal supporting the application

Command parameters/data:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 - X | Application specific authentication data | X |

### Response Data

The response data is application specific and the interpretation is known by the terminal issuing the AUTHENTICATION command using parameters P1 and P2 to indicate the algorithm and reference data.

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 - X | Application specific authentication response data | X |

### Command Specific Errors

| SW1 | SW2 | Error description |
| --- | --- | --- |
| '98' | '62' | - Authentication failure, application specific |
| '98' | '64' | - Authentication failure, reference data mismatch |

### Codings to be incorporated in 31.102

Table y3 Coding of the reference control P2

| Coding b8-b1 | Meaning |
| --- | --- |
| '00000000' | No information given |
| '0-----------' | Global reference data (e.g. MF specific/Application independent key |
| '-XX-----' | '00' Normal processing of Reference data '01' Resynchronisation Required '10' Mutual authentication data requested '11' Proprietary |
| '1------------' | Specific reference data (e.g. DF specific/application depenent key |
| '---XXXXX' | Reference /Number of secret |

Command parameters/data:

In case P1, P2='00':

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 - X | Proprietary | X |

In case P1<>'00':

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1-X | TLV Object(s) using Tags as defined in | X |

### Response Data

The response data in case P1<>'00' is as defined by the tags in table y4

Table y4 Tag definitions for response data

| Tag | Meaning |
| --- | --- |
| 'XX' | Random Challenge |
| 'XY' | Verification Challenge |
| 'XZ' | Data Reference |

### Command Specific Errors

| SW1 | SW2 | Error description |
| --- | --- | --- |
| '98' | '62' | - Authentication failure, integrity problem |
| '98' | '64' | - Authentication failure, reference data mismatch |