TSG-T3 (USIM) meeting #7	Tdoc T3-99198

5th to 7th of July

Lund, SWEDEN

Agenda Item:10.

For discussion

Source: NTT DoCoMo, Japan Telecom Co.,Ltd.

Structure of SIMATK specifications in 3GPP

1.Introduction(quoted from #6 meeting report)

During the #6 meeting, it was proposed that the SIM Application Toolkit functionality described in GSM 11.14 should be transferred to the 3GPP and be incorporated into the December 1999 release. A discussion of this request indicated that there would be several problems:

-	how will it affect the basic platform?

-	the existing commands in 11.14 would need to be divided into 2 groups - those that relate to the UICC and those that relate to the USIM;

-	GSM 11.14 is based on 11.11 which, in the 3GPP, is split up into 2 parts;

-	the co-existence of more than one USIM (each having their own toolkit applications) would be very complex;

-	which companies would provide resources to transfer the functionality.

It was proposed that those interested in adding the SIM application toolkit functionality should provide input to T3 #7 meeting addressing the above issues.

2.Discussion

2.1.Proposed Principle

Followings are the latest assumptions made in SA.

- SIMATK shall be provided in 3G, according to 3GTS22.121(VHE)

- Number of voice-call will be limited in 1 in R99, according to 3GTS22.135(Multicall)

- Multiple USIM environment is out-of requirement of R99, according to 3GTS21.100(Phase1 Capa.). Once T3 discussed this, and concluded that specification work for multi-USIM is needed for the purpose of future-proof, even in R99. Therefore this feature should not be a killer-item.

Considering the situation above, we believe that current 11.14 is valid enough also in 3G R99 technically.

Therefore we would like to propose to transfer 11.14 to 3GPP with minimum change/modification(i.e. editorial). And further work like division into UICC and USIM part, then, can be considered as a work items for R2000 USIM/UICC.

2.2.Solutions to the problems arose at the previous meeting

If we apply the basic principle above, following measures are led.

- how will it affect the basic platform?

- Nothing in R99, but should be studied for R2000 later on

- the existing commands in 11.14 would need to be divided into 2 groups - those that relate to the UICC and those that relate to the USIM;

- This can be considered as work item for R2000.

- GSM 11.14 is based on 11.11 which, in the 3GPP, is split up into 2 parts;

- This can be considered as work item for R2000.

- the co-existence of more than one USIM (each having their own toolkit applications) would be very complex;

- see DoCoMo contribution Tdoc199.

- which companies would provide resources to transfer the functionality.

- Provided limit to minimum change/modification, we don't think so much resource is needed.

3.Proposal and Conclusion

In this document, we would like to propose to transfer 11.14 to 3GPP with minimum change/modification (i.e. editorial). And further work like division into UICC and USIM part, then, can be considered as a work items for R2000 USIM/UICC.