3GPP T3  USIM adhoc													Tdoc t3z99021

14-16 September 1999

Turku, Finland

Source: De La Rue

Comparing clause G commands with GSM and ISO:

For all the commands, it is still unclear in 31.101 whether the status bytes returned by the USIM are compatible with ISO/IEC 7816-4. The current table 12.9 is confusing as it would allow several different values for the same meaning (e.g. 63CX/920X, 6200/9000, 6CXX/67XX, etc). Clarification is required before we can check the compatibility.

As stated by ISO, FCD ISO/IEC 7816-9 “is still under study and subject to change. It should not be used for reference purposes”.

Compatibility of 31.101 with GSM and ISO, not taking status bytes into account

| Clause G | INS | GSM 11.11 | 7816-4 | 7816-7 | FDIS 7816-8 | FCD 7816-9 (No stable reference) | Possible CLA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SELECT FILE | 'A4' | SELECT FILE GSM answer currently not supported by 31.101 ! | SELECT FILE Once GSM is supported, the corresponding answer is compliant via CLA=A0 (no tag). For 3G, compatibility depends on stability of 7816-9 (SFI tag) and on tbd security status handling. But currently not compatible for cyclic EFs (NEXT is record 1 for 7816, and record 2 for 31.101 or GSM 11.11) | - | - | - | 8X, 9X, AX |
| STATUS | 'F2' | STATUS GSM should be supported ! | - | - | - | - | 8X, 9X, AX |
| READ BINARY | 'B0' | READ BINARY No support of SFI, but no compatibility problem with current operational GSM EFs | READ BINARY Identical | - | - | - | 0X, 8X, 9X, AX, BX, CX |
| UPDATE BINARY | 'D6' | UPDATE BINARY Same as for READ B. | UPDATE BINARY Identical | - | - | - | 0X, 8X, 9X, AX, BX, CX |
| READ RECORD | 'B2' | READ RECORD Compatible if SFIs are not used | READ RECORD Not compatible: Partial read allowed. Record identifier versus record number if P1 is not 0. Also record 255 for cyclic EF. | - | - | - | 8X, 9X, AX |
| UPDATE RECORD | 'DC' | UPDATE RECORD Same as READ R. | UPDATE RECORD Same as READ R. | - | - | - | 8X, 9X, AX |
| SEEK | 'A2' | SEEK | - | - | - | SEARCH RECORD Not compatible | 8X, 9X, AX |
| INCREASE | '32' | INCREASE | - | - | - | - | 8X, 9X, AX |
| VERIFY PIN | '20' | VERIFY CHV Compatible if b8 of P2 is 0 | VERIFY Compatible | - | - | - | 0X, 8X, 9X, AX, BX, CX |
| CHANGE PIN (only P1=0 has to be supported) | '24' | CHANGE CHV Compatible if b8 of P2 is 0 | - | - | CHANGE REF. DATA Compatible | - | 8X, 9X, AX |
| DISABLE PIN (only P1=0 has to be supported) | '26' | DISABLE CHV Compatible if b8 of P2 is 0 | - | - | DISABLE VERIF. REQ. Compatible | - | 8X, 9X, AX |
| ENABLE PIN (only P1=0 has to be supported) | '28' | ENABLE CHV Compatible if b8 of P2 is 0 | - | - | ENABLE VERIF. REQ. Compatible | - | 8X, 9X, AX |
| UNBLOCK PIN (only P1=0 has to be supported) | '2C' | UNBLOCK CHV Compatible if b8 of P2 is 0 | - | - | RESET RETRY COUNTER Compatible | - | 8X, 9X, AX |
| INVALIDATE | '04' | INVALIDATE Identical | - | - | - | DEACTIVATE FILE 2G not compatible as READ/UPDATE may still be allowed | 8X, 9X, AX |
| REHABILITATE | '44' | REHABILITATE Identical | - | - | - | ACTIVATE FILE Compatible | 8X, 9X, AX |
| INTERNAL AUTHENTICATE | '88' | RUN GSM ALGO. Compatible if P1=P2=0 | INTERNAL AUTHENTICATE Identical | - | - | - | 0X, 8X, 9X, AX, BX, CX |
| GET RESPONSE | 'C0' | GET RESPONSE Identical | GET RESPONSE Identical | - | - | - | 0X, 8X, 9X, AX, BX, CX |
| TERM. PROFILE (Ambiguous if SAT not in Phase 1) | '10' | TERM. PROFILE Identical | - | P.S.O. Completely different cmd | - | - | 8X, 9X, AX |
| ENVELOPE (Ambiguous if SAT not in Phase 1) | 'C2' | ENVELOPE Identical | ENVELOPE Identical coding but different purpose | - | - | - | 8X, 9X, AX |
| FETCH (Ambiguous if SAT not in Phase 1) | '12' | FETCH Identical | - | P.T.O. Completely different cmd | - | - | 8X, 9X, AX |
| TERMINAL RESPONSE (Ambiguous if SAT not in Phase 1) | '14' | TERM. RESPONSE Identical | - | P.U.E. Completely different cmd | - | - | 8X, 9X, AX |
| MANAGE CHANNEL | 70 | - | MANAGE CHANNEL P1 coding not compatible. It should be ! More details have to be specified before compatibility can assessed | - | - | - | FFS |