ETSI SMG9 Testing WP #13	Tdoc T3Z000092

08– 10 August, 2000

Brussels, Belgium

Subject: 	Proposal of revised format of SIM Application Toolkit Test Spec 11.10-4 for the proactive commands: EVENT DOWNLOAD -USER ACTIVITY (GSM 11.14 release99).

Date:		08 August 2000

Source:	GEMPLUS

#### 27.22.4.3

#### EVENT DOWNLOAD -USER ACTIVITY

##### 27.22.4.3.1 	EVENT DOWNLOAD -USER ACTIVITY(normal)

###### 27.22.4.3.1.1	Definition and applicability

This test is only applicable to ME’s that support the EVENT DOWNLOAD -USER ACTIVITY proactive SIM facility.

###### 27.22.4.3.1.2	Conformance Requirement

The ME shall support the EVENT DOWNLOAD -USER ACTIVITY as defined in the following technical specifications:

TS GSM 11.14 [15] clause 5.2 (Terminal Profile), clause 6.4.16 (Set Up Event List), clause 6.8 (Terminal Response), clause 6.6.16, clause 6.11, clause 11 (Event Download), clause 11.5 (User Activity event), clause 12.6 (Commands details),  clause 12.25 (Event List).

###### 27.22.4.3.1.3	Test Purpose

To verify that the ME performed correctly the procedure of USER ACTIVITY EVENT.

###### 27.22.4.3.1.4	Method of Test

27.22.4.3.1.4.1	Initial Conditions

The ME is connected to the SIM Simulator.

The elementary files are coded as Toolkit default with the following exceptions.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.3.1.4.2	Procedure

Expected Sequence 1 (EVENT DOWNLOAD -USER ACTIVITY)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER -> ME | Reset | The current event list is removed if it’s existing |
| 2 | USER | press any key | Check if no envelope Event Download-User activity sending to the SIM |
| 3 | SIM ® ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1 | [event User Activity] |
| 4 | ME ® SIM | FETCH |  |
| 5 | SIM ® ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1 |  |
| 6 | ME ® SIM | TERMINAL RESPONSE: SET UP EVENT LIST 1.1 | [command performed successfully] |
| 7 | USER | press any key |  |
| 8 | ME ® SIM | ENVELOPE EVENT DOWNLOAD -USER ACTIVITY 1.1 |  |
| 9 | ME | remove the event User Activity in the current list of events |  |
| 10 | USER | press any key | check if no envelope Event Download-User activity sending to the SIM |

Proactive SIM Command 1.1: SET UP EVENT LIST

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:

Device identities

Source device:	SIM

Destination device:	ME

Event List	User Activity

Coding:

BER-TLV:	D0	15	81	03	01	05	00	82	02	81	82	99

01	04

TERMINAL RESPONSE: SET UP EVENT LIST 1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:

Device identities

Source device:	ME

Destination device:	SIM

Result

General Result:	Command performed successfully

Coding:

BER-TLV:	81	03	01	05	80	82	02	82	81	83	01	00

Envelope 1.1: EVENT DOWNLOAD -USER ACTIVITY

Logically:

Event List 	User Activity

Device identities

Source device:	ME

Destination device:	SIM

Coding:

BER-TLV:	D6	0C	81	03	01	05	00	82	02	82	81		99	01	04

###### 27.22.4.3.15.5	Test Requirement

The ME shall operate in the manner defined in expected sequence 1.