| 3GPP T3 Meeting #14 Visby, Sweden, 24 - 26 May, 2000 | Tdoc T3-000196 |
| --- | --- |

Source:	Ericsson

Contact:	Jeremy Norris (jeremy.norris@eml.ericsson.se

Title:	Clarification on the files that can be linked.

Document for:	Discussion 3GPP T3

### Introduction:

With the allowance for both a DFGSM  and a USIM application with files that contain effectively the same information. This poses the question that there should be clarification whether these files are linked within the UICC.

### Problem:

With files that are shared between the DFGSM and the USIM application, a file can be duplicated more than once on the UICC. Such files can be the EFKc,  EFIMSI ,PLMN files etc….

In the case of the EFKc, this can be in two places ie DFGSM , and the USIM application for GSM access.

If for instance we take the case when the EFKc is read under GSM authentication assuming we also have the GSM access files within the USIM application and a DFGSM ,  which one do we use if they are separate files and not linked ?

This needs to be clarified.

Duplicate files also seem to be a waste of UICC resources, to store effectiverly duplicated files.

In another case if the PLMN selector is updated when in a GSM access condition do we have to update the copies if they are not shared in the DFGSM and the USIM application. This could potentially slow the ME when updating the fields and uses resources in the UICC which could better used.

With these two cases ie dual information storage and dual information updates we can potentially also have the problem that under certain condtions the ME will act differently depending on the manufacturer of the ME and UICC and therefore is open to interpretation