3GPP Conference Call on 3GPP Spec Modernization #4	6GSM-250324

# Electronic, 10th Nov 2025, 13:00-15:00 UTC

Source:	Huawei

Title:	Pseudo-CR on Multiple formats for single specification with Word as baseline

Document for:	Decision

Agenda item:	4

Spec:	3GPP TR 21.802

Version:	TR version 0.1.2

Work Item:	FS_6GSpecs

Comments

This pCR proposes a new sub-clause 5.X for describing an option where multiple formats are used for a single specification with Word as baseline.

Proposed Changes

* * * First Change * * * *

## 5.X	Proposal #X: Multiple formats for single specification with Word as baseline

### 5.X.1	Description

In the table below it is described how the various specification elements of a single specification could be stored, i.e. which file format they could be stored, where the main document of the specification is written with Word.

It should be noted that some specification elements can be captured in different formats depending on what is to be captured. One such example is tables, e.g. small tables might best be captured inline with the procedural text and hence captured in Word, but very large tables might best be captured in for example JSON.

Table 5.X.1-1: Specification elements of a single specification, files formats and tools.

| Specification element (NOTE 1,2) | File formats | Tools | Notes |
| --- | --- | --- | --- |
| Main document of the specification, including procedural text and similar (formatted text) | Word (*.docx) | MS Word, WPS | Tools are mentioned as examples. |
| Tables | JSON (*.json) for large tables Small tables within the main Word document Excel (*.xlsx) for tables requiring mathematical calculation. | Any text editor such as Notepad++, Visual Studio Code, Vim, etc JSON syntax checker/validator For excel: MS Windows, WPS | Open-source free tools that have open-source extensions for JSON syntax checking and validation are preferred. |
| Sequence diagrams | Text source file (.txt, or .puml, etc) + generated PNG,JPEG or SVG. | Any text editor + PlantUML or MSC Generator | Any text editor for the textual description of the flow chart. PlantUML or MSC generator for generating the corresponding PNG or JPEG file. |
| Images | PNG, SVG | According to solution 19 (Table 4.2-2): Only cross platform tools. The study may recommend one or more such tools among MSC-gen, plantUML, Mermaid, and Microsoft Word built-in drawing tools. Use of Visio should be discontinued entirely. Editors should be encouraged to use plantUML, MSC, or Mermaid where appropriate and only resort to images without editable source if there is no other option. | Solution 17 was also listed in Table 4.2-2: Store figure source files - For editable figures, store at least the source file, named in accordance with the figure number, alongside the specification. |
| ASN.1 | Text file (*.asn) Normative comments may be included (e.g. -- [normative] this is a normative comment --) | Any text editor such as Notepad++ or Visual Studio Code + optionally ASN.1 syntax checking extension (e.g. ASN.1 by OSS Nokalva Inc. extension for VSCode) | The ASN.1 elements can be distributed in multiple files which then can be referenced/imported in other ASN.1 files. |
| ASN.1 with normative comments and field description tables (such as used in RRC specification) | A text-based format such as Markdown (*.md) or Asciidoc (*.adoc) or LaTeX (*.tex). | Any text editor such as Notepad++, Visual Studio Code, Vim, etc. | This is for the case where ASN.1 with normative comments and field description tables is not directly written in the main Word document(i.e. no need for a tool that could extract the ASN.1 from a Word file) Use of Git is a separate consideration to be further discussed: e.g. editing within Git, using Gitlab for ASN.1 review (i.e. for drafting work) |
| Equations | Within the main Word document (.docx) | Word built-in equation editor |  |
| OpenAPI | Text file (.yaml) | YAML text parser (Notepad++) to generate YAML file (note that new swagger version does not support saving of YAML file and cross checking). OpenAPI YAML syntax checker (Swagger tool) | As done in SA and CT WGs (cf Annex B) |
| XML Schemas | .xsd (XML Schema Definition) | XML syntax checker/validator. | As done in SA3 and SA5 (cf Annex B) |
| YANG model | .yang | SA5 YANG data models are validated by an automated pipeline that runs for every push or merge-request event | As done in SA and CT WGs (cf Annex B) |
| Machine processable files specified in TTCN-3 language | .ttcn | Several TTCN-3 compilers can be used for syntax checking the TTCN-3 files. | As done in RAN5 (cf Annex B) |
| SDL diagrams in SDT binary files | .ini | .ini file supplied on the 3GPP server | cf TR21.801 Annex I |
| NOTE 1: some elements may appear only in some specifications but not in all specifications. NOTE 2: to the extent possible, redundancy shall be discouraged, e.g. if ASN.1 is stored in a separate file, the same content shall not be duplicated in the .DOCX document. |  |  |  |

* * * End of Changes * * * *