3GPP Conference Call on 3GPP Spec Modernization #5	6GSM-260021

Electronic, 15th January 2026, 13:00-15:00 UTC

Source:	Nokia

Title:	pCR for TR21.802 on bit pattern representations in specifications

Agenda item:	5

Work Item:	FS_6GSpecs – Release 20

Document for:	Discussion and Decision

Spec:	3GPP TR21.802 v1.0.0

# 1	Introduction

In this pCR we propose additional entry for the benefits of the current tools as discussed in 6GSM-250115 as discussed during the September 3rd conference call.

# 2	Handling of UP control elements

## 2.1	Bit pattern formats depicted by figures

The bit patterns (or bitmap) representations are often used in RAN2 specifications to describe PDU formats in e.g. MAC, RLC, and PDCP. An example of this is shown below (from MAC specification).

The example on the right shows some typical characteristics of the MAC CEs:

- Depiction of a structure whose bit positions encode specific information

- Text descriptions of each bit string in the MAC CE

- Fixed or variable length size (i.e. need to be able to define both without great difficulty), including showing how to calculate the size of each MAC CE

- Possibility to extend the structure either via leaving some bit positions undefined (via the “R-bits”, or if they are not present simply defining another MAC CE with different logical channel ID)

Since these are images that are sometimes editable (by Visio) and sometimes not, it would be preferable to have a text-based representation for them instead to allow better change tracking.

Observation 1: Replacing the bit pattern Visio figures with text-based formats would help with change tracking.

## 2.2	Text-based alternatives to the bit pattern formats

We see two main alternatives to handling the bit pattern representations:

- Use ASCII text tables similar to IETF RFCs (e.g. as defined by RFC 6949)

- Use ASN.1 to depict the content of the PDUs (e.g. similar to RRC specification, i.e. based on BASIC-PER unaligned variant encoding rules)

To illustrate the differences, we show the same example from above depicted using each of the different formats in Table 1 below.

| Representation | Format |
| --- | --- |
| Current MAC |  |
| ASCII | +----+-----+----+----+----+----+----+----+ ¦ LCG ID ¦ Buffer size ¦ +----+-----+----+----+----+----+----+----+ +------+------+------+------+------+------+------+------+ ¦ LCG7 ¦ LCG6 ¦ LCG5 ¦ LCG4 ¦ LCG3 ¦ LCG2 ¦ LCG1 ¦ LCG0 ¦ +------+------+------+------+------+------+------+------+ ¦ Buffer size 1 ¦ +------+------+------+------+------+------+------+------+ ¦ ... ¦ +------+------+------+------+------+------+------+------+ ¦ Buffer size m ¦ +------+------+------+------+------+------+------+------+ |
| ASN.1 | BufferStatusReportShort ::= SEQUENCE { -- LCG: logicalChannelGroupIdentity INTEGER (1..maxLCG)), -- (Quantized) buffer size: bufferSize INTEGER (0..maxShortBS-ID } maxLCG INTEGER ::= 8 -- Maximum number of LCG identities maxShortBS-ID INTEGER ::= 31 -- Maximum number of short BS IDs BufferStatusReportLong ::= SEQUENCE ( SIZE (1..maxLCG)) OF BufferStatusLong BufferStatusLong ::= SEQUENCE { -- LCG: logicalChannelGroupIdentity INTEGER (1..maxLCG)), -- (Quantized) buffer size: bufferSize INTEGER (0..maxShortBS-ID } maxLCG INTEGER ::= 8 -- Maximum number of LCG identities maxShortBS-ID INTEGER ::= 255 -- Maximum number of long BS IDs -- Alternative implementation BufferStatusReportLong ::= SEQUENCE { -- (Quantized) buffer size for LCG1: bufferSize_LCG1 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG2: bufferSize_LCG2 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG3: bufferSize_LCG3 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG4: bufferSize_LCG4 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG5: bufferSize_LCG5 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG6: bufferSize_LCG6 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG7: bufferSize_LCG7 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG8: bufferSize_LCG8 INTEGER (0..maxShortBS-ID) OPTIONAL } |

Table 1. Comparing ASCII and ASN.1 formats for Short and Long BSR MAC CEs

Comparing these formats, we see that both the ASCII-based format looks quite similar to existing figures, while the ASN.1 format looks very different but the definition is more compact and (likely) scales better when having more complicated tables. The encoded structure shown differs from a structure defined using frame protocol description to illustrate more typical ASN.1 structural choices, which has some impact on the encoding size. Note that in this case it would be possible to have an encoding that has exactly the same size (but not the exact format) as the current MAC CE encoding if desired (it would just look “ugly”).

Finally, using simple Markdown pipe tables (e.g., using online or other tools such as this) could be an alternative, with the main difficulty being not having merged cells, but in most cases that might not be a big hindrance.

Observation 2: If the bit patterns must look similar to existing figure formats, the ASCII text-based approach is very close to the existing figure format.

Observation 3: Using ASN.1 could make complicated structures shorter but does require comprehending how ASN.1 encoding works.

Observation 4: Using Markdown pipe tables can work but requires avoiding merged cells, which may make some bit patterns look uglier.

Observation 5: Regardless of which way is chosen, it is feasible to replace the figures depicting bit pattern representations with plain text-based alternatives, possibly through the use of automatic generation.

# 3	Conclusions

In this pCR we propose to capture how the MAC/RLC/PDCP/SDAP control elements could be handled in RAN2 and other specifications using Markdown or other text-based formats.

# Proposed changes for TR 21.802

# * * * Start of Changes * * *

#### 5.3.1.7	Handling of bitmap or bit pattern elements in specifications (e.g. MAC, RLC, PDCP)

The bitmap or bit pattern representations are used in e.g. MAC specification to describe PDU formats and control element structures using figures (created using e.g. Visio). These can be represented either via figures or via textual representations using ASCII text or ASN.1 as exemplified below.

| Representation | Format |
| --- | --- |
| Current MAC |  |
| ASCII | +----+-----+----+----+----+----+----+----+ ¦ LCG ID ¦ Buffer size ¦ +----+-----+----+----+----+----+----+----+ +------+------+------+------+------+------+------+------+ ¦ LCG7 ¦ LCG6 ¦ LCG5 ¦ LCG4 ¦ LCG3 ¦ LCG2 ¦ LCG1 ¦ LCG0 ¦ +------+------+------+------+------+------+------+------+ ¦ Buffer size 1 ¦ +------+------+------+------+------+------+------+------+ ¦ ... ¦ +------+------+------+------+------+------+------+------+ ¦ Buffer size m ¦ +------+------+------+------+------+------+------+------+ |
| ASN.1 | BufferStatusReportShort ::= SEQUENCE { -- LCG: logicalChannelGroup ID INTEGER (1..maxLCG)), -- (Quantized) buffer size: bufferSize INTEGER (0..maxShortBS-ID } maxLCG INTEGER ::= 8 -- Maximum number of LCG identities maxShortBS-ID INTEGER ::= 31 -- Maximum number of short BS IDs BufferStatusReportLong ::= SEQUENCE ( SIZE (1..maxLCG)) OF BufferStatusLong BufferStatusLong ::= SEQUENCE { -- LCG: logicalChannelGroup ID INTEGER (1..maxLCG)), -- (Quantized) buffer size: bufferSize INTEGER (0..maxShortBS-ID } maxLCG INTEGER ::= 8 -- Maximum number of LCG identities maxShortBS-ID INTEGER ::= 255 -- Maximum number of long BS IDs -- Alternative implementation BufferStatusReportLong ::= SEQUENCE { -- (Quantized) buffer size for LCG1: bufferSize_LCG1 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG2: bufferSize_LCG2 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG3: bufferSize_LCG3 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG4: bufferSize_LCG4 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG5: bufferSize_LCG5 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG6: bufferSize_LCG6 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG7: bufferSize_LCG7 INTEGER (0..maxShortBS-ID) OPTIONAL, -- (Quantized) buffer size for LCG8: bufferSize_LCG8 INTEGER (0..maxShortBS-ID) OPTIONAL } |

Table 1. Example of using ASCII or ASN.1 formats for Short and Long BSR MAC CEs

* * * End of Changes * * *