3GPP Conference Call on 3GPP Spec Modernization #5	6GSM-260035

# Electronic, 15th January 2026, 13:00-15:00 UTC

Source:	Huawei

Title:	Pseudo-CR on clarifications of proposal 7 (Multiple formats for single specification with Word as baseline)

Document for:	Endorsement

Agenda item:	5

Spec:	3GPP TR 21.802

Version:	TR version 1.0.0

Work Item:	FS_6GSpecs

Comments

This document bring a clarifications to proposal 7.

The clarification brought to proposal 7 are the following:

- If .asn file is used for the RRC ASN.1, then normative comments can be used to convey field descriptions instead of tables (which are used today in RRC specifications). This offers an alternative to using another text-based format for the RRC ASN.1 while still using tables (in Markdown or AsciiDoc or LaTeX) for field descriptions. The decision should be left to RAN2 (or at least validated by RAN2 within this study) whether field descriptions would be using tables or normative comments, assuming the RRC ASN.1 would be separated in a different file format from the main body of the RRC specifications written with Word.

- It is clarified that restrictions to docx from proposal 6 are applied with proposal 7

- Other clarifications on formats and tools

Proposed Changes

* * * First Change * * * *

## 5.7	Proposal #7: Multiple formats for single specification with Word as baseline

### 5.7.1	Description

In the table below it is described how the various specification elements of a single specification could be stored, i.e. which file format they could be stored, where the main document of the specification is written with Word.

It should be noted that some specification elements can be captured in different formats depending on what is to be captured. One such example is tables, e.g. small tables might best be captured inline with the procedural text and hence captured in Word, but very large tables might best be captured in for example JSON.

Table 5.7.1-1: Specification elements of a single specification, files formats and tools.

| Specification element (NOTE 1,2) | File formats | Tools | Notes |
| --- | --- | --- | --- |
| Main document of the specification, including procedural text and similar (formatted text) | Word (*.docx), with restrictions as in Proposal #6 (see clause 5.6) | MS Word, WPS | Tools are mentioned as examples. |
| Tables | JSON (*.json) for large tables Small tables within the main Word document Excel (*.xlsx) for tables requiring mathematical calculation. | Any text editor such as Notepad++, Visual Studio Code, Vim, etc JSON syntax checker/validator For excel: MS Windows, WPS | Open-source free tools that have open-source extensions for JSON syntax checking and validation are preferred. |
| Diagrams (sequence diagrams, block diagrams, etc.) | Text source file (.puml, .msc, .mmd) + generated SVG. | Any text editor + PlantUML or MSC Generator or Mermaid |  |
| Graphics | SVG + source file | Vector drawing tools such as draw.io, Illustrator or other WYISWYG tools such as Powerpoint to create the graphics. Vector graphics editors such as Illustrator or Inkscape to edit the SVG. | Superset of diagrams; diagrams should be preferred over graphics |
| Images | PNG | Any image editor | 150dpi a good rule of thumb for standard images that are not graphics. Graphics should be preferred over images. |
| ASN.1 | Text file (*.asn) Normative comments may be included (e.g. -- [normative] this is a normative comment --). Normative comments may be used to convey field descriptions instead of tables (tables are used today for field descriptions in RRC specs) | Any text editor such as Notepad++ or Visual Studio Code + optionally ASN.1 syntax checking extension (e.g. ASN.1 by OSS Nokalva Inc. extension for VSCode) | The ASN.1 elements can be distributed in multiple files which then can be referenced/imported in other ASN.1 files. This is for the case where ASN.1 with normative comments and field description is not directly written in the main Word document (i.e. no need for a tool that could extract the ASN.1 from a Word file) |
| ASN.1 with normative comments and field description tables (such as used in RRC specification) | A text-based format such as Markdown (*.md) or Asciidoc (*.adoc) or LaTeX (*.tex), to produce IE descriptions like in 5G RRC specification. | Any text editor such as Notepad++, Visual Studio Code, Vim, etc. | This is for the case where ASN.1 with normative comments and field description tables is not directly written in the main Word document (i.e. no need for a tool that could extract the ASN.1 from a Word file). A script is needed to extract the ASN.1 from the Markdown or AsciiDoc or LaTeX document, which should be easier than extracting from Word. Use of Git is a separate consideration to be further discussed: e.g. editing within Git, using Gitlab for ASN.1 review (i.e. for drafting work) |
| Equations | Within the main Word document (.docx) as OMML | Word built-in equation editor, textmath for converting mathematical formulas to Office Math Markup Language (OMML) | Legacy OLE-based equations (Microsoft Equation and MathType) to be converted to OMML. |
| OpenAPI | Text file (.yaml) | YAML text parser (Notepad++) to generate YAML file (note that new swagger version does not support saving of YAML file and cross checking). OpenAPI YAML syntax checker (Swagger tool) | As done in SA and CT WGs (cf Annex B) |
| XML Schemas | .xsd (XML Schema Definition) | XML syntax checker/validator. | As done in SA3 and SA5 (cf Annex B) |
| YANG model | .yang | SA5 YANG data models are validated by an automated pipeline that runs for every push or merge-request event | As done in SA and CT WGs (cf Annex B) |
| Machine processable files specified in TTCN-3 language | .ttcn | Several TTCN-3 compilers can be used for syntax checking the TTCN-3 files. | As done in RAN5 (cf Annex B) |
| SDL diagrams in SDT binary files | .ini | .ini file supplied on the 3GPP server | cf TR21.801 Annex I |
| NOTE 1: some elements may appear only in some specifications but not in all specifications. NOTE 2: to the extent possible, redundancy shall be discouraged, e.g. if ASN.1 is stored in a separate file, the same content shall not be duplicated in the .DOCX document. |  |  |  |

### 5.7.2	Evaluation against requirements of section 4.3

* * * End of Changes * * * *